ANNEX E

SPECIFICATIONS

Table of contents

1	Introduction	3

2	High-level description	3

2.1	General	3

2.2	Operational context and limitations	3

2.3	Model 1 – Line Maintenance (‘Line’)	4

2.4	Model 2 - Phase/Calendar Maintenance (‘Phase’/’Calendar’)	4

2.5	Model 3 – Depot Maintenance (‘Depot’)	6

2.6	Guiding principles	6

3	Requirements Specifications	6

3.1	Introduction and instructions	6

3.2	General requirements	7

3.3	Specific requirements – Model 2 Phase/Calendar and Model 3 Depot	10

4	Climate and environmental considerations	10

Attachment 1 – Dimensioned drawings

Attachment 2 – Illustrated folded pylon

Attachment 3-n - TO BE INSERTED BY THE CONTRACTOR

- Introduction

This Annex provides a high-level description of the needs including prerequisites and limitations for use and handling of Work Stations for the MH-60R Seahawk helicopters as well as specification of preferences and requirements.

- High-level description

  - General

The primary purpose of the contract is to ensure safe and efficient on-site maintenance of MH-60R helicopters (also referred to as ‘aircraft’) at RNoAF premises by the manufacturing and delivery of customised Work Stations in the quantities as specified in Appendix A.

The delivery includes the design, manufacturing and delivery including detailed fitting of the Work Stands as well as associated usables, spare parts and tools, documentation and training in assembly, use, handling, inspection and maintenance.

The Work Stations shall fit the aircraft in designated maintenance docks and be suitable for different levels of maintenance; Line, Phase/Calendar and Depot maintenance. Due to the repetitive nature of certain maintenance tasks the Work Stands must be modular and easy to handle and maneouver.

It is emphasised that the responsibility of the Contractor to ensure that the Work Stands are adequately adjusted and fitted to the curvature/shape of the aircraft may be completed after delivery and assembly of the first set of each model.

The Work Stations shall comply with relevant European directives and standards, including statutory requirements in accordance with applicable Norwegian HSE legislation, as referred in Annex F Statement of Work, chapter 2.

It is foreseen that user training is provided by the Contractor on-site together with assembly of the first delivery of each Work Stand Model.

  - Operational context and limitations

The Works Stands are used in daily and periodic inspections and maintenance of the MH-60R, and will be used on flat, painted concrete surface inside aircraft hangars. The size of the allocated area for the footprint of helicopter and Work Stands is 21,0 m long and 10,0 m wide. The Work Stands can be moved/ rolled between two hangars. The surface between hangars is flat tarmac/concrete.

For the Work Stands to be used as desired, they must provide unobstructed access to the aircraft from ground level, including cockpit and cabin doors which must not be blocked, and to the top of the helicopter and to strategic points along the body, like tail boom and tail rotor. The must enable work to be done simultaneously at both levels.

On the main stands along the fuselage 5-6 persons shall be able to work safely at the same time. On the tail rotor stands 2 persons shall be able to work at the same time. This applies for all Work Stands.

All models shall have a solid anti-slip floor which is resistant to weather (rain/snow) and fuel/oil spill without deteriorating its properties, and a panel around the floor of the platform to prevent equipment, tools, or parts to drop from height onto personnel, helicopter or floor. Access to platform to be secured by anti-slip stairs with railings. All parts facing the aircraft shall be rubber padded to avoid damage to the fuselage, pylon and tail rotor. There shall not be any railings on the side of the Work Stand facing the helicopter.

  - Model 1 – Line Maintenance (‘Line’)

The Line model shall be a lightweight and easily manoeuvrable model for quick and light maintenance tasks. This stand is moved frequently for daily inspections and must be movable by two persons. The wheels of the line model stands shall be lockable.

The Line model should include four modules:

- Fuselage left hand side module

- Fuselage right hand side module

- Tail rotor left hand side module

- Tail rotor right hand side module

Access is required for main rotor blades, rotor head, gear box, engines, hydraulics and other components on the “top” of the helicopter. Work Stands are required along the LH and RH side of the fuselage. The engine covers are folded over the Work Stands when access to engine is required. The height of the flooring of the platform for the LH and RH fuselage Work Stands should be 1850 mm. Approx length of RH/LH fuselage stands is 5750 mm.

Cabin doors RH side must not be obstructed as components are taken in and out of the helicopter. Width of door is 1150 mm. It is placed 4300 mm from the front end of the FLIR camera. The door is placed 750 mm from the ground and the height is 1380 mm. The cabin door slides backwards along the fuselage and stops 2230 mm from the front end of the FLIR camera.

Pilot doors RH/LH side shall be not be obstructed by the Work Stand. They are placed 1780 mm from the front end of the FLIR camera, and are 1030 mm wide. When closed the doors are 1320 mm from the center line. When open the doors extend 2200 mm from the center line and the max height of the door is 2200 mm from the floor.

In order to test the use of the winch the RH side platform shall have a hatch adjusted to the position of the winch. The winch wire is 4900 mm from the front end of the FLIR camera and 390 mm from the helicopter side. The hatch opening is 300 mm parallell to helicopter center line and 250 mm wide.

Two platform modules are required for the tail rotor maintenance/inspection. The modules shall allow for personnel to move on on both sides and behind the tail and tail rotor. Access between the stands is required.

The tail rotor is on the right-hand side of the tail, its diameter is 3,35 m and must be able to turn without obstructions. The height of the LH side platform is 2450-2500 mm and the height of the RH side platform is 2100-2150 mm.

  - Model 2 - Phase/Calendar Maintenance (‘Phase’/’Calendar’)

Model 2 requires adjustable height, LED lighting, electrical connections, USB-C connections, pneumatic connections. Preferably movable tool tray, laptop tray, garbage bins. This model is used both for frequent calendar inspections/ maintenance and for ‘heavier’ flight hour-based maintenance.

Electrical components must be CE-labeled and provide electrical power via EU type F plug (230V 16A) according to EU directive 2014/35/EU, LED lighting to provide 1000 lumen working light at ground level.

The Work Stands shall have legs that are adjustable in height, 600 mm. The legs are required to place the stands in fixed positions off wheels. The adjustment can be achieved by manual means, like hand crank, jack or similar.

The Phase/Calendar model should include five modules:

- Fuselage left hand side module

- Fuselage right hand side module

- Tail rotor left hand side module

- Tail rotor right hand side module

- Front module (only one -1- front module is required)

Access is required for main rotor blades, rotor head, gear box, engines, hydraulics and other components on the “top” of the helicopter. The Work Stand running along the fuselage should cover the length from the rear end of the pilot door to the rear tire. 
NOTE 1: Engine cover will fold sideways and lay on top of Work Stands.
NOTE 2: As illustrated in Appendix 2 the pylon is folding on the left side, hence the LH fuselage module should be shorter than the RH stand.

Cabin door RH side must not be obstructed as they are used for taking components in and out of the helicopter. Width of cabin door is 1150 mm and the height is 1380 mm. Cabin door is placed 4300 mm from the front end of the FLIR camera, 750 mm from the ground. The cabin door slides backwards along the fuselage.

Pilot doors RH/LH side shall be not be obstructed by the Work Stand.  They are placed 1780 mm from the front end of the FLIR camera, and are 1030 mm wide. When closed the door is 1320 mm from the center line. When open the door extends 2200 mm from the center line and the max height of the door is 2200 mm from the floor.

Avionics compartment/junction boxes are placed on both LH and RH side with hatches/openings placed 3200 mm from the front end of the FLIR camera. The width of the hatches are 690 mm. The hatches are equally placed on both sides. The doors tilt upwards, hinged at the top end.

In order to test the use of the winch the RH side platform shall have a hatch adjusted to the position of the winch. The winch wire is 4900 mm from the front end of the FLIR camera and 390 mm from the helicopter side. The hatch opening is 300 mm parallell to helicopter center line and 250 mm wide.

Access to mission power cooling fan outlet. During maintenance a silencer is attached to the power outlet. The outlet is placed on LH side in front of the left tire/under the junction boxes 3520 mm from the front end of the FLIR camera. The silencer’s end piece is sticking out 150 mm more than the rim of the tire.

Access to electric power inlet. The inlet is on the RH side in front of the tyre. 3300 mm from the front end of the FLIR camera.

Access to fuel cells. Fuel cell access covers are on the LH side of the helicopter. The combined width of the covers is 550 mm, and the center of the covers is 6620 mm from the front end of the FLIR camera.

On the right-hand side behind the cabin doors an external life right pod (ELRP) is attached. To avoid conflict with the ELRP and Work Stand, any Works Stand legs should be placed 600 mm from the side of the helicopter or further back along the fuselage.

Two platform modules are required for the tail rotor maintenance/inspection. The modules shall allow for personnel to move on on both sides and behind the tail and tail rotor. Access between the stands is required. The tail rotor is on the right-hand side of the tail, its diameter is 3,35 m and must be able to turn without obstructions.

The tail rotor stabilizers flex up and down. Current data states 160 -200 mm upward tilt from the 0-degree tilt. Height centerline of stab is 1586 mm. However, max tilt will be subject to verification. Please note that Tail rotor stand adjustability (600 mm) must be taken into account.

In front of the rotor head it is possible for maintenance personnel to move between the LH and RH side modules. There shall be a detachable safeguard/rail/chain, securing users from falling forwards, between the right- and left-hand fuselage modules’ railings.

Railings on the Work Stand should be able to fold down in order to rotate the rotor with blades. The tail rotor Work Stand must not interfere or obstruct the possibility to turn the tail rotor.  A board for tools will/can be attached to the railings of the Work Stand Model 2 and 3. The weight of the board with tools is 30 kg. The railings shall be able to hold this weight.

RH and LF fuselage stands must allow sufficient space for a front stand used for changing/repairing wind screens to be placed in between. Maximum front stand height is 1600 mm.

  - Model 3 – Depot Maintenance (‘Depot’)

This model will be used for heavier maintenance tasks. Depot maintenance usually lasts several months and requires adjustable height, tool tray, laptop tray, LED lighting, garbage bins, electrical connections, USB-C connections, pneumatic connection and forklift pockets.

Model 3 is largely similar with the Model 2, but with an additional bridge module to connect the right and left side fuselage modules placed at the front end of the LH and RH side modules. The width of the bridge is 600 mm and it must carry the weight of two persons.

There shall be a detachable safeguard/rail/chain, securing users from falling forwards, between the right- and left-hand fuselage modules’ railings.

Railings on the Work Stand should be able to fold down in order to rotate the rotor with blades. The tail rotor Work Stand must not interfere or obstruct the possibility to turn the tail rotor.  A board for tools will/can be attached to the railings of the Work Stand Model 2 and 3. The weight of the board with tools is 30 kg. The railings shall be able to hold this weight.

Accordingly, the Depot model includes six modules

- Fuselage left hand side module

- Fuselage right hand side module

- Tail rotor left hand side module

- Tail rotor right hand side module

- Front module

- Bridge to connect the right hand and left hand side fuselage modules

  - Guiding principles

Based on the preceding descriptions in this chapter the Purchaser envisions the following guiding principles for best fulfilling the purpose of the procurement:

- Adaptation and unrestricted access to aircraft

- Solid and robust construction, designed for long lifetime (30 - 40 years)

- Compliance with HSE regulations and requirements

- Modularity, flexibility and mobility/maneouvrability; easy to use and handle

- Requirements Specifications

  - Introduction and instructions

The preferences and requirements are listed in the following manner:

No.:		Unique serial number for the requirement

Description:		Description of the requirement

The Contractor shall respond to the specified preferences and requirements in the following manner:

Fulfilment:	The extent to which the contractor commits to comply with the requirement, answered by Yes, No or Partially. If the requirement is responded to as partially fulfilled, the solution must describe which parts of the requirement that are not fulfilled.

Solution:	Detailed information describing how the requirement is fulfilled.

When documenting its fulfilment of the stated preferences and requirements it imperative that the Contractor envisions that the Purchaser must be able to require fulfilment in accordance with the content and wording of the preferences/requirements. This implies that the Contractor must ensure that fulfilment is committed and verifyable; i.e

- the response confirms fulfilment,

- fulfilment is made in a binding way, and

- documentation suitable for verification of how fulfilment will be ensured is provided.

Such documentation may include (in order of preference):

| Technical data sheet | Technical Data Sheets providing key technical specifications, calculations, performance characteristics and essential product information. |
| --- | --- |
| Product documentation | Product documentation includes any material that supports verification of the Contractor’s fulfilment of the requirements and the offered solution. The Contractor is encouraged to document the approach and method for fulfilling the requirements and provide supporting material that provides a coherent picture of the intended solution. Such documentation may include dimensioned drawings, photos, preliminary configurations, technical descriptions or other relevant evidence. |
| Self-declaration | Alternatively, if Technical Data Sheet or other Product Documentation is not available or applicable a self-declaration from the Contractor on how compliance will be achieved may be provided. |

In the event the Contractor finds it impractical to document ‘solution’ in the requirement table, the documentation may be attached as a separate document, please refer to Annex E – Attaxhment 3. In such case the solution column for any requirement fulfilment specification shall include an unambiguous reference to where the solution is detailed. Similarly, such attachment documentation shall contain a precise cross-reference back to the relevant requirement.

  - General requirements

The requirements and preferences in this section apply for all models.

| Purchaser’s requirements |  | Contractor’s response |  |
| --- | --- | --- | --- |
| No. | Description | Fulfilment | Solution |
| Functional Requirements |  |  |  |
|  | For each Work Stand Model (Model 1, 2 and 3) the Contractor shall describe how the solution meets the functional and operational characteristics and needs described in chapter 2, in support of the guiding principle of Adaptation, cf. section 2.6. The description should include: - Shaped-to-fit the helicopter in accordance with relevant dimensions described above and in Attachments 1 and 2 - Avoidance of conflict zones between the Work Stands and the helicopter. - Correct interface gaps and tolerances. - Platform capacity for assembly and disassembly. - Interchangeability of components. - Any limitations or assumptions related to the use of the Work Stands. - Any additional features, characteristics or solution elements which may contribute to increased value for the operational use of the Work Stand. |  |  |
|  | For each Work Stand Model (Model 1, 2 and 3) the Contractor shall describe how the solution meets the functional and operational characteristics and needs described in chapter 2, in support of the guiding principle of Design and construction, cf. section 2.6. The description should include: - Structural design quality including solid and robust construction, load-bearing capacity (and per quare meter(kg/m2) and maximum number of personnel that can work on the helicopter simultaneously. - Integration of functional features including work tables, garbage bins, tool and laptop trays, power supply, lighting, air supply and other task-supporting features relevant to the use cases. - Any limitations, dependencies or assumptions related to the design. - Any additional features, characteristics or solution elements which may contribute to increased value for the operational use of the Work Stand. |  |  |
|  | For each Work Stand Model (Model 1, 2 and 3) the Contractor shall describe how the solution meets the functional and operational characteristics and needs described in chapter 2, in support of the guiding principle of HSE, cf. section 2.6. The description should include: - Markings and visual cues, including visibility, clarity and relenace for safe operation. - Slip resistance and surface properties. - Safe access and egress including steps, ladders, handrails, fall-prevention measures and stability. - Reach and operator ergonomics. - How the platform supports safe work at both ground level and platform level. - Any limitations, assumptions or operational contraints. - Any additional features, characteristics or solution elements which may contribute to increased value for the operational use of the Work Stand. |  |  |
|  | For each Work Stand Model (Model 1, 2 and 3) the Contractor shall describe how the solution meets the functional and operational characteristics and needs described in chapter 2, in support of the guiding principle of Mobility and Maneuverability, cf. section 2.6. The description should include: - Mobility and handling by personnel including weight and weight distribution, handling characteristics and stability during movement and wheel design. - Positioning and adjustment including height adjustments and how easily the Work Stands can be aligned, positioned and adjusted in accordance with intended useas described in chapter 2. - Modularity including interchangeability and ability to add, remove or reconfigure modules. - Transport and storage when not in use. - Limitations, assumptions and operational constraints. - Any additional features, characteristics or solution elements which may contribute to increased value for the operational use of the Work Stand. |  |  |
| Training |  |  |  |
|  | The Contractor shall specify recommended training to achieve the necessary user competence for correct assembly, operation and maintenance of the system. |  |  |
| Inspection and Maintenance |  |  |  |
|  | The Work Stands shall be allowed for inspection, repair and maintenance on-site by the Royal Norwegian Air Force (RNoAF) personnel, using documentation provided by the Contractor. |  |  |
|  | Any special tools for operation or maintenance of the Work Stands shall be included in the delivery. |  |  |
|  | The Contractor shall commit to supplying the RNoAF with spares, consumables and document updates for a minimum of 5 years after each delivery including options. |  |  |
| Documentation |  |  |  |
|  | The Work Stands shall comply with relevant European directives and standards, cf. Annex F. If available, certificate from accredited organ shall be available or included in shipping packages. |  |  |
|  | All materials, components and chemical substances used in the Work Stands shall comply with REACH Regulation (EC) No. 1907/2006. List of material composition shall be provided. |  |  |
|  | The Contractor shall provide the following manuals in English: - Operator's Manual (OM) - Inspection and Maintenance Manual (IMM) - Electrical Wiring Diagrams (WD) - Pneumatic Diagrams (PD) - Illustrated Parts Catalogue (IPC) Documentation to be provided no later than date of delivery. |  |  |
|  | All documentation shall be made available in Portable Document Format (PDF). |  |  |

  - Specific requirements – Model 2 Phase/Calendar and Model 3 Depot

The requirements and preferences in this section apply particularly for Model 2 Phase/Calendar and Model 3 - Depot.

| Purchaser’s requirements |  | Contractor’s response |  |
| --- | --- | --- | --- |
| No. | Description | Fulfilment | Solution |
|  | The Work Stand shall have regular EU 230V outlets that is not in conflict with work areas |  |  |
|  | The Work Stand shall have compressed air outlets that is not in conflict with work areas. |  |  |
|  | The Work Stand shall be equipped with lighting that provides a minimum of 1000 lux at ground level during use. |  |  |

- Climate and environmental considerations

The Contractor shall minimize the impact on the environment and climate associated with the delivery during the Work Stand life-cycle in:

- Materials selection

- Construction and Manufacturing

- Packing and Shipment

- Assembly, Operation and Maintenance

- Disposal

The contractor shall document fulfilment of the requirements below in accordance with the instructions as provided in section 3.1 above.

| Purchaser’s requirements |  | Contractor’s response |  |
| --- | --- | --- | --- |
| No. | Description | Fulfilment | Solution |
|  | The percentage (%) of total electricity used in the construction of the Work Stands that originates from sustainable sources (solar, wind, hydro, biomass, nuclear or geothermal). Examples of acceptable documentation include certificates or statements from electricity suppliers or equivalent verified sources. |  |  |
|  | Measures taken to minimize environmental impact in the production of the material used in the Work Stands, and in the construction of the Work stands, including but not limited to: percentage (%) of recycled materials, reduction of emissions and waste handling. Examples of acceptable documentation include Environmental Product Declarations (EPDs), purchasing documentation, or material specifications from suppliers. |  |  |
|  | Measures taken to minimize environmental impact related to packing and shipping of the Work Stands, including but not limited to: weight, packing volume and space-optimization, waste reduction and resource efficiency. Examples of acceptable documentation include Environmental Product Declarations (EPDs) and descripitons with supporting verifiable documentation. |  |  |
|  | Measures taken to minimize environmental impact during operation and maintenance of the Work Stands, and to extend the functional lifetime, including but not limited to: expected maintenance frequency, corrosion resistance and durability, surface treatments, waste generation and resource use. Examples of acceptable documentation include Environmental Product Declarations (EPDs) and descripitons with supporting verifiable documentation. |  |  |
|  | How the Work Stands enable environmentally efficient end-of-life handling, including sorting, special waste handling and recyclability of materials. Examples of acceptable documentation include Environmental Product Declarations (EPDs) and descripitons with supporting verifiable documentation. |  |  |
|  | Presence of PFAS, PFOA and PFOS in the Work Stands shall be provided, including substance name, CAS number, concentration, component location and the functional purpose of each PFAS substance. |  |  |