| Part 2 | Draft Contract |  |  |
| --- | --- | --- | --- |
|  | Annex E-1 | System Segment Specification Portable Meteorological Sounding System |  |

TABLE OF CONTENTS

1	Scope	4

1.1	Identification	4

1.2	System overview	4

1.3	Standards for interpretation of this specification	4

2	Applicable documents	4

2.1	General	4

2.2	Standards and publications	5

2.3	Referenced documents	5

2.4	Order of precedence	6

2.5	Guidance to requirements	6

2.5.1	Requirement types	6

2.5.2	Column “Compliant (yes/no)”	6

2.5.3	Column “Bidder’s response”	6

3	Requirements	7

3.1	Functional and performance requirements	7

3.1.1	Missions	7

3.1.2	Required states and modes	7

3.1.3	Use profile	8

3.1.4	Entity capability requirements	9

3.1.4.1	General requirements	9

3.1.4.2	Power supply	12

3.1.4.3	Operational environment	13

3.1.4.4	Transportation	13

3.1.4.5	Maintenance	14

3.1.4.6	Documentation	15

3.1.4.7	Environmental	15

4	Verification	17

4.1	Methods of verification	17

4.2	Classes of verification	17

4.3	Referenced verification documents	18

4.4	Execution of verification	18

Appendixes

Scope

Identification

Norwegian Defence Material Agency shall procure a Portable Meteorological Sounding System. This specification defines the requirements for the Portable Meteorological Sounding System hereby referred to as “the System”. Some of the requirements refer to the equipment itself as the overall system and some requirements are specific for different segments of the system.

System overview

The System will be used to document weather conditions in altitudes up to 30km. The system shall be portable and set up at various test grounds in Scandinavia. The System shall be an “off the shelf” product (COTS) with tested and proven solutions. The purchaser will not partake in testing non- mature technology. The system supplier must deliver radiosondes, balloons (if balloons are part of the system) and sounding station.

Standards for interpretation of this specification

Throughout the specifications, the following standards shall apply:

- Whenever requirements are stated herein to "include" a group of items, parameters, and/or other considerations, "include" shall be construed to mean, "Included but not limited to."

- Whenever reference is made to a section, task and/or paragraph, the reference shall be construed to include all subordinate and referenced paragraphs.

- "Will" statements are non-mandatory, and/or they imply intent on the part of the Purchaser.

- The expression “and/or” shall be interpreted to mean “inclusive or”, i.e. all specified conditions shall be satisfied, not necessarily simultaneously.

Applicable documents

General

In this specification, reference to documents might be done directly in the text or indirectly by reference to the tables in chapter 2.2, 2.3 or 4.3. The table in chapter 4.3 summarises verification documents, while the table in chapter 2.2 summarise standards and publications and 2.3 summarises all other referenced documents.

Standards and publications

The following documents of the specified issue, shown in the table below, are part of this specification to the extent specified herein. In the event of conflict between the documents referenced and the contents of this specification, the contents of this specification is to be considered the superseding requirement.

| Doc # | Identification | Title | Comment |
| --- | --- | --- | --- |
|  | STANAG 4082 | Adoption of a Standard Artillery Computer Meteorological Message. |  |
|  | NATO STANDARD AOP-65 | Firing techniques to determine ballistic data for fire control systems. |  |

Table 2-1

Referenced documents

The following documents of the specified issue, shown in the table below, are part of this specification to the extent specified herein. In the event of conflict between the documents referenced and the contents of this specification, the contents of this specification is to be considered the superseding requirement.

| Doc # | Identification | Title | Comment |
| --- | --- | --- | --- |

Table 2-2

Order of precedence

In the event of a conflict between the text of this document and the references cited herein, the text of this document takes precedence. Nothing in this document, however, supersedes applicable laws and regulations unless a specific exemption has been obtained.

Guidance to requirements

Requirement types

All requirements are either “Shall” requirements or “Should” requirements.

The use of SHALL and SHOULD requirements is as follows:

- A SHALL requirement is mandatory. This is a requirement that is critical to the systems performance and functionality, and a direct result of the identified needs. It is mandatory for the Bidder to agree to meet the SHALL requirements in order to be compliant. The Bidder should document for the Purchaser how these requirements are met. Non-compliance with a SHALL requirement will lead to rejection of the offer, unless the Bidder has documented a solution that the Purchaser finds will cover the required function and functionality of the requirement.

- SHOULD (1) requirements indicate strong needs where some variation is possible. The SHOULD (1) statements will become SHALL statements at contract placement and may incorporate an agreed change to the text.

- SHOULD (2) requirements indicate a "want" rather than a "need". SHOULD (2) Will be rewarded less points under the evaluation.

The Bidders response shall be clear, concise, and include sufficient detail for effective evaluation and for substantiating the validity of stated claims.

The Bidder should not simply rephrase or restate the requirements, but rather provide convincing rationale to address how the Bidder intends to meet these requirements.

Requirements are evaluated on the Bidders response to the requirement as well as actual performance and verification documentation. The answer to the technical requirements must be convincingly described and verified by documentation in order to achieve the highest evaluation score. Simply answering “yes” will automatically be rewarded the minimum score for fulfilled requirement.

Column “Compliant (yes/no)”

The Bidder shall for each requirement fill in either “yes” or “no” based on whether the tender will comply with the requirement or not.

Column “Bidder’s response”

The Bidder shall use this column to describe the proposed solution and how the requirement will be met, as well as limitations that apply or adaptions that will have to be made in order to meet the requirement.

The response from the Bidders will be used by the Purchaser to evaluate the tender.

If the information can be found in a document attached to the tender, the Bidder may reference where the specific relevant information can be found. In this case the precise name of the attachment, page or page range, and/or section of the document shall be specified.

Requirements

Functional and performance requirements

Missions

The System will be used to collect weather data during artillery indirect firing and other relevant tests. The System will be used in all typical Scandinavian climate conditions. The System will be set up partially in a temperature-controlled place, e.g. inside a vehicle or tent/office container. The System will be transported in a standard van and powered by national grid or GFE generator.

Typical 10-20 radiosondes will be launched in the scope of one day, with tests lasting for one to three weeks continuously. If the system is delivered with balloons, a non-volatile gas like Helium is preferred. A standard portable computer will be used to run system software.

Operational use

- The sequence for operations could be as follows:

  - Transport to the mission area.

  - Establish a base of operation.

  - Preparing the System for use.

  - Perform measurements.

  - Data storage.

  - Analysis and reporting.

Required states and modes

The status of the System may vary between:

- Storage.

- Transport.

- Operational use (live measurements in field trials).

- Maintenance.

- Service by supplier.

Use profile

The expected functional lifetime of the System is 10 years.

The System will typically have the following use profile in a 1-year cycle:

- Operator training and course.

- Measurements under various climatic conditions.

- Maintenance according to suppliers' recommendation and NDMA requirements, including calibration either in Norway or at suppliers' location.

- When not in use, the System will be stored ready for operation, in climate-controlled storage at Test & Verification’s location.

Entity capability requirements

General requirements

| Requirement | Comply Yes/No | Bidder's reply (Bidder shall reply to each required reply (SSSR) e.g. performance, reference to specifications/tests etc.) |
| --- | --- | --- |
| The System SHALL be commercially off the shelf (COTS) product. |  |  |
| The System SHALL be portable. |  |  |
| The System SHALL enable storage handling, transport by vehicle, assembly and disassembly to be performed by one operator. |  |  |
| The System SHALL have an integrated stationary ground GNSS receiver determining ground station position, using multiple (more than one) GNSS systems like GPS, Galileo, GLONASS, BeiDou. |  |  |
| The System SHALL be able to automatically determine the ground station position. |  |  |
| The System SHALL have the option to automatically initiate sounding when the balloon is released. |  |  |
| The System SHALL be capable of receiving information from two or more radiosondes simultaneously. |  |  |
| The System SHOULD (1) be capable of receiving information from four or more radiosondes simultaneously. |  |  |
| The System SHOULD (1) be capable of receiving and storing information from multiple radiosondes simultaneously in a single software instance. |  |  |
| The System SHALL be capable of stop sounding and start tracking a new radiosonde. |  |  |
| The System SHALL be delivered with software capable of initiating radiosondes, performing radiosonde self-test, handling sounding, analysis and automatic/manual report generation, as well as data storage. |  |  |
| The System SHALL be delivered with a software compatible with a standard portable computer (GFE) running latest Windows release. |  |  |
| If software is license based, the license SHOULD (1) not be locked to computer hardware. |  |  |
| The System SHALL be connected to GFE portable computer by USB or ethernet. |  |  |
| The supplier SHOULD (1) supply radiosondes and unwinder assembly. |  |  |
| The supplier SHOULD (1) supply balloons. |  |  |
| The System SHALL Comply with STANAG 4082 (Adoption of a Standard Artillery Computer Meteorological Message). |  |  |
| The System SHALL produce standard meteorological reports and METCM messages. |  |  |
| The System SHALL store all weather data either internally or in the GFE computer. |  |  |
| The System SHALL Comply with NATO STANDARD AOP-65 (Firing techniques to determine ballistic data for fire control systems). |  |  |
| The System SHALL be able to collect meteorological data up to 30 000m. |  |  |
| The System SHALL be able to collect; date and time (min), height (m), air pressure (hPa), air temperature (°C), humidity (%RH), wind speed (m/s) and wind direction (degrees or/and mils). |  |  |
| The collected data SHALL have a required tolerance of: - - Date and time: 1 min. - - Height: 5 m - - Air pressure: 1 hPa - - Air temperature: 1° C - - Humidity: 5 % RH - - Wind speed: 1 m/s - - Wind direction: 17 mils |  |  |
| The System SHALL have a logging frequency of minimum 1 Hz. |  |  |
| The receiver system SHALL have a frequency range including 400.15 – 406 MHz. |  |  |
| The Systems UHF antenna and GPS antenna SHOULD (1) be delivered with tripod. |  |  |
| The System SHOULD (2) be delivered with a system for filling/launching balloons. Gas canisters are not included. |  |  |
| The System SHALL be delivered with cable length between antennas and receiving sounding system of minimum 20m. |  |  |

Power supply

| Requirement | Comply Yes/No | Bidder's reply (Bidder shall reply to each required reply (SSSR) e.g. performance, reference to specifications/tests etc.) |
| --- | --- | --- |
| The System SHALL work on both national grid and generator supply of 230V, 50Hz, max 16A, and the power cable must be delivered with a Schuko plug. |  |  |

Operational environment

| Requirement | Comply Yes/No | Bidder's reply (Bidder shall reply to each required reply (SSSR) e.g. performance, reference to specifications/tests etc.) |
| --- | --- | --- |
| The System SHALL be fully operational in climate zone A1 to C1. (STANAG 4370 (AECTP-230)). Equipment not required to be outside during operation is exempted, Example: data processing unit, computer etc. |  |  |
| The System SHOULD (1) be able to be stored in climate zones A1 to C1. (STANAG 4370 (AECTP-230)). |  |  |
| The radiosondes SHOULD (1) remain fully operational after a storage period of three (3) years under the following standard environmental storage conditions: - Ambient temperature: +10 °C to +30 °C - Relative humidity: ≤ 85 % (non-condensing) |  |  |

Transportation

| Requirement | Comply Yes/No | Bidder's reply (Bidder shall reply to each required reply (SSSR) e.g. performance, reference to specifications/tests etc.) |
| --- | --- | --- |
| It SHALL be possible to transport the System in a small van (maximum 2,5m3, gas canisters not included). |  |  |
| It SHOULD (1) be possible to transport the System in a small van (maximum 1,5m3, gas canisters not included). |  |  |
| It SHOULD (2) be possible to transport the System in a small van (maximum 1m3, gas canisters not included). |  |  |
| The System SHALL be packed in rugged transport cases (e.g. Pelicases) |  |  |
| Each transport case including its contents SHALL not weigh more than 35kg. |  |  |
| Each transport case including its contents SHOULD (1) not weigh more than 25kg. |  |  |

Maintenance

| Requirement | Comply Yes/No | Bidder's reply (Bidder shall reply to each required reply (SSSR) e.g. performance, reference to specifications/tests etc.) |
| --- | --- | --- |
| It SHALL be possible to perform Operator level preventive maintenance without requiring special tools or equipment. |  |  |
| It SHALL be possible to perform operator level maintenance in the field. |  |  |

Documentation

| Requirement | Comply Yes/No | Bidder's reply (Bidder shall reply to each required reply (SSSR) e.g. performance, reference to specifications/tests etc.) |
| --- | --- | --- |
| The Operator’s Manual SHALL contain, but not be limited to, the following types of information: - - All conditions and procedures which are necessary to prepare the equipment for use. - - The procedure required to start the equipment described step by step and well-illustrated - - All controls required to operate the equipment - - Safety procedures |  |  |
| All documentation SHALL be in Norwegian and/or English. |  |  |
| All electronic documentation SHALL be in PDF format. |  |  |
| Unless all operator level maintenance is included in the Operator’s Manual, the Supplier SHALL deliver a Maintenance Manual describing the procedures for all operator level maintenance tasks. |  |  |

Environmental

| Requirement | Comply Yes/No | Bidder's reply (Bidder shall reply to each required reply (SSSR) e.g. performance, reference to specifications/tests etc.) |
| --- | --- | --- |
| The radiosonde SHOULD (1) be biodegradable. |  |  |
| Unwinder and tether materials SHOULD (1) be fully biodegradable or compostable within ≤24 months in natural environments. |  |  |
| Balloon materials SHOULD (1) be manufactured from ≥95% natural latex or equivalent biodegradable elastomer, fully biodegradable within ≤36 months. |  |  |
| The combined mass of the radiosonde and unwinder assembly SHOULD (1) not exceed 150 grams. |  |  |
| Packaging of radiosondes and balloons SHOULD (1) be made of recyclable materials. |  |  |

Verification

Methods of verification

Methods utilised to accomplish verification in this specification include:

| Analysis: | A method of verification that utilises established technical or mathematical models or simulation, algorithms, charts, graphs, diagram or other scientific principles and procedures to provide evidence that the stated requirements are met. A documentation of a performed test by the manufacturer can also be accepted as a verification. Analysis of the documentation provided. |
| --- | --- |
| Test: | A method of verification that utilises specified procedures and acceptance criteria to provide evidence that the stated requirements are met. Test generally denotes the determination, by technical means, of the properties or elements of items, including functional operation, and involves the application of established scientific principles and procedures. |
| Inspection: | A method of verification consisting of investigation, without the use of special laboratory appliances or procedures. Inspections are generally non-destructive and typically include the use of sight, hearing, smell, touch, and taste; simple physical manipulation; mechanical and electrical gauging and measurement; and other forms of investigation. This verification method might also be used for systems not yet implemented. Inspection performed at the design stage means a less formal examination of the design than the Analysis method described above. In most cases when it is referred to Inspection, there will be no need for any particular procedure. The inspection will be performed based on the wording of the requirements in the System Specification. |

Classes of verification

At different stages from design to delivery, the following classes of verification are referred to:

| Design Verification: | Verification during design of materiel |
| --- | --- |
| First Article Verification: | Verification on items from the first production line |
| Acceptance Verification: | Verification on items from each production line. The verification may be performed on each item or on samples. Samples might be taken from each production line or from each production lot. |

Referenced verification documents

The following documents of the specified issue shown in the table below are part of this specification to the extent specified herein. In the event of conflict between the documents referenced and the contents of this specification, the contents of this specification is to be considered the superseding requirement.

| Reference | Type of test | Identification | Title | Description / Comments |
| --- | --- | --- | --- | --- |

Table 4-1 Verification procedures and reports

Execution of verification

The table below, Table 4-2, states identified verification activities covered and specified in this Contract. The table lists selected requirements (SSS) and specify verification activities related to these requirements. The table is called the “Verification Matrix”.

Some of these verification activities might have been executed before Contract award, but most of the verfication activities stated in the Verification Matrix shall be performed as part of this Contract. Most of the verification precedures and reports are collected and listed in Table 4-1 and referred to in the Verification Matrix. (Some referencies might however be to documents listed in Table 2-2).

For each of the listed requirements (SSS), the following specification of verification activities is given in Table 4-2:

Class of verification (Design Verification, First Article Verification, Acceptance Verification)

Verification method (Analysis, Test, Inspection)

Reference to verification documentation (listed in Table 4-1 or in Table 2-2). The reference shall include both document reference number and reference within the document (paragraph, page etc).

| Requirement # | Verification methods and references |
| --- | --- |
|  | Acceptance Verification |
| SSS1 | Analysis |
| SSS2 | Inspection |
| SSS3 | Inspection |
| SSS4 | Inspection |
| SSS5 | Inspection |
| SSS6 | Inspection |
| SSS7 | Inspection |
| SSS8 | Inspection |
| SSS9 | Inspection |
| SSS10 | Inspection |
| SSS11 | Analysis |
| SSS12 | Inspection |
| SSS13 | Analysis |
| SSS14 | Analysis |
| SSS15 | Inspection |
| SSS16 | Analysis |
| SSS17 | Analysis |
| SSS18 | Analysis |
| SSS19 | Inspection |
| SSS20 | Analysis |
| SSS21 | Analysis |
| SSS22 | Inspection |
| SSS23 | Inspection |
| SSS24 | Analysis |
| SSS25 | Inspection |
| SSS26 | Inspection |
| SSS27 | Inspection |
| SSS28 | Inspection |
| SSS29 | Inspection |
| Power supply |  |
| SSS30 | Analysis |
| Operational environment |  |
| SSS31 | Analysis |
| SSS32 | Analysis |
| SSS33 | Analysis |
| Transportation |  |
| SSS34 | Inspection |
| SSS35 | Inspection |
| SSS36 | Inspection |
| SSS37 | Inspection |
| SSS38 | Inspection |
| SSS39 | Inspection |
| Maintenance |  |
| SSS40 | Inspection |
| SSS41 | Inspection |
| Documentation |  |
| SSS42 | Inspection |
| SSS43 | Inspection |
| SSS44 | Inspection |
| SSS45 | Inspection |
| Environmental |  |
| SSS46 | Analysis |
| SSS47 | Analysis |
| SSS48 | Analysis |
| SSS49 | Analysis |
| SSS50 | Analysis |

Table 4-2. The Verification Matrix

A separate documentation table for referenced verification documents is established in chapter 4, Verification, adjacent to the Verification Matrix to make it more convenient to fill in and read both the verification documentation table and the Verification Matrix.

If applicable, this column indicate the type of test and possible abbreviations shall be explained in this footnote. Examples of possible types of tests are 1) FAAT (First Article Acceptance Test) as a qualification test or 2) FAT or SAT (Factory / Site Acceptance Test) as a production test executed at Supplier’s / Purchaser’s site.