Contents

1	Lake and Riverbed Mapping	1

1.1	Data sources	1

2	Requirements	1

2.1	Products standards	1

2.2	Security clearance and notifying the Norwegian National Security Authority (NSM)	2

3	Requirements for collecting data and deliveries	2

3.1	General information about requirements for methodology and proposed solutions	2

3.2	Date and time of measurement work/date collection	3

3.3	Requirements for mapping whit ALB/ALS systems	3

3.3.1	Planning of the mapping campaign	3

3.3.2	Control surfaces, control profiles and cross lines	3

3.3.3	Point cloud accuracy requirements	4

3.3.4	Point cloud processing	4

3.3.5	Point cloud verification	4

3.4	Point cloud density Requirements	4

3.5	Positioning	4

3.6	High and projected coordinate reference	5

3.7	Data deliveries	5

3.7.1	Report on Climate and Environment impact	6

3.8	Important dates and projects follow-up	6

3.8.1	Contact person in NVE	7

3.9	Final report	7

Lake and Riverbed Mapping

In connection with natural hazards mapping, safety measures and analyses of environmental conditions in and along watercourses, good terrain data is needed both above and below water, such as for riverbeds, lake beds, riverbanks and adjoining areas of land.

This Specification of Requirements covers the special requirements that have been set for technical equipment, type and scope of data collection, data processing and data deliveries. It also covers the documentation and reporting of the data collection, processing, project follow-up and results.

Data sources

The mapping areas are covered by previously collected laser data along the watercourses on land, available from https://hoydedata.no.

Requirements

Products standards

The following standards and product specifications are relevant to this project

Standard for geografisk informasjon: Produksjon av basis geodata 2.0 See also attachment: Rapport - Publikasjon - Veileder

Satellittbasert posisjonsbestemmelse (Use consultation draft of the document: Geodetisk grunnlag – koordinatbaserte referansesystemer og kvalitetssikring av grunnlagspunkt 1.0)

Grunnlagsnett (Use consultation draft of the document: Geodetisk grunnlag – koordinatbaserte referansesystemer og kvalitetssikring av grunnlagspunkt 1.0)

Produktspesifikasjon Leveranse av bildedata, orienteringsdata og vertikalbildedekning fra flyfotografering (Produktspesifikasjon-Vertikalbilde)

Produktspesifikasjon Ortofoto 5.0 (Produktspesifikasjon Digitale ortofoto)

Produktspesifikasjon Punktsky and the point cloud must meet the accuracy requirements and point density requirement for a category Psky_1_ALB_B for ALB and category Psky_1_ALS_B for ALS.

Standards referenced in this document are available at https://www.geonorge.no/Geodataarbeid/standardisering/

Security clearance and notifying the Norwegian National Security Authority (NSM)

The assignment shall be carried out in accordance with the applicable regulations on the protection of classified defense secrets in Norway.
This assignment has been assessed as not classified.

None of the subprojects are covered by the Regulation on Classified Procurement because there are no prohibited areas within the areas that will be mapped in these subprojects. The NSM requires contractors to notify the NSM prior to data capture since sensor systems other than photos/videos will be used. The NSM can be notified electronically via the same website that provides an overview of prohibited areas: https://nsm.geodataonline.no/sensorapp.

More information from the NSM about airborne sensor systems and the duty to notify can be found here: https://nsm.no/fysisk/luftbarne-sensorsystemer/.

From 1 January 2024, terrain information about the seabed from 0 to 30 meters deep has been ungraded, albeit with some exceptions. See information about the new regulation here: https://lovdata.no/dokument/SF/forskrift/2023-12-15-2061. It is important that no data is collected deeper than 30 meters outside the riverbed.

Requirements for collecting data and deliveries

General information about requirements for methodology and proposed solutions

Contractors must use one or more ALB and ALS system as the data capture method for mapping the riverbed and the surrounding terrain.

It is important for all the subprojects to ensure some overlap between land and aquatic areas and topographic lidar acquisition will be required for land.

The following chapters set out expectations regarding product standards, quality, point density, accuracy, data deliveries and documentation.

These websites can be of help in analyzing the river stretches:

•	https://norgeibilder.no/  -> here you get access to aerial images from different years, which give indications about depth and visibility of the rivers

•	https://temakart.nve.no/tema/tverrprofil --> here you get access to bathymetric profiles of some of the rivers and for some stretches.

•	https://hoydedata.no/ --> Here you can find digital elevation models over Norway, that can be useful.

Date and time of measurement work/date collection

The contractor is responsible for planning the data collection so that it takes place at the best possible time for measurements in relation to weather, water discharge, ice, vegetation conditions, etc. Importantly, data collection must be carried out when there is no flooding. Contractors should therefore exhibit some flexibility regarding the timing of data capture and must plan the best time for data capture in close consultation with NVE. NVE must be informed a reasonable time in advance of the planned time for data capture.

Information about water flow and prognoses can be found on https://sildre.nve.no/ and on http://www.senorge.no, and flood warnings on Varsler om naturfarer i Norge | Varsom.no

Data collection must be carried out when there is no risk of ice. If the contractor faces difficulties collecting data before the deadline for data collection, the contractor must contact the client to discuss whether all or part of the project should be cancelled or continued.

Requirements for mapping whit ALB/ALS systems

Planning of the mapping campaign

The appropriate scanning campaign shall be planned in line with “Standard for geografisk informasjon: Produksjon av basis geodata 2.0” and the attachment, unless otherwise is specified. Any deviations from this standard beyond this must be explained and justified.

Control surfaces, control profiles and cross lines

In order to remove a potential vertical datum shift, the contractor must test the point cloud using GCPs inside control surfaces, which must be surveyed and documented within each individual stretch that is mapped. This must comply with the requirements (“krav” in Norwegian) no. 34, 35, 36 in the standard: “Standard for geografisk informasjon: Produksjon av basis geodata 2.0”.

For surveys utilizing alternative processing approaches, the contractor must describe how they will use the GCPs in processing in order to ensure a rigid and accurate point cloud.

In order to identify a potential horizontal datum shift, the contractor must test the point cloud using GCPs inside control profiles, which must be surveyed and documented within each individual stretch that is mapped. This must comply with the requirements (“krav” in Norwegian) no. 37, 38, 39 in the standard: “Standard for geografisk informasjon: Produksjon av basis geodata 2.0”.

For classical ‘lawnmower’ surveys, crosslines are required to strengthen the strip adjustment. For ‘corridor surveys’, following a valley bottom or a river, the contractor must describe how they will ensure a rigid and accurate point cloud without the use of crosslines.

Point cloud accuracy requirements

In addition to the accuracy requirements listed in Produktspesifikasjon: Punktsky 1.0.3 (geonorge.no) for the categories Psky_1_ALB_B for ALB and category Psky_1_ALS_B for ALS, this has to be followed:

To ensure no datum shift between the point cloud delivery and the surrounding Norwegian National Elevation Model, each area must be tested and adjusted to fit the current terrain model in each area. The reference point clouds are available for download at hoydedata.no. The final report (see chapter 3.9) must state the agreement between the point cloud and the national elevation model both pre and post adjustment. The National Elevation point clouds used in the adjustment must be listed in the report. This must comply with the requirements (“krav” in Norwegian) no. 43 and 44 in the “Standard for geografisk informasjon: Produksjon av basis geodata 2.0”.

Point cloud processing

The contractor needs to document the approach to strip adjustment ensuring point cloud rigidity and accuracy, see requirements no. 41 and 42 in the “Standard for geografisk informasjon: Produksjon av basis geodata 2.0”.

Point cloud verification

In order to identify poor strip adjustment a dH plot (homogeneity plot) must be produced showing the strip-to-strip difference in height. See requirement no. 45 in the “Standard for geografisk informasjon: Produksjon av basis geodata 2.0”.

Point cloud density Requirements

The acquisition must be carried out in a way ensuring the following point density requirement:

Minimum point density of 5 points/m2. This applies both for terrain data above water (class 2) and for classified seafloor / riverbed points (class 40).

Minimum point density must be met for 80% of all 2m x 2m cells within a 10m x 10m area (Produktspesifikasjon: Punktsky 1.0.2 (geonorge.no), chapter 7.1). NB! Be aware that this deviates from what the Produktspesifikasjon: Punktsky 1.0.2 requires for category Medium/B (Psky_1_ALS_B and Psky_1_ALB_B). For ALB, the density requirement applies to 2m below the water surface at the time of data capture, and we are aware that the density will decrease with increased depth.

If the 80% requirement for point density is not met, the achieved percentage shall be stated, broken down by data above water and data classified as seafloor/riverbed.

Positioning

The georeferencing can be carried out using CPOS RTK positioning. Areas with poor mobile connection can be encountered and the bid must outline an alternative positioning solution ensuring accurate georeferencing.

High and projected coordinate reference

The following applies to all data deliveries in this project:

Projected coordinate reference: XY: EUREF89 UTM32N

Height datum: NN2000 (based on HREF2018B_NN2000_EUREF89)

Data deliveries

Data delivery shall include the following:

- LAZ files that cover the entire mapping area (common for ALB and ALS), delivery divided by map sheet (1:1000) and as described in Produktspesifikasjon: Punktsky 1.0.3, chapter 11.1. The Datum and Projection must be: EPSG 5972 (XY: EUREF 89 UTM32N, Z: NN 2000). The classification must be done according to the following minimum classes:

(see also chapter 14 Appendix A in Produktspesifikasjon: Punktsky 1.0.3.)

1 – Unclassified points on land

2 – Ground - Terrain points on land

7 - Noise

17 – Bridge Deck

21 - Snow

40 – Seafloor / riverbed Bathy

41 – Water surface Bathy (sea/river/lake surface elevation at time of data acquisition)

NB! Be aware that this deviates from what the Produktspesifikasjon: Punktsky 1.0.3 (geonorge.no) requires, as it also has classes 42 and 45 as compulsory classes.

- Orthophoto – the contractor must strive to deliver orthophoto based on images collected at the same time as the laser scanning. If the date of data collection is during early winter, this requirement is not absolute and should be discussed with the client before planned data collection. The orthophotos shall comply with the requirements for temporary orthophoto stipulated in chap. 8.6.4 in “Produktspesifikasjon for ortofoto” [Product Specification for Orthophotos], current version. A minor adjustment of bridges and other technical installations shall be carried out. Request for GSD 10, minimum requirement for GSD 25. The contractor must specify the resolution (GSD) it can deliver in its tender. The ortophoto must be delivered as a GeoTIFF with EPSG:25832 projection with JPG compression, or as an ECW file. The contractor must also upload the complete delivery to “Norge i bilder” (norgeibilder.no) via the management solution, one upload for each river.

- RGB colored point cloud for the entire mapping area, using the RGB values from the imageries.

- The following rasters must be produced from the obtained point clouds and delivered as a lossless compressed GeoTiffs with EPSG: 25832 projection:

- Density Grid (Class 2 and 40) at 2x2m resolution

- 1m DTM grid generated from a TIN model (natural neighbors) based on Class 2 and 40

- 25cm DTM grid generated from a TIN model (natural neighbors) based on Class 2 and 40

- Delivery of the ALB and ALS data to the Norwegian national management solution hoydedata.no, as described in Produktspesifikasjon: Punktsky 1.0.3 chapter 11, one upload for each river. This includes all necessary data and metadata about the project:

- Delivery of point cloud, see chapter 11.1.

- Metadata, see chapter 11.2.

- Metadata – coverage overviews and flightlines for each individual mapping area must be delivered in SOSI, shapefile format or ESRI file geodatabase.

- Map overview of data holes larger than c. 50 m2 (polygon) for each individual mapping area that documents any missing data due to failures or other reasons for missing data with a unique reference to textual explanation in the report. The polygons must be delivered in SOSI, shapefile format or ESRI file geodatabase.

- Trial delivery: the main deliveries for an agreed short river stretch. Main deliveries are:

- LAZ-files as described in point A.

- Temporary orthophoto as described in point B.

- RGB colored point cloud as described in point C

- Rasters as described in point D

- Metadata as described in point F, should be delivered for the whole subproject and not only for a short stretch

Report on Climate and Environment impact

- When data collection has been completed, the supplier shall report the type of aircraft used for the execution of the assignment (registration number), along with the corresponding flight time (in decimal hours) and fuel consumption (in liters) per aircraft. The report shall be submitted to the project manager at NVE.

- Any deviations from the measures and routines described in section 4 in Tender Documentation shall be specified in the monthly report, along with a justification.

Important dates and projects follow-up

The deadline for data collection has been set to 15.08.2027 for all subprojects, but data collected in 2026 is preferred. Contractors will have a maximum of two weeks to fix problems after feedback on the delivery is provided from the client.

The trial delivery shall include all the main data deliveries for a short river stretch (c. 1-2 km), see chap.2.7, point H.

The client has set out the following timeframes for the process:

| Activity | Date and time |
| --- | --- |
| Absolute deadline for data collection | 15.08.2027 |
| Trial delivery of sub-areas | 8 weeks after data collection. |
| Feedback on trial delivery from NVE/Mapping Authority | Two weeks after delivery of trial delivery. |
| Final delivery, data and report. | 12 weeks after the last data collection. |

During the entire project and up to delivery, the contractor shall keep the client continuously informed of its progress, as well as any factors of relevance for the execution, quality and delivery at the right time.

Start-up meetings via videoconferencing are planned (preferably Teams). Or, if desired, the meeting can be in-person meeting at NVE's main office in Oslo.

Time must also be set aside for monthly video-conferencing meetings with the client during execution of the project.

The contractor must submit short status “reports” (email is sufficient) on a weekly basis, preferably every Friday. As a minimum, these must contain a short summary of the status of the measurement work, processing of data, and preparation of deliveries or the reasons for any lack of data collection, as well as an overview of mapped areas for each stretch (screenshot attached to the email is sufficient). The report/email must also provide information about activities planned for the current week.

The contractor shall, within a reasonable space of time, and a maximum of one week, inform the client of any challenges relating to the collection of data due to rapids, ponds, vegetation, shallow areas, weather, ice, etc.

Contact person in NVE

Name: Linn Varhaugvik Arto

Title: Senior engineer

Adress: PB 5091 Majorstuen, 0301 Oslo

Telephone: +47 44 87 79 26

Email: liva@nve.no

Final report

A final report written in Norwegian (bokmål) or English in PDF format (without read-only protection). The LiDAR survey shall be documented in accordance with the requirements set out in “Standard for geografisk informasjon: Produksjon av basis geodata 2.0” chapter 7.5. In addition, and importantly, the report must document whether the results meet the requirements for point density and accuracy, divided into ALB and ALS data. In areas where these requirements were not met, the report needs to explain why and to show where these areas are.