Scope of Work (SOW) – Acquisition of new coaches Class FDe (Diesel-Generator wagon)

1. Purpose

Norske tog AS invites suppliers to submit offers for the delivery of new Diesel-generator containers or complete Diesel generator wagons, intended for replacement of/on existing Finke/FDE diesel generator wagons. The goal is to secure reliable and redundant power supply for 5–7 years.

2. Scope of Supply

2.1 Power Generation System

- Minimum two (2) independent and identical diesel motor/generator sets,

- Total generated output approx. 600 kWe continuous output to the train heating/power system.

- The output to train heating shall be 1000VAC 50 Hz.

- Fuel Capacity for continuous operation approx. 36 h

- All sets shall be fully independent, allowing single-unit operations to maintain power supply in case one unit fails (full operational redundancy).

- Each generator set shall at least include:

- Diesel Engines

- Generators

- Control system

- Exhaust system with improved emission and noise control

- Diagnostics system

- Hour counter

- Fire detection according to Fire Requirements EN 45545

- The client will place emphasis on the performance (efficiency) of the offered engine/generator system. The client will particularly focus on performance at and around the expected load, and will take into account interaction with support systems.

- The generator sets shall be able to run on biodiesel, minumum B20. Compatibility with higher blends up to B100 shall be described.

Low maintenance impact and/or need for modification when using higher blends will be viewed favourably in the evaluation.

- Supplier to describe maintenance load (frequency) and cost estimates for the operator. This will consititute part of the evaluation.

2.2 Control and Monitoring System

- To synchronize the two generators to the same grid, there is a need for a way to

synchronize the generators.

- New control system suitable for multi-generator configuration.

- Capability for:

- Manual or automatic switching between generators

- Monitoring of engine and generator health parameters

- Alarms for critical states

- Integration with existing Finke interfaces where feasible

- The supplier will be evaluated based on the extent to which the offer includes innovative solutions or improvements that contribute to Norske tog's climate and environmental goals, for example in energy efficiency, operational optimization, life extension, or reduced resource consumption.

This applies to all systems.

2.3 Container and Mechanical Structure

The supplier shall deliver a new container designed for installation on the existing Finke bogies. The requirements include:

- A robust container structure with all required reinforcements and adaptations for the existing underframe.

- Improved ventilation and thermal management to ensure stable operation of the engines and generators.

- Thermal and acoustic insulation to reduce heat, vibration, and noise.

- Mounting interfaces compatible with the existing bogies and wagon structure.

- Solution shall respect the railways NO1 loading gauge.

- The supplier will be evaluated on the extent to which reused, recycled and/or low-carbon materials are used in the delivery. This must not come at the expense of safety. Assessments will be made in accordance with the waste hierarchy. Documentation such as a life cycle assessment (LCA) or environmental product declaration (EPD) is advantageous.

- Container should have sufficient sump capacity to prevent minor leaks.

2.4 Noise Reduction

The container and all installed components shall be designed to minimize noise emissions, both for:

- Personnel working near the wagon

- Passengers in connected coaches

- Surroundings during stationary operations (e.g., overnight stops)

TSI noise must be respected

2.5 Options Requested

Suppliers shall price the following options in addition to the base delivery:

Option 1 – Integrated Air Compressor

Supply and installation of an air compressor with Airdryer capable of supplying compressed air to passenger coaches in the event of locomotive failure.

Requirements:

- Compressor shall be operated manually from inside the container.

- System to include:

  - Local control panel

  - Safety features (e.g., overpressure protection)

  - Necessary hoses/fittings for interface to train air system

- Capacity requirement: Adequate to sustain brake and system air pressure during stationary or low-speed recovery operations. Approx. 600l/min at 10 bar.

Option 2 – Delivery of Complete Finke Unit (Container + Wagon)

Supplier shall provide an option for delivery of a completely new Finke unit, including:

- New container (as described above)

- New or fully refurbished goods-wagon-type chassis suitable for mounting the container

This option shall be quoted separately and include all necessary design work, structural verification and documentation.

Option 3 – One extra Finke according to SOW

Option 4 – Shutdown of engines/generators

- Provision should be made for shutting down engines/generators from the locomotive. This applies only to modifications in the “Finken” unit and not to the locomotive itself.

2.6 Documentation Requirements

Supplier shall provide:

- Technical specification sheets for engines, generators, control systems and exhaust systems

- Drawings of the container and main components

- Weight distribution and load calculations

- Fuel consumption data, preferably including a specific fuel consumption curve.

- Emission and noise documentation

- Maintenance instructions and recommended spare parts

- Installation manual for integration on existing Finke underframe and bogies

- An estimate of the climate impact from the provided system, including a breakdown of the weight of major components. Life Cycle Assements (LCA) or environmental priduct declarations (EPDs) are appreciated.

- The supplier will be evaluated based on how waste is handled, including the systematic approach to reuse, sorting, recycling, and material recovery of metal, chemicals, and other residual materials, as well as a documented high proportion that goes to approved reuse or material recovery.

- The supplier will be evaluated based on the quality, traceability, and verifiability of the environmental data provided in connection with the delivery (e.g., waste, energy, transport, or material use), including methodology and documentation.

2.7 Testing and Acceptance

- Functional testing of generator sets prior to shipment

- FAT documentation

- Test run, round-trip Trondheim–Bodø with wagons without faults related to the Finke unit

2.8 Delivery Schedule

Supplier shall specify:

- Delivery plan including milestones and delivery dates for each "finke"

- The client will place emphasis on the extent to which the supplier uses low- or zero-emission solutions for the transport of engines, generators, and components included in the delivery.

2.9 Warranty Requirements

Minimum 24-month warranty on all delivered components, with options for extended warranty.