Saksnummer: 26/144644

Dato: 12.05.2026

Kravspesifikasjon for OPCUA-grensesnittet mellom dataloggere og Trafikkdatasystemet

Kravspesifikasjon for grensesnitt mot Trafikkdatasystemet

Table of contents

Introduction	2

Definitions	3

1.	Data collection principle	4

1.1	Approval of the connection between the RTDCE and NPRA Traffic data system	4

1.2	RTDCE system integration	4

1.3	RTDCE buffer and history read	5

2.	General OPC UA requirements	5

3.	OPC UA address space	7

3.1	NodeId	7

3.2	RTDCE nodes	7

3.2.1	Equipment nodes	8

3.2.2	Firmware nodes	10

3.2.3	GNSS nodes	12

3.2.4	HAConfiguration	13

3.2.5	Network nodes	14

3.2.6	NTP nodes	15

3.2.7	Sensor nodes	16

3.2.8	Time nodes	18

3.3	Server status nodes	19

3.4	Traffic event nodes	21

3.4.1	Bicycle event nodes	23

3.4.2	Motor vehicle event nodes	24

3.5	Device event nodes	26

3.5.1	Power event nodes	29

3.5.2	Voltage event nodes	29

3.5.3	Sensor event nodes	29

3.5.4	Clock event nodes	30

3.5.5	Firmware event nodes	31

3.6	Vendor specified nodes	31

Change log

| Date | ID | New | Old |
| --- | --- | --- | --- |
| 2026-05-12 | 18 Remote URL | HTTPS endpoint | HTTP endpoint |

Introduction

This document covers the requirements related to Roadside Traffic Data Collection equipment.

The document describes the OPC UA interface used for communication between RTDC equipment and the NPRA Traffic data system.

This document serves the following purposes:

- It shall be used as the basis for the response to the Tender documents.

- Development of a test plan and at a later stage, detailed test procedures, for qualifying the equipment on the requirements in this document.

The audience for this document is:

- The bidders for the RTDC equipment

- The Contractor’s personnel who is involved in realising the system being: HW/SW engineers involved in development, test engineers and project management.

- The customer’s personnel who are involved in the commissioning of the delivered RTDC equipment.

Definitions

The glossary in Table 1 is an alphabetical list with the explanation of terms and phrases used in the specifications.

Table 1 Alphabetical list

| DHCP | Dynamic Host Configuration Protocol |
| --- | --- |
| GNSS | Global Navigation Satellite System |
| NPRA | Norwegian Public Roads Administration |
| OPC UA | OPC Foundation Unified Architecture communication protocol |
| RTDCE | Roadside Traffic Data Collection Equipment |
| VBV | Vehicle-by-vehicle |
| TCK | Test Compatibility Kit |

Data collection principle

The main principle for NPRA’s roadside traffic data collection is that the data must be processed on a vehicle-by-vehicle (VBV) basis. Accumulation of data will take place in the NPRA’s Traffic data system, which is not included in this tender.

Equipment offered in this tender will be placed into a Data Acquisition Network, owned by NPRA. The Data Acquisition Network is a separate, private and isolated VPN. NPRA routers are deployed roadside, extending the network from the roadside cabinets to our central data centre.

Data is transmitted through use of the OPC UA M2M communication protocol. The updated standard is freely available for download at OPC Foundation website. A number of third party vendors have developed tools to support the standard, notably SDKs/toolkits for various platforms with methods that support the official specification. These are available at https://opcfoundation.org/certified-products.

Equipment in the NPRA Data Acquisition Network will not access the public internet. Inbound VPN access can be arranged, allowing troubleshooting or similar tasks.

Approval of the connection between the RTDCE and NPRA Traffic data system

A test compatibility kit (TCK) mimicking the NPRA Traffic data system client side will be made available, so that vendors may verify the compatibility of the RTDCE’s OPC UA address space model.

The implementation may also be verified using the UAExpert application.

The NPRA Traffic data system is developed using Prosys PMS Ltd’s OPC UA Java SDK.

RTDCE system integration

All traffic data registered by the RTDCE must be exposed on an OPC UA server. This server must be set up with a browsable address space just like NPRA’s model, as described in this document.

In addition to the specified address space, the vendor may supply their own additional nodes in the address space with values for their specific equipment. This is only to be used if the specified address space does not support some nodes/methods. Note that these additional nodes will not be supported by the TCK for testing.

RTDCE buffer and history read

When network is not available, the RTDCE must not buffer real-time events, but store them in the archive. When the network connection is back up, the archived events will be fetched by the history read feature of the OPC UA specification. When history read is used for larger amounts of data, the continuation points feature must be used to ensure completeness of the data transfer from the RTDCE archive.

The RTDCE must archive data in a way that makes it possible to use history read at any time interval, also through hourly shift.

General OPC UA requirements

General requirements for the OPC UA server is described in Table 2.

Table 2 General OPC UA requirements

| ID | Subject | Requirement description | Requirement gradation |
| --- | --- | --- | --- |
| 1 | OPC UA specification compliance | The RTDCE’s OPC UA server implementation must follow the official specification as published by the OPC Foundation. Any deviation must be explained to and approved by the NPRA. | Mandatory |
| 2 | Certified SDK | The Bidder shall use an OPC UA Server SDK/Toolkit certified by the OPC Foundation, available at: https://opcfoundation.org/certified-products. The SDK must be named in the tender. | Mandatory |
| 3 | Network configuration | The communication between the RTDCE and NPRA’s central system must be based on OPC UA over TCP/IP. | Mandatory |
| 4 |  | The equipment shall support IPv4 and IPv6. | Mandatory |

| 5 |  | The RTDCE must by default be enabled for allocation of IP address and network parameters using DHCP. DHCP shall be enabled at every boot or reboot. If an IP is not made available, the unit must retry continuously until an IP is given. In the case of network loss after acquiring an IP, the unit shall never fall back to a static IP, but should ask for an IP over DHCP every 10 minutes indefinitely. | Mandatory |
| --- | --- | --- | --- |
| 6 | Failsafe bootload | The equipment shall have a failsafe bootloader, which allows the device to function with a basic vendor-provided configuration, or the last known good configuration, after a failed upload and/or flashing of new firmware. | Mandatory |
| 7 | RTDCE method call response | For the OPCUA method calls, the RTDCE OPCUA server shall implement the relevant response codes described in the OPCUA specification. Example for history read operations: Correct use of the response codes Bad_NoData and Good_NoData. Bad_NoData is used when something goes wrong and no data can be returned. Good_NoData is used when there simply is no data matching the request. | Mandatory |
| 8 | History read | The RTDCE must archive data in a way that makes it possible to use history read at any time interval, also through hourly shift. | Mandatory |
| 9 | Documentation | If Contractors wish to supply information via OPC UA nodes that is not specified, this information must be put in a separate OPC UA namespace url (to ensure separation of Contractor address spaces). The Contractors must also supply a UA Modeler file describing the nodes with all relevant information under Instance, Additional Attributes and so forth. In the event that new firmware/software releases alters the namespace or adds new modes/methods, both the document and UA modeler file must be updated and provided as part of the change documentation. | Mandatory |
| 10 | Monitored items | Each subscription must handle at least 100 monitored items. | Mandatory |

OPC UA address space

This chapter gives a detailed description of the OPC UA address space to be implemented on the RTDCE. See the attachment “NPRA_OPC_UA_adress_space.zip” for the complete address space. The attachment can be viewed in the UaModeler software, which is available at https://www.unified-automation.com/products/development-tools/uamodeler.html. The Contractor is strongly urged to use the UaModeler software to acquire a more comprehensive understanding of the address space used by the NPRA. Also note that the address space will be developed over time as more knowledge is acquired by the NPRA/the Suppliers – all alterations will be made known to the Contractor.

NodeId

NodeIds in the address space should match those of the reference model. Nodes residing within custom contractor address spaces are exempted from this rule.

RTDCE nodes

Relevant information about the RTDCE itself is to be given as shown in Figure 1. The RTDCE is a BaseObjectType with folders containing nodes, as specified below.

Figure 1 RTDCE nodes

Equipment nodes

Figure 2 shows the nodes in the OPC UA node tree.

See also Chapter 3.3 Server status nodes for other relevant nodes for RTDCE information.

Figure 2 Equipment nodes

Table 3 gives further details about the nodes.

Table 3 Equipment nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 11 | Restart | Method | A method that initiates a soft reboot of the RTDCE where the operating system closes all programs and finalizes all pending input and output operations before rebooting. | Mandatory |
| 12 | SerialNumber | Variable | The vendor specific serial number of the RTDCE. | Mandatory |
| 13 | SiteID | Variable | A readable and writable node holding a String (maximum 7 digit number). This value will be set by the NPRA Traffic data system client side when the RTDCE is deployed roadside. | Mandatory |
| 14 | StartTime | Variable | UTC Timestamp for the last start-up time of the OPC UA server (RTDCE). | Mandatory |
| 15 | Temperature | Variable | Value of RTDCE’s ambient or internal temperature. | Optional |
| 16 | TrafficTypes | Varaiable | String array which denotes what types of traffic the RTDCE may register: - vehicles - bicycles - pedestrians | Mandatory |
| 17 | Voltage | Variable | Giving the present voltage value of RTDCE’s power supply. | Mandatory |

Firmware nodes

Figure 3 shows the nodes in the OPC UA node tree.

Figure 3 Firmware nodes

Table 4 gives further details about the nodes.

Table 4 Firmware nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 18 | RemoteUrl | Variable | A read and writable node to contain the website URL, specified by the NPRA, with the path where the RTDCE can download the published firmware binaries to be used for upgrade. The URL will point to an HTTPS endpoint in our back-end solution. It will not point directly to the firmware files themselves, but the endpoint will produce the firmware files when called. The URL has arbitrary format and length, and includes query parameters. The filename used by the vendor will be part of the content disposition header. | Mandatory |
| 19 | Rollback | Method | A method that reverts to the latest known good firmware. Latest known good firmware must be stored on the RTDCE so that a restore can be done directly from the unit. | Mandatory |
| 20 | Upgrade | Method | A method that initiates: - download from RemoteUrl, - verification of the downloaded file, - flash, reboot, and self-tests for the new firmware. Both successful and failed firmware updates must be logged. | Mandatory |
| 21 | CurrentVersion | Variable | A readable node denoting the version number of the currently running firmware/software for the RTDCE. If the RTDCE runs on more than one firmware/software internally, the versions should be separated by “/”. Version numbers must be composed of numbers and dots only. If Contractors need separate firmware/software modules internally, a single version number shall still be exposed to the Customer (as a wrapper or the like) for ease of use at node BuildInfo/SoftwareVersion. | Optional |

GNSS nodes

Figure 4 shows the nodes in the OPC UA node tree.

Figure 4 GNSS nodes

Table 5 gives further details about the nodes.

Table 5 GNSS nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 22 | GnssTime | Variable | Last UTC time value from GNSS source, e.g. GPS or GLONASS used by the RTDCE. | Mandatory |
| 23 | Location | Variable | Four values giving altitude, latitude, longitude and timestamp for the present location of the RTDCE. The datatype is String. Positions from GNSS shall use decimal degrees with at least five decimals for latitude and longitude, indicating the position of the RTDCE. | Mandatory |

HAConfiguration

Figure 5 shows the nodes in the OPC UA node tree.

Figure 5 HAConfiguration

The use of the Historical Access implements the data types as described in the OPC UA specification.

Network nodes

Figure 6 shows the nodes in the OPC UA node tree.

Figure 6 Network nodes

Table 6 gives further details about the nodes.

Table 6 Network nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 24 | Ipv4Address | Variable | RTDCE’s IPv4 adress. | Mandatory |
| 25 | Ipv6Address | Variable | RTDCE’s IPv6 adress. | Mandatory |
| 26 | MacAddress | Variable | RTDCE’s MAC adress. | Mandatory |

NTP nodes

Figure 7 shows the nodes in the OPC UA node tree.

Figure 7 NTP nodes

Table 7 gives further details about the nodes.

Table 7 NTP nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 27 | NtpServer | Variable | Writable node for setting the NTP server address used by the RTDCE. The default setting must be one of these: - ntp.vegvesen.no - ntp2.vegvesen.no - 146.2.0.71 - 146.2.0.142 | Mandatory |
| 28 | NtpTime | Variable | Last time value given by the NTP server used by the RTDCE. | Mandatory |

Sensor nodes

Figure 8 shows the nodes in the OPC UA node tree. The Sensors node is an array which can be used to describe from 1 to n sensors, depending on sensor type. Contractors are encouraged to further supplement sensor information within their own namespace, if suitable.

Figure 8 Sensor nodes

Table 8 gives further details about the nodes.

Table 8 Sensor nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 29 | Lanes | Variable | An array denoting which lane(s) the sensor is covering. | Mandatory |
| 30 | Frequency | Variable | Gives the frequency of the loop. May be skipped for sensor where frequency is not applicable. This node must be updated minimum every 5. minutes. | Mandatory where applicable |
| 31 | Kind | Variable | Denotes kind of sensor (e.g. radar, loop, piezo, camera etc). | Mandatory |
| 32 | Id | Variable | Denotes the individual sensor ID number. | Mandatory |
| 33 | Status | Variable | Indicates the current status of the sensor. If functioning normally, the status should be “ok”. If not functioning normally, the status could be “not ok” or similar. For example, inductive loop status could be: “open”, “shorted”. The Bidder must inform about all status alternatives in the tender response. | Mandatory where applicable |

Time nodes

Figure 9 shows the nodes in the OPC UA node tree.

Figure 9 Time nodes

Table 9 gives further details about the nodes.

Table 9 Time nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 34 | CurrentTime | Variable | Two values giving the current time source (GNSS or NTP) and the time value of the clock that is used to set the timestamp on the registered vehicles. | Mandatory |

Server status nodes

Figure 10 shows the Server node tree as specified by the OPC UA standard.

Figure 10 Server status nodes

The following nodes are mandatory to use as explained in Table 10.

Table 10Server status nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 35 | BuildInfo/ ManufacturerName | Variable | Name of the RTDCE’s manufacturer. | Mandatory |
| 36 | BuildInfo/ ProductName | Variable | Generic device name of the RTDCE. | Mandatory |
| 37 | BuildInfo/ SoftwareVersion | Variable | Version number of the currently running software (firmware). | Mandatory |
| 38 | StartTime | Variable | UTC Timestamp for the last start-up time of the server (RTDCE). (This node is replaced by the StartTime node under Equipment.) | Optional |

Traffic event nodes

The traffic events are modelled as events under Types/EventTypes/BaseEventType/TrafficEvent.

Figure 11 Traffic event

The nodes under TrafficEvent are as shown in Figure 12.

Figure 12 Vehicle event nodes

All events must have the values described in Table 11:

Table 11 Vehicle event requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 39 | Direction | Variable | Denotes which direction the registered event had. “Boolean” value denoted by: “1” for all traffic with positive speed in odd numbered lanes, and negative speed in even numbered lanes. “2” for all traffic moving in opposite direction to “1”. | Mandatory |
| 40 | Lane | Variable | Specifies which lane the event is registered in. For lane numbering and directions, see the main tender. | Mandatory |
| 41 | Sequence Number | Variable | Each RTDCE must number each registered and approved event with a unique sequence number. | Mandatory |
| 42 | SiteID | Variable | Each event must also carry the SiteID, which is the value of the Equipment variable SiteID. | Mandatory |
| 43 | Timestamp | Variable | Each event must have its timestamp in UTC milliseconds. | Mandatory |

In addition to these values, the events must contain the values as described below, according to their event type.

If, for some reason such as due to failure on part of the sensors, the events are registered with just the values in table 11, we still want them stored and transferred as normal.

Bicycle event nodes

Bicycles events must have the following values described in Table 12, in addition to the ones mentioned in Table 11:

Table 12 Bicycle event nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 44 | Class | Variable | Classification of the registered event. | Mandatory |
| 45 | ClassQuality | Variable | A quality parameter denoting the accuracy of the class assigned to the event. | Optional |
| 46 | Gap | Variable | Time gap back to front between last event and current event. | Mandatory |
| 47 | Length | Variable | Measured length of registered event. | Optional |
| 48 | Speed | Variable | Measured speed of registered event. Negative speeds are defined on events travelling in the opposite direction. | Mandatory |
| 49 | SpeedQuality | Variable | A quality parameter denoting the accuracy of the measured speed. | Mandatory |
| 50 | StrengthSensorN | Variable | Signal strength for sensor N. One such node per sensor per event. | Mandatory where applicable |

The resolution and accuracy of the parameters are given in the main tender.

The vendor may add other nodes for relevant values if applicable.

Motor vehicle event nodes

Motorized vehicle events must have the following values, in addition to the nodes mentioned in Table 11:

Table 13 Motor vehicle event nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 51 | Class | Variable | Classification of the registered event. See information and requirement about classification in 1 – requirement specification – RTDCE regarding vehicles. | Mandatory |
| 52 | ClassQuality | Variable | A quality parameter denoting the accuracy of the class assigned to the event. | Mandatory |
| 53 | Gap | Variable | Time gap back to front between last event and current event. | Mandatory |
| 54 | Length | Variable | Measured length of registered event. | Mandatory |
| 55 | LengthQuality | Variable | A quality parameter denoting the accuracy of the measured length. | Optional |
| 56 | Speed | Variable | Measured speed of registered event. Negative speeds are defined on events travelling in the opposite direction. | Mandatory |
| 57 | SpeedQuality | Variable | A quality parameter denoting the accuracy of the measured speed. | Mandatory if 2+ samples |
| 58 | Axles | Variable | Measured number of axles on the vehicle. | Optional |
| 59 | AxleSpacing | Variable | Measured distance between adjacent axles of the vehicle. | Optional |
| 60 | AxleWeight | Variable | String array which contains axle weight for each vehicle axle in order. | Optional |
| 61 | Weight | Variable | Measured total weight of the vehicle. | Optional |
| 62 | StrengthSensorN | variable | Signal strength for sensor N. One such node per sensor per event. | Mandatory where applicable |

The resolution and accuracy of the parameters are given in the main tender.

Device event nodes

Device events are new events (2021) and will be used in the Traffic Data system for automatic alarms and information about time periods where traffic data needs a manual control. The device events are modelled as events under Types/EventTypes/BaseEventType/DeviceEvent.

Figure 13. Device Event

Figure 14. Device event nodes

All events must have the values described in Table 114:

Table 144 Device event requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 63 | DeviceSerialNumber | Variable | Each event must carry the serial number of the RTDCE, which is the value of the Equipment variable SerialNumber. | Mandatory |
| 64 | SiteID | Variable | Each event must carry the SiteID, which is the value of the Equipment variable SiteID. | Mandatory |
| 65 | DeviceEventID | Variable | Each RTDCE must number each registered and approved event with a unique sequence number. Note: This variable is independent of the TrafficEvent variable! | Mandatory |
| 66 | Timestamp | Variable | Each event must have its timestamp in UTC milliseconds. | Mandatory |

In addition to these values, the events must contain the values as described below, according to their event type.

Power event nodes

Power events must have the following values described in Table 125, in addition to the ones mentioned in Table 114:

Table 155 Power event nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 67 | PowerStatus | Variable | “Boolean” value denoted by: “1” for when the RTDCE is powered on. “2” for when the RTDCE is powered off. | Mandatory |

Voltage event nodes

Voltage events must be sent every 30 min, only if the voltage is less than 12V. Voltage events must have the following values described in Table 126, in addition to the ones mentioned in Table 114:

Table 166 Voltage event nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 68 | VoltageValue | Variable | Value of current voltage, which is the value of the Equipment variable Voltage. | Mandatory |

Sensor event nodes

Sensor events must be sent when the sensor frequency is measured outside the normal frequency range for the equipment and when the sensor frequency again is measured inside the normal frequency range.

Sensor events must have the following values described in Table 127, in addition to the ones mentioned in Table 114:

Table 177 Sensor event nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 69 | SensorID | Variable | Denotes the individual sensor ID number, which is the value of the Sensor variable ID. | Mandatory |
| 70 | SensorStatus | Variable | Indicates the current status of the sensor, which is the value of the Sensor variable Status. | Mandatory if applicable |
| 71 | SensorFrequency | Variable | Value of current frequency, which is the value of the Sensor variable Frequency. | Mandatory if applicable |

Clock event nodes

Clock events are sent every time the clock is synchronised. Clock events must have the following values described in Table 128, in addition to the ones mentioned in Table 114:

Table 188 Clock event nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 72 | NewTime | Variable | Value of new time, after the clock synchronisation. | Mandatory |
| 73 | OldTime | Variable | Value of old time, the clock had before it was synchronized. | Mandatory |

Firmware event nodes

Firmware events are sent when the firmware is changed. Firmware events must have the following values described in Table 129, in addition to the ones mentioned in Table 114:

Table 199 Firmware event nodes requirements

| ID | Node Name | Node Class | Explanation | Requirement gradation |
| --- | --- | --- | --- | --- |
| 74 | FirmwareVersion | Variable | Value of current firmware, which is the value of the ServerStatus variable BuildInfo/ SoftwareVersion. | Mandatory |

Vendor specified nodes

The vendor may add other nodes for relevant values the RTDCE registers, if applicable.