Appendix 1 Buyer’s requirement specification

The Buyer shall complete Appendix 1

## Purpose of the procurement

Conservation Studies at the Department of Archaeology, Conservation and History (IAKH) wishes to acquire a high-resolution infrared reflectography (IRR) camera, in order to maintain its ability to conduct top-level research in technical art history and to provide essential teaching in art conservation based on technical analysis of artworks. The camera will primarily be used to examine the underlayers of painted works of art and is intended to replace the section’s “artist’s camera,” a multispectral camera that has been in use in the section for nearly 20 years.

Suppliers are invited to answer the Buyer’s requirement specification in Appendix 2.

- General requirements

- Warranty of at least 12 months is required.

- If you offer more than 12 months of warranty, please detail the offer. If there is additional price included in this, please specify in Appendix 3a Price Matrix.

- The proposed product should preferably be a dedicated solution for art and cultural heritage imaging.

- Camera and Sensor Specifications

- Must have Indium Gallium Arsenide sensor (InGaAs).

- Must work into far IR (above 1.5 µm wavelength is required).

- Must support high dynamic range (HDR) for enhanced differences in contrast (above 10 000: 1).

- Software and Image Processing

- Shall have acquisition software compatible with MS Windows that allows for automatic stitching.

- Can offer large area scanning, with resulting high-resolution image after acquisition and stitching. Please detail how the acquisition and stitching works.

- Accessories and Optics

- Compatible macro lens(es) for detailed imaging. At least one, if more, please detail and specify.

- Compatible filters for imaging specific IR wavelengths. Please offer your list of compatible filters.

- Please include details on dedicated lighting solutions.

- Include mounting (a tripod) and travel cases for the system

- If necessary for operating the camera and its associated software, please include an offer for an optional laptop. The price shall be listed in Appendix 3a - Price matrix.

- Support, Service and Repairs

- Suppliers shall describe what they offer in terms of support, service, repairs, and software updates.

- Sustainability

- Expected lifespan of 7-10 years is required. Suppliers shall specify the expected operational lifespan of the offered instrument under normal maintenance conditions and provide documentation supporting this estimate, including information on service support and spare parts availability.

- Suppliers shall describe the end-of-life arrangements for the instrument, including take-back, recycling, or disposal options for the equipment at the end of its operational life.

- Packaging should minimize environmental impact, reduce waste, and improve transport efficiency while maintaining necessary protection for the equipment. Suppliers should provide documentation on the sustainability of packaging materials and transport processes.

Appendix 2 Supplier’s solution specification

If there is no space in the bottom cell of the individual table or if it is relevant to have an attachment, reference must be made to the attachment in the offer (not web pages) and where in the attachment the information can be found.

The supplier shall provide their responses/comments in the yellow field under each requirement.

Priority A = 	Absolute requirements that must be fulfilled. Tenders that do not fulfil all the absolute requirements will be rejected.

Priority B = 	Important requirements that should be fulfilled, but they are not an absolute requirement. Answers will be of great importance for the evaluation of the tender.

To be answered by the supplier

| 1 General Requirements |  |
| --- | --- |
| A | Warranty of at least 12 months is required. |
| B | If you offer more than 12 months of warranty, please detail the offer here. If there is additional price included in this, please specify in Appendix 3a Price Matrix. |
| B | The proposed product should preferably be a dedicated solution for art and cultural heritage imaging. |
| Confirm fulfillment and describe what is offered. Elaborate if applicable: |  |

| 2 Camera and Sensor Specifications |  |
| --- | --- |
| A | Must have Indium Gallium Arsenide sensor (InGaAs). |
| A | Must work into far IR (above 1.5 µm wavelength is required). |
| A | Must support high dynamic range (HDR) for enhanced differences in contrast (above 10 000: 1). |
| Confirm fulfillment and describe what is offered. Elaborate if applicable: |  |

| 3 Software and Image Processing |  |
| --- | --- |
| A | Shall have acquisition software compatible with MS Windows that allows for automatic stitching. |
| B | Can offer large area scanning, with resulting high-resolution image after acquisition and stitching. Please detail how the acquisition and stitching works. |
| Confirm fulfillment and describe what is offered. Elaborate if applicable: |  |

| 4 Accessories and Optics |  |
| --- | --- |
| B | Compatible macro lens(es) for detailed imaging. At least one, if more, please detail and specify. |
| B | Compatible filters for imaging specific IR wavelengths. Please offer your list of compatible filters. |
| B | Please include details on dedicated lighting solutions. |
| B | Include mounting (a tripod) and travel cases for the system. |
| B | If necessary for operating the camera and its associated software, please include an offer for an optional laptop. The price shall be listed in Appendix 3a – Price Matrix. |
| Confirm fulfillment and describe what is offered. Elaborate if applicable: |  |

| 5 Support, Service and Repairs |  |
| --- | --- |
| B | Suppliers shall describe what they offer in terms of support, service, repairs, and software updates. |
| Confirm fulfillment and describe what is offered. Elaborate if applicable: |  |

| 6 Sustainability |  |
| --- | --- |
| A | Expected lifespan of 7-10 years is required. Suppliers shall specify the expected operational lifespan of the offered instrument under normal maintenance conditions and provide documentation supporting this estimate, including information on service support and spare parts availability. |
| B | Suppliers shall describe the end-of-life arrangements for the instrument, including take-back, recycling, or disposal options for the equipment at the end of its operational life. |
| B | Packaging should minimize environmental impact, reduce waste, and improve transport efficiency while maintaining necessary protection for the equipment. Suppliers should provide documentation on the sustainability of packaging materials and transport processes. |
| Confirm fulfillment and describe what is offered. Elaborate if applicable: |  |

Appendix 3 Price and payment terms

# Price

All the Buyer's payment obligations appear from this appendix, as well as Appendix 3a Price matrix.

Table 1: Prices

[Unless there is a reference to a price matrix in the section above, please insert an agreed price list prior to contract formation]

## Invoice format

All UiO’s European suppliers must send invoices and credit notes in Peppol-format. UiO’s Peppol ID: 0192:971035854.

Foreign suppliers who are not able to send invoice in Peppol-format, may send PDF invoices to invoices@admin.uio.no

The Supplier must use UiO's chosen format and invoice processing method at all times. At least three (3) months' notice must be given before any change takes place. The Supplier carries their own costs associated with the delivery of electronic invoice as well as credit notes.

## Invoice quality

Invoices must have a reference to the correct purchase order number from the Buyer’s purchasing system. The Buyer reserves the right to return all invoices without correct reference.

The Buyer uses a system for analysis and statistics, which demands high quality of the inbound invoices. The invoices sent to UiO must therefore contain following information on each line row:

- Unique product number pr. product name

- Full product description

- Serial number/ unique item number (applies to purchase of equipment with serial number)

- Product price (excl. VAT)

- VAT rate

- Alternatively, any agreed fees (example: minimum order fee)

- VAT registration number and/or tax ID number placed at the invoice header

The product number and product name on the invoice must correspond to what is stated in the catalogue /punch out. To achieve auto match of invoices, it is necessary that the product number in both the catalogue /punch out are identical.

To ensure that invoices are processed correctly via UiO's systems, a test invoice must be conducted. Please contact the officer responsible for the contract and make further agreements.

Person sensitive information, such as health information, ethnicity or other sensitive personal data covered by GDPR Article 9, shall not appear in the invoice lines.

Please follow the link to guide on implementation of electronic invoice at UiO: https://www.uio.no/english/about/collaboration/invoice/index.html

Appendix 4 Administrative provisions

Project- and progress plan

[Fill out the relevant deliveries, partial deliveries and activities with the associated schedule, commencement date and delivery date for the deliverables]

Cooperation

[If the delivery requires meetings to be held: Specify that these are part of the offer and that there will be no additional costs for this.]

Key personnel

The following Supplier’s key personnel shall carry out the delivery of the service/goods:

| Name | Position | Area of expertise | E-mail | Telephone |
| --- | --- | --- | --- | --- |

Approved subcontractors

The Buyer approves the following subcontractors:

[To be completed at the contract formation]

Use of third parties

The Buyer will involve the following third parties:

[To be completed at the contract formation]

Delivery times

[To be completed at the contract formation]

Place(s) of delivery

Blindernveien 31, Kristine Bonnevies Hus, Konservering

0371 Oslo

Norway

The Supplier shall be responsible for the transportation, delivery, unloading, and placement of the equipment at the designated location within the delivery address. The Supplier shall bear all risk for the equipment until delivery and placement have been completed.

Routines for order acquisition, handling and follow-up

The Supplier must be able to receive orders from the Buyer’s purchasing system. Order confirmation must be sent to the stated order reference (on the order) and must contain the following information:

- Name of the recipient

- Order description (product and quantity)

- Order number (UiO's reference number starting with number 6)

- Delivery time

- Any partial deliveries

Delays or other deviations associated with the delivery must be reported immediately to the Buyer, once the deviation is discovered.

Information on contract, contractor and employees

It is the client’s duty to report the assignment when the value of the assignment exceeds NOK 20.000. The foreign contractor is obliged to report the employees that work on the assignment.

Further information on the reporting obligation can be found on the Norwegian Tax Agency's website:

https://www.skatteetaten.no/en/forms/rf-1199-information-about-contracts-contractors-and-employees/

VAT and Registration Requirements for Foreign Suppliers

The Supplier acknowledges that Norwegian VAT rules differ from general EU VAT rules. In Norway, foreign suppliers delivering services physically in Norway (such as installation, training, or on-site support) — either alone or in combination with goods — and where the total value of such deliveries to one or more Norwegian customers equals or exceeds NOK 50 000 within a 12-month period, are required to register in Norway. The reverse charge mechanism commonly used within the EU does not apply in these cases.

The Supplier shall, prior to issuing any invoice, be registered in Norway with a Norwegian organization number (NUF) and be registered in the Norwegian VAT Register. The Supplier shall ensure that all invoices issued under this Contract include and handle Norwegian VAT in accordance with applicable Norwegian tax regulations.

The Customer shall have the right to withhold payment of any invoice until the Supplier has fulfilled the above registration requirements. This applies regardless of whether the Contract is entered into with the foreign parent company or a Norwegian-registered branch.

For more information and guidance on what foreign suppliers must do, see:

- https://info.altinn.no/en/start-and-run-business/direct-and-indirect-taxes/indirect-taxes/vat-specific-information-for-foreign-companies/

- https://info.altinn.no/en/start-and-run-business/planning-starting/Choosing-Legal-Structure/norwegian-branch-of-a-foreign-company-nuf/

- https://www.brreg.no/en/other-types-of-organisation/norwegian-registered-foreign-business-nuf/duty-to-register-with-the-register-of-business-enterprises-for-nuf/?nocache=1738149867038

- https://www.skatteetaten.no/en/business-and-organisation/foreign/registration-obligation/registration-obligations-for-foreign-limited-businesses-in-norway/

Appendix 5 Options

[All options relevant under this agreement shall be described in this Appendix (does not apply to option for contract prolongation). Contractual clauses concerning changes/options needs to state clearly which changes the Buyer can make, to which extent and under which terms. Such clause does not allow the contract’s overall nature to change].

Appendix 6 Changes to the general contractual wording

| Clause | Original text | New text |
| --- | --- | --- |
| 4.4 | Unless otherwise agreed, the warranty period is two (2) years calculated from the time of delivery in accordance with clause 7.1. | 1 General requirements in Appendix 1 Buyer’s requirement specification. |

Appendix 7 Changes subsequent to the formation of the contract

The table below shall enlist all changes or additions to the contract subsequent to the contract formation.

The change table shall be kept by date on a continuous basis and state who initiated the change.

In the column “Date” the date of enforcement of the change shall be stated.

In the column “Initiated by” it shall be stated which of the parties requested the change.

In the column “Reference” the reference to the place in the contract where the change or amendment belongs shall be stated.

In the column “Change” the text changing the existing text shall be inserted.

In the column “Addition” the text that is added to the contract shall be stated.

In the column “Change” the following formatting shall be used:

Striked out text = deleted text

Bolded text = new text

| Date | Initiated by | Reference | Change | Addition |
| --- | --- | --- | --- | --- |