SSA-L Appendix 2026

Appendices to SSA-L

Appendices to the Agreement concerning Ongoing Purchases of Services via the Internet

The Norwegian Government's Standard Terms and Conditions for IT Procurement

Appendix to SSA-L

# Appendix to SSA-L – Agreement concerning Ongoing Purchases of Services via the Internet

# Contents

Contents	2

Appendix 1: Customer specification of requirements	7

Appendix 2: The Supplier’s description of the service	38

Appendix 3: Plan for the establishment phase	69

Appendix 4: Service level with standardised damages	81

Appendix 5: Administrative provisions	85

Appendix 6: Total price and pricing provisions	89

Appendix 7: Changes to the general contractual wording	92

Appendix 8: Changes to the service after the establishment of the Agreement	94

Appendix 9: Terms for the Customer’s access to and use of third-party deliverables	95

## Glossary of terms

Capabilities

Resources have capabilities, these are used as a determining factor when the WPS is selecting appropriate resources based on users’ specified requirements. Capabilities can for example apply to decoders, what transports they are able to decode, what their location is. Capabilities can also be applicable to physical spaces.

Capacities

Resources have capacities, these capacities can detail how many concurrent media functions a resource is capable of. This may additionally include the amount of CPU or GPU that a device has, which can be utilised to determine if the device has sufficient capacity for more media functions to be activated or not.

Clashes

Situations where one or more resources are allocated to multiple jobs, often also referred to as overprovisioning

Events

An Event is a collection of Jobs that are all part of the same activity, such as the FIFA World Cup. A match with several feeds would constitute an Event. A Job would be created for each incoming feed. The purpose of this is to put content at the centre.

Job

A job is a booking of a singular source. For example the booking of a LiveU backpack & decoder for a live insert to a news programme. The LiveU element of this is classed as a job. The job would also have to cater for any return feeds. Within a job it would be possible to specify multiple formats of the same source.

Monitoring Profile

Monitoring Profiles are pre-defined levels of monitoring that are to be applied to resources that are being used for a Job or Event. This would include the number of probe points that are added to any signal paths that are created.

On-Boarding

On-Boarding is the initial addition of new devices and services to the media and management networks, this also includes making new devices and services available within the relevant systems so that they are available for use.

Media Functions

Devices may have multiple templated configurations, known as personalities. What this means is that it is possible to deploy one or many personalities to a device to change its operational mode.

Media Operations Center (MOC)

Operators and Leads at the MOC, serve as the primary operational hub of NRKs productions. They are responsible for real-time resource allocation, signal routing, overarching administration, and ensuring the uninterrupted flow of media for both TV and radio broadcasts.

PMCS

A Platform Monitoring and Control System (PMCS) is the collector and aggregator of monitoring and alarm data from across NRKs production platform.

Production Manifest

A Production Manifest is a collection of runtime data and actions resulting from users’ production requirements. Manifests utilise Programme Templates as their base data, additional data such as staff and device configuration can be added.

Properties

Properties are detailed resource information that users may add to give context to resources.

Provisioning

Provisioning is the function by which the WPS selects required resources from pools. These resources may all have the same Capabilities or Capacities or they may be different, based upon the needs of the request, the correct resource is chosen from the corresponding Resource Pool. Allocating Resources to a Job is a part of the Provisioning.

Resource

Resources can be thought of as singular instances of “things” such as hardware, software, and physical spaces. In NRK’s IP broadcast environment, resources are considered to be collections of endpoints on devices.

Resource Pool

Resource Pools are collections of multiple different Resources. Pools are structured by capabilities, and can be navigated by users or automation based on user’s requirements and corresponding needs.

Templates

Templates are collections of predetermined rules for a programme. Templates can be applied to a programme, enabling required capabilities to be determined and met at the very beginning of when the programme is scheduled.

Workflow

Workflows are reusable actions applied to events and jobs to automate resource selection and reduce manual user effort. For instance, a workflow detailing device connections can automatically establish the connections, preventing users from having to do so manually.

Defined Operational Deliverables

These are the currently known deliverables that NRK has defined. These will be developed in collaboration with the Supplier, and will be subject to adjustment during the negotiation phase.

Defined Integrations

These are the currently known integrations that NRK has defined. These will be developed in collaboration with the Supplier, and will be subject to adjustment during the negotiation phase.

## Description of requirements in appendices

Must - These are requirements that must be fulfilled, and will be evaluated based on the submitted documentation/Supplier’s response.

Based on the bidder's answers to NRK’s questions, any over-fulfillment of the requirements that provides added value to NRK may qualify for a higher score in the evaluation.

- Response Instructions: The Supplier shall describe to what extent and in what manner the requirement is fulfilled.

- Consequence: Non-compliance may lead to the rejection of the proposal. Upon compliance, the description will be assessed and scored in accordance with the award criteria.

Should - These are requirements that should be fulfilled, but are not mandatory for participation.

Based on the bidder's answers to NRK’s questions, any over-fulfillment of the requirements that provides added value to NRK may qualify for a higher score in the evaluation.

- Response Instructions: The Supplier shall describe to what extent and in what manner the requirement is met.

- Consequence: Non-compliance will not lead to rejection of the proposal, but will result in a deduction of points or a score of zero in the evaluation.

Explanation for tables to be filled out by Supplier:

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| Requirement number for references. <YYY> | Requirement type as explained above <Must/Should> | Description of requirement | Supplier to answer if solutions fulfil requirements. <Y: Yes P: Partially (must be explained why partially) N: No> |
| Supplier's Response |  | <Placeholder for response. If longer responses are required use space below with reference to requirement number and title.> |  |

The Supplier shall specify whether the requirements are currently supported by standard functionality or if they will be developed during the implementation phase. If a requirement is not currently met but is scheduled for development, the Supplier must explicitly state this and provide a timeline for delivery.

# Appendix 1: Customer specification of requirements

## Background and purpose of the Agreement

The Workflow Provisioning System (WPS) is designed to be the central, intelligent resource orchestration layer crucial to NRK's Modernised Production Platform (MPP) programme. Its primary objective is to meet NRK’s long-term strategic goals by facilitating a transformation to native AV over IP (AVoIP) services, centralising production processing resources in external data centres, and preparing for the integration of public cloud services. This supports NRK's environmental and financial strategy by dynamically provisioning resources from centralised pools and automating de-provisioning of workflows to spin down or suspend idle resources, and providing the organisation relevant information on resource usage.

The WPS is being implemented to automate and enhance production processes across the entire organisation, fulfilling these key functions:

- Resource Allocation and Scheduling: The WPS acts as the central engine for Provisioning by selecting required Resources (hardware or software endpoints) from defined Resource Pools based on the needs of a request. This includes providing an intelligent schedule capable of determining and flagging Clashes of resources, such as devices or rooms.

- End-to-End Production Planning: A core function is to take data from multiple existing systems—such as channel schedules from What's On and location details from production planning—and use pre-determined rules to automatically allocate and provision resources.

- Operational Orchestration: The system uses Workflows to apply reusable, automated actions to Events (collections of jobs) and Jobs (singular bookings), reducing manual user effort for tasks like establishing device connections. Initial operational deliverables focus on Ingest Management (providing a user interface to select sources for recording) and phased Device Provisioning.

- Contextual Monitoring: The WPS applies context to alarms by correlating resource data from provisioned events and devices with alarm data from the PMCS stack, providing clear, contextual alarming to MOC operators.

The WPS system is intended to work across NRK’s whole platform, and is expected to be delivered as a resilient and scalable system, that supports NRK’s dual data centre model, and make sure that NRK has a working platform. It is expected that the WPS system has an active-active concept or similar, so that NRK have the WPS capability also in the event of larger errors. Key components and concepts include:

- Data Foundation: Core objects like Resource Pools, Blueprints (defining functionality in a space), Capabilities (used for resource selection), Jobs, and Events.

- Functional Requirements: Modules for Resource Management, Scheduling, Clash Management, Workflows, and a comprehensive API (HTTPS REST or GraphQL) for all functionality.

- Key Integrations: The system requires bi-directional communication with external systems, e.g. the Broadcast Control System (BCS) and integration with systems including Vimbiz (for resource/people booking), the PMCS Stack (for contextual alarming), ENPS (for news planning correlation), What's On (for schedules), and the Configuration Manager (for deploying configuration data to devices).

Purpose

NRK is currently in the process of performing a business transformation from SDI and AES3 broadcast technologies to native AV over IP services (AVoIP). The AVoIP model will form the standard model for real-time broadcast activities at all sites.

In addition to the transformation to AVoIP, the MPP programme aims to centralise production processing resources in external datacentres, and to lay the groundwork for integrating public cloud services into broadcast workflows.

The transformation was initiated by the decision to establish new headquarters in Oslo at Ensjø, due to outdated facilities at the current Oslo location at Marienlyst. In addition, several large projects have been added to the programme, including establishing a new WAN, and moving NRK’s second largest facility in Trondheim to new premises, making this the largest transformation in NRK’s history.

## 1. Scope of the Agreement & the Service (Clause 1.1 & 2.1)

The objective of this RFP and the subsequent dialogue is to provide NRK with a WPS capable of meeting NRK’s long-term strategic goals.

### 1.1 Value for NRK

The Workflow Provisioning System (WPS) will act as the operational backbone of NRK’s modernized production platform. Its primary value lies in bridging the gap between editorial intent and technical execution, making the transition to dynamic, centralized resource sharing seamless for the users.

By unifying long-term planning, intuitive self-service booking, and automated machine-to-machine provisioning into a single ecosystem, the WPS will eliminate manual bottlenecks. This empowers NRK to maximize the utilization of shared assets, drastically reduce configuration times, and enable our staff to produce more content with greater confidence and a significantly reduced environmental footprint.

Environmental savings is important for NRK, and WPS will be a system that supports and enables a reduction in our footprint. As stated in Chapter 1.6 Environment and Sustainability Requirements:

“Legacy broadcast architectures typically rely on localized infrastructure where backend resources are statically tied 1:1 to specific studio/gallery facilities. With a modernized, software-defined production platform (MPP), these technical resources will be centralized in data centres and dynamically shared across various editorial departments and productions.

The WPS is deemed mission-critical for NRK’s ability to efficiently request, allocate, and release these pooled resources. To achieve this, NRK expects the system to utilize scheduling templates, automated orchestration logic, and predictive resource allocation based on historical utilization data.”

#### 1.1.1 Requirements - Value for NRK

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.1.1 | Must | Dynamic Resource Pooling & Sharing The system must provide real-time visibility into resource availability across all of NRK. It should automatically find and allocate the best-suited shared resources (whether physical studios, hardware, or software compute) for a given production, eliminating the need for dedicated, single-use setups. Minimum fulfilment of requirement is to be able to connect shared data centre resources to a production facility. Describe how your solution is able to make visibility of resources and how it operates and how it facilitates resource sharing. Describe how the solution will behave if there is a lack of available resources in the pool. |
| 1.1.2 | Should | Editorial Empowerment & Automation Non-technical staff must be able to plan and book standard setups independently through intuitive self-service solutions. By automating routine provisioning and routing tasks, the system will reduce human errors, accelerate time-to-air, and free MOC/technical operators from doing equipment allocation to focus on complex productions rather than manual routing. Describe how your solution supports Non-technical staff on planning and booking. Describe how the solution will behave if there is a lack of available resources in the pool. |
| 1.1.3 | Must | Support Sustainability The platform must make it easier to make sustainable choices. By making possible goals for "work from anywhere", remote productions and maximizing the reuse of centralized resources. As a minimum the system must directly contribute to reducing NRK’s carbon footprint by making centralised equipment available to productions or sites. Hence reducing the need for travel to or transport of media production equipment. Describe how your solution can support NRK in reducing carbon footprint for NRKs future production platform. If relevant use cases, please share one-pager about the environmental benefits in the different use cases. One-pager can be added as a combined pdf-file gathering all use cases into one document. |
| 1.1.4 | Should | Produce More with Confidence & Resilience The system must provide the structural reliability needed to safely schedule more concurrent productions on the same technical footprint. This includes robust exception handling and "Strategic Preemption"—the ability to automatically tear down or re-route lower-priority workflows to free up capacity for Priority 1 events like Breaking News. Describe how your solution supports scheduling productions and exception handling. Describe if the solution supports some kind of “demand for elevated rights” to cancel ongoing bookings, to free up resources. |

### 1.2. Users and respective value from WPS

Users of the WPS can be roughly divided into two groups:

- Users who interact with the WPS directly through a dedicated user interface (UI). These are referred to as direct users.

- Users who interact with functions or data provided by the WPS, but utilise UIs in other systems. We define these users as indirect users. It should be noted that a current indirect user could become a direct user of the WPS if it is decided that there should be developed a dedicated UI for them at a later stage.

We have preliminarily identified a list of user groups, both direct and indirect, and their respective core needs. These users, and our logic for distinguishing between direct and indirect, are shown below. This is not a complete list of possible users of the WPS but rather who we view as most relevant for the initial stage of the WPS, as their needs are prioritised for the concepts to be developed first in the WPS.

Figure 1. There is a high-resolution PDF attached to the RFP.

Based on these groups we have defined the timeline of deliverables that we feel best suits the needs of NRK, see Appendix 3 “Plan for the establishment phase”. As the timeline progresses, certain users are likely to move across the two groups, based on both the WPS deliveries and the implementation, use and decommissioning of other NRK systems.

Requirements below will be relevant for multiple user groups and to be viewed as system capabilities to be supported.

#### 1.2.1 Direct Users

Direct users are those that NRK believes initially will interact directly with the WPS through its own user interfaces.

##### Operators at the Media Operations Center (MOC)

Operators and Leads at the MOC, serve as the primary operational hub within the system architecture. They are responsible for real-time resource allocation, signal routing, overarching administration, and ensuring the uninterrupted flow of media for both TV and radio broadcasts.

MOC’s Needs

Operating in a highly dynamic, time-sensitive environment, MOC staff require the ability to rapidly identify current routing states, manage complex exceptions, and resolve booking conflicts without relying on manual memory or fragmented communication. Their overarching need is cognitive relief through a single unified operational truth showing capacity and proactive conflict management.

##### Requirements - MOC

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.2.1.1 | Should | Unified operational truth The system should act as the definitive repository for all routing states and provisioning data, providing a single, accurate overview of connections to replace manual logging. Describe how this will be managed by the solution, and in particular how the solution will make sure to get updated state from other systems, e.g. the Broadcast control system. |
| 1.2.1.2 | Should | Proactive Conflict Management The system should proactively detect scheduling Clashes and generate intelligent warnings to prevent the accidental severance of active, on-air broadcasts. Describe how the solution will distinguish between immediate resource conflicts that must be handled now, and future resource challenges that can be handled later. NRK should be able to adjust threshold values ​​for when and where "close" events that require immediate handling. Please describe how the system allows that. |
| 1.2.1.3 | Should | Automated Alternative Routing Upon detecting a resource conflict, the system should automatically calculate, validate, and suggest viable alternative routing paths to the operator |

##### Ingest Operations

Ingest operations consist of technical staff tasked with overseeing the secure intake of recorded media and live streams into the organization's central storage architecture for editing and archiving.

Ingest’s Needs

Ingest teams require high degrees of automated orchestration to handle their dynamic workloads safely. They need to allocate recording ports efficiently and continuously monitor the real-time status of active streams to prevent data loss, while reducing their dependency on manual configuration steps.

##### Requirements - Ingest

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.2.2.1 | Should | Automated Port Allocation The system should facilitate the automated allocation and management of recording ports based on scheduled broadcast events. |
| 1.2.2.2 | Should | Real-Time Stream Monitoring The system should provide live, real-time visibility into the status of active ingest streams to quickly identify and mitigate potential data loss. |
| 1.2.2.3 | Should | Autonomous Configuration Capabilities The system should provide standardized templates allowing staff to independently configure complex ingest requirements without dependency on technical staff. |
| 1.2.2.4 | Should | Ingest time-slot expansion Due to the nature of ingest, there will be situations where a pre-ordered ingest-time, and the following resource allocation will need to be extended. Please describe how the solution will support this. |

##### Post-production

Personnel working within editing and post-production suites. These users require fast provisioning of resources, such as shared processing compute from data centres or specific editing configurations.

Post-Production’s Needs

Post-production users need friction-free access to required technical resources without requiring deep engineering knowledge. Their overarching need is to utilize simplified, template-driven functions to instantly attach their workstation to the necessary backend capabilities.

##### Requirements - Post-production

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.2.3.1 | Should | Template-Based Provisioning The system should allow users to trigger standard, pre-defined workflow templates to quickly configure and access post-production resources. |
| 1.2.3.2 | Should | Dynamic Resource Pooling The system should seamlessly allocate shared data centre resources (such as compute/GPU power) based on real-time availability and task requirements. |
| 1.2.3.3 | Should | Affinity-based provisioning The system should be capable to allocate resources based on data centre affinity for the users profile, eg. If a user in the northern most office of NRK requests a post-production compute capacity, the system must book and allocate a resource from the northern most DC if there is capacity. If there is not available resources, the system should be capable of suggesting utilizing potential available resources from other DC-location. |

#### 1.2.2 Indirect Users

Indirect users will continue to utilise existing systems within NRK, however these systems and interfaces are subject to change.

##### Technical Operations Managers (TOMs)

Technical Operations Managers are technical leads, physically located within studio control galleries. They act as the bridge between the digital provisioning system and the physical hardware residing in the production room.

TOMs’ Needs

The primary focus of a TOM is rapid, reliable resource activation (both physical and digital resources). They need a standardized mechanism to initialize and assume control over pre-provisioned backend resources assigned to their specific physical room immediately prior to a live show.

##### Requirements - TOMs

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.2.4.1 | Should | The system should allow for operational users to bulk activate connections between backend and front end devices and configurations on devices. This should be achievable by selecting a job or event for use. |

##### Editorial Offices & Production Planners

Includes editorial project managers, production planners, radio show hosts, and district TV producers. They coordinate logistics over longer term production planning, manage ad-hoc news events, and are responsible for booking rooms, equipment, and transmission lines across all locations.

Editorial Offices & Production Planners’ Needs

Editorial teams require a high degree of autonomy to ensure their technical setups are functional prior to a broadcast. They need intuitive, self-service capabilities to reserve complex bundles of equipment and personnel far in advance, abstracting the underlying technical complexity away from the user.

##### Requirements - Editorial Offices & Production Planners

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.2.5.1 | Should | Self-Service Provisioning The system should provide a dedicated, non-technical interface allowing editorial staff to independently plan and book standard setups. |
| 1.2.5.2 | Should | Advance Capacity Reservation The system should allow users to reliably reserve specific contribution equipment, physical rooms, and transmission lines well in advance of the actual production date. |
| 1.2.5.3 | Should | Schedule Integration The system should offer an integrated overview (e.g., calendar integration) that aligns technical resource requests directly with the editorial broadcast schedule. |
| 1.2.5.4 | Should | Allow Pre-order of jobs and events In a situation where users are planning a future event, the system should allow jobs/events to be created, if the status of the job/event is pre-order then no resources are allocated, however the system validates availability and highlights to the operator if the availability has a status of good or bad. |
| 1.2.5.5 | Should | Highlight future resource-limitations The system should be able to highlight and show future resources capacity issues. This so that the organization can prepare for any capacity expansion or other mitigating steps. |

##### Management & Strategy

This group consists of executive leadership, production strategy, and business insight analysts operating at a macro level. They consume aggregated system data to perform strategic planning, financial analysis, and resource allocation across the enterprise.

Management & Strategy’s Needs

Strategic stakeholders rely on accurate analytics and utilization planning. They need the ability to visualize historical trends and real-time consumption data to prevent double bookings, optimize capital investments, and accurately forecast the organization's maximum production capacity.

##### Requirements - Management & Strategy

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.2.6.1 | Should | Analytics and Utilization Reporting The system should provide macro-level dashboards and extractable data demonstrating how frequently specific physical and digital resources are utilized. |
| 1.2.6.2 | Should | Output Forecasting The system should supply predictive capacity metrics to help identify future resource deficits or surpluses well in advance of large events. |
| 1.2.6.3 | Should | Financial Optimization Insights The system should allow analysts to evaluate resource availability against historical booking trends to identify inefficiencies and empirically justify budget decisions. |

### 1.3. NRK Concepts

Detailed below are a number of concepts that are under development at NRK. These concepts highlight NRKs desire to increase efficiency in planning, execution and monitoring of jobs and events. Is it believed that the WPS is best suited to enable these desired outcomes.

#### 1.3.1 End to End Production Planning

A core function of the WPS is for it to take data from multiple systems and use pre-determined rules, to allocate and provision resources. The aim is to enable end-users to utilise their preferred UI, and for the WPS to push the provided data to relevant systems, arbitrating the outcome. An example of this is the following:

- Channel schedules are created and published in Mediagenix What's On - WPS now has the schedule of every show on every channel for the upcoming week.

- Production staff/planners make the template for the production, regarding what capacities it needs.

- Production staff/planners allocate a studio and PCR (Production Control Room) that will be used for one of the shows in the TV schedule - WPS now has location details of where the Job will take place.

- Production staff/planners allocate resources to the Job, based on the template for the production.

Coupling these pieces of data with preset rules such as a ‘high watermark’ for Datacentre usage. The WPS is able to determine if the Studio and Gallery can use their preferred Datacentre or not. This decision then means that any further resources that are required for the show are provisioned from the same datacentre, thus reducing the size of the pool that is being picked from.

Figure 2. There is a high-resolution PDF attached to the RFP.

#### 1.3.2 Contextualised Monitoring & Alarming

Jobs, Events and Programme Templates may all contain details of SLA level. The SLA level determines the levels of resilience that is required and also what level of monitoring should be applied to signal chains and services that are being used.

The objective of the WPS is to utilise this SLA information and the data it has around resources that are in use and activate the relevant level of probing on signals, also instructing those probes what alarming threshold templates to apply.

Further to this, the WPS may be able to pass the contextualized data of the Job/Event name to the PMCS stack where a dashboard may be created containing information about the resources that have been assigned in the WPS.

These dashboards are then job/event specific and provide greater focus to MOC operators to monitor the in use services.

#### 1.3.3 Programme Templates

To aid the desired automation, NRK wishes to be able to create templates for programmes, where NRK are able to define specific elements that aid the rules and automated actions within the WPS.

Examples of this are:

- SLA Level - The SLA level drives what level of resilience is required for the programme in terms of service levels, this also informs the WPS of the monitoring profile that should be applied to the programme

- Studio Size (Minimum Req)

- Gallery Size (Minimum Req)

- Services Capacity - This details the number of each service that are required; for example, graphics, playback and slow mo engines.

- Format - Details what the format is that is required. NRK are standardising on 1080p50 SDR and HDR, however there may be a need for UHD. This is detailed to be applied to any sources that may need format conversion.

Program Templates will be the fundament of a Production Manifest which is unique to that specific show.

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.3.1 | Should | End to End Production Planning Please reflect about how proposed solution could support this |
| 1.3.2 | Should | Contextualised Monitoring & Alarming Please reflect about how proposed solution could support this |
| 1.3.3 | Should | Programme Templates Please reflect about how proposed solution could support this |

### 1.4 . Functional and technical requirements

To support the value desired from a WPS system, deliverables and user requirements described in chapters above, core functionality must be supported to ensure a valuable system for NRK. These requirements are described below and must be answered if it is fulfilled or will be fulfilled during the implementation phase.

#### 1.4.1. Resource Management

Resources are a core foundation utilised by the WPS. Resources may be hardware devices or software services. However, being aware of this is no longer enough. Where software services are capable of co-existing on the same server hardware, it is necessary for the WPS to be aware of the ‘compute’ a software service requires to function, this includes but is not limited to CPU and GPU.

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.4.1.1 | Should | The WPS should have a method for creating resource pools. |
| 1.4.1.2 | Should | The WPS should have a method for adding capabilities to resources. |
| 1.4.1.3 | Should | The WPS should have a method for adding ‘minimum requirements’ to resources in the case of software services for example. |
| 1.4.1.4 | Should | The WPS should have a method for enabling users to create ‘composite resources’ for example video and audio ST2110 essence can be combined to create a single resource. |
| 1.4.1.5 | Should | The WPS should provide a method allowing users to ‘breakaway’ essence within a composite resource. |
| 1.4.1.6 | Should | The WPS should allow for metadata to be added to resources such as location. |
| 1.4.1.7 | Should | The WPS should allow for the linking of resources, for example where certain resources are always used with another resource, having a link to provision both at the same time is desirable. |
| 1.4.1.8 | Should | The WPS should have a method for capturing system virtual resources, for example intercom IFBs & Conferences. |

#### 1.4.2 Scheduling & Clash Management

A core function of the WPS is to provide a schedule with automation and intelligence.

This means that the WPS scheduling is capable of detecting potential clashes of resources such as devices or rooms,and proactively notify this to the appropriate users.

NRK believes that for any scheduling system to perform correctly and accurately, it must be able to handle clash management. Being able to detect and predict where the usage of devices and services result in conflict, or there will be a lack of resources, is crucial for the system to perform efficiently and provide value to NRK.

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.4.2.1 | Should | The WPS should have a scheduling function allowing for planning of events up to 24 months in the future, including resource allocation and warnings regarding conflicts. |
| 1.4.2.2 | Should | The WPS should have a ‘repeat event’ function to allow for replication of events that occur on a daily, weekly, monthly, yearly basis. |
| 1.4.2.3 | Should | The WPS should allow for repeat events to be modified in isolation without affecting others within the series. |
| 1.4.2.4 | Should | The WPS should allow the user to choose if changes to a repeat event should be replicated to all future events or only to the selected event. |
| 1.4.2.5 | Should | The WPS should have a method for detecting where resources are not available and highlighting to the operator. This should be a continuous procedure as opposed to waiting until all data is populated. |
| 1.4.2.6 | Should | The WPS should be able to display connections between resources that are made using a third party system such as the BCS. |
| 1.4.2.7 | Should | The WPS should not break any connections made by third party systems automatically. Describe how conflicts will be highlighted to the user, and could be handled. |
| 1.4.2.8 | Should | Should a third party system connection utilise resources that are planned for an upcoming event, the WPS highlight this to the relevant operational users and suggest alternative resources if available |
| 1.4.2.9 | Should | The WPS should allow for multiple views of schedule. This includes a job/event view and Resource views. |
| 1.4.2.10 | Should | All views should also show third party connections |

#### 1.4.3 Workflows

Workflows are a tool that can be used to provide a layer of automation to any task within NRKs daily operations. This may range from workflow designed to take incoming signals and automatically apply processing to the video and audio, to one that helps users to find available edit clients and suitable rooms within a given location.

Figure 3. There is a high-resolution PDF attached to the RFP.

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.4.3.1 | Should | The WPS should support workflows, where resource pools are added and connected together. |
| 1.4.3.2 | Should | It should be possible within a workflow to set rules for individual resource pools. |
| 1.4.3.3 | Should | Workflows may be applied to multiple jobs concurrently. |
| 1.4.3.4 | Should | Workflows may be edited on a ‘per job’ basis, without change affecting existing jobs utilising the same workflow nor the originating workflow. |
| 1.4.3.5 | Should | It should be possible to create workflows via API |
| 1.4.3.6 | Should | It should be possible to modify workflows via API |

#### 1.4.4 API

That the WPS system has an API (North and South bound) is crucial for NRK to achieve not only the integrations that are required, but also for the automation of data capture between systems.

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.4.4.1 | Must | The system must have an API. The API must be documented. |
| 1.4.4.2 | Should | The API documentation should be complete, describing all endpoints. Documentation of this requirement is solved by giving NRK access to either OpenAPI or a PDF-file containing the complete API documentation. |
| 1.4.4.3 | Should | The WPS should have a HTTPS REST or GraphQL API. |
| 1.4.4.4 | Should | The WPS should support north and south bound APIs. To connect to WPS from other systems and to push actions in other systems. |
| 1.4.4.5 | Should | The API should cover all functionality in the Product. Describe what is not possible via API, if any. |
| 1.4.4.6 | Should | The API should be documented using OpenAPI. |
| 1.4.4.7 | Should | The API should have an interactive Swagger interface. |
| 1.4.4.8 | Should | The WPS should have support for configuration using Ansible or Terraform with an Ansible module or Terraform provider supported by the Supplier. |
| 1.4.4.9 | Should | The WPS should support administrator-configurable webhooks to interact with other generic APIs (e.g., on status updates). |
| 1.4.4.10 | Should | The WPS should support sending and receiving events on a service bus using AMQP. |

#### 1.4.5 User Interface & User Experience

NRK has a strong focus on ensuring inclusive and accessible workflows for all employees. The user interface is expected to comply with accessibility standards. Semantic markup is fundamental for ensuring robust keyboard navigation, and it is absolutely essential for assistive technologies.

The user interface must be responsive and scale dynamically to fit seamlessly across various screen sizes and proportions (viewports). Our users will operate the system from a wide range of devices and screen types.

Self Developed User Interface

NRK have a history of developing systems themselves and the WPS might be connected to other user interfaces. While it is also expected that the WPS will have its own proprietary user interface that can be customised. NRK should have the ability to develop user interfaces that would utilise any API made available from the Supplier for communication.

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.4.5.1 | Should | The WPS should have a standardised set of ‘out of the box’ (OOTB) user interfaces. |
| 1.4.5.2 | Should | It should be possible to customise these ‘OOTB’ interfaces. |
| 1.4.5.3 | Must | The WPS user interface must be accessible as a responsive web interface. |
| 1.4.5.4 | Should | The WPS should meet the latest Web Content Accessibility Guidelines (WCAG) at levels A and AA. |
| 1.4.5.5 | Should | The WPS web interface should support the common browser versions from Edge, Chrome, Firefox, and Safari. |
| 1.4.5.6 | Should | The user interface should be adaptable to NRK's own design guidelines and visual identity. The solution must be flexible enough to accommodate specific user needs when configuring tools for different workflows. This includes the ability to override the system's default styling, as well as supporting the creation and use of custom views, templates, or presets for specialized use cases. |
| 1.4.5.7 | Should | The WPS should support usage of standard design tools, such as Figma, for the creation and development of user interfaces. |
| 1.4.5.8 | Should | It should be possible to develop a standalone user interface in any language NRK chooses and connect it to the WPS with full capabilities available. |
| 1.4.5.9 | Should | It should be possible to reuse components across multiple user interfaces as opposed to creating bespoke components for each interface. |

#### 1.4.6 Monitoring & Alarming

The WPS is not expected to be the aggregator for all monitoring and alarms across the NRK estate, however it is highly likely that it will possess the most data from multiple systems to apply context to these alarms.

Contextual alarming is critical to give operators the clearest picture possible of alarms and faults. The WPS will hold data of events and the devices that are in use for an event. It is expected that the WPS will be capable of correlating this data with alarm data from the PMCS stack to provide the contextual alarming to users.

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.4.6.1 | Should | The WPS should be capable of connecting to external data aggregators and visualizing in monitoring overview. |
| 1.4.6.2 | Should | The WPS should be capable of taking external data and contextualising it with data already stored to provide a more useful overview to operators. |
| 1.4.6.3 | Should | The WPS should have an integration to Grafana, Loki, Mimir. If not in place at the current point of time, please describe how this will be performed and the estimated time of delivery. |

### 1.5 Future development of WPS and roadmap

NRK views the WPS as a dynamic ecosystem that will require continuous development over a long lifespan, not a static, one-off IT delivery. As our organization evolves, the platform must be capable of scaling iteratively to onboard new user groups. This requires an architecture built for constant change, where custom integrations with both legacy systems and future third-party tools can be developed and deployed.

Furthermore, the platform must remain agile enough to adapt to emerging technologies and industry paradigms, such as Dynamic Media Facilities (DMF) and AI-driven orchestration, as these concepts mature over the coming years.

As we cannot fully predict how our operational needs will shift, the contractor's roadmap and approach to continuous development are important. This continuous development, along with major system upgrades, must be handled with strict backward compatibility, to ensure that we can continuously innovate and expand the system without disrupting daily broadcast operations or accruing unsustainable technical debt.

| # | Type of req | Requirement |  |
| --- | --- | --- | --- |
| 1.5.1 | Must | 3-year roadmap The Supplier must share their roadmap for the next three years. |  |
| 1.5.2 | Should | Roadmap alignment to NRK needs The Supplier should indicate where the roadmap items map to NRK requirements and state where the Supplier is willing to prioritise items to increase alignment with the NRKs timeline. |  |
| 1.5.3 | Should | Design & architecture alignment The proposed solution in combination with the roadmap fully aligns with NRKs design principles, system architecture and technical requirements within the given timelines. |  |

### 1.6 Environment And Sustainability Requirements

As stated in NRK’s 2025 Sustainability Report, our strategic objective is to "prioritize initiatives that yield measurable emission reductions from our own operations". This commitment has successfully driven a 34% reduction in direct emissions compared to 2019 levels, targeting a 50% reduction by 2030. A demonstration of this commitment is NRK signing a letter of intent to develop our upcoming headquarters at Normansløkka as a FutureBuilt flagship project, enforcing sustainability and environmental criteria significantly exceeding industry standards.

Approximately three-quarters of NRK's total carbon footprint originates from indirect Scope 3 emissions (the procurement of goods and services), representing the vast majority of the organization's environmental impact. The WPS falls directly into this category, as the platform will be procured and operated by an external supplier.

NRK anticipates minimal emissions from the WPS itself, as this tender does not include the purchase of physical hardware or other highly emission-intensive components. However, the system will generate a lifecycle environmental impact driven by data centre energy consumption and the electronic waste (E-waste) management of host servers at End-of-Life (EOL). Specific compliance requirements addressing these aspects are detailed below.

Conversely, the WPS is expected to deliver a substantially more positive environmental impact through its enabling effect, by facilitating operational flexibility and maximizing the utilization of technical devices and physical facilities. NRK expects that the system's core orchestration capabilities will optimize asset lifecycles, thereby reducing the required volume of future capital expenditure and their related emissions.

Optimize utilization of devices

NRK’s ongoing technological transformation centers on the decentralization and virtualization of production infrastructure. Legacy broadcast architectures typically rely on localized infrastructure where backend resources are statically tied 1:1 to specific studio/gallery facilities. With a modernized, software-defined production platform (MPP), these technical resources will be centralized in data centres and dynamically shared across various editorial departments and productions.

The WPS is deemed mission-critical for NRK’s ability to efficiently request, allocate, and release these pooled resources. To achieve this, NRK expects the system to utilize scheduling templates, automated orchestration logic, and predictive resource allocation based on historical utilization data.

Optimize utilization of facilities

NRK’s relocation to the new headquarters at Normansløkka involves a significant reduction in total square meters compared to our current facilities at Marienlyst. NRK is confident that the new facility will enhance operational workflows due to its optimized, efficient architectural design and NRK being able to efficiently utilize facilities.

Similarly to NRK’s intended utilization of devices, physical facilities such as studios, production rooms, and control rooms, will be pooled as common resources and shared across the organization. With the exception of the News division who will have some dedicated resources, productions must utilize a centralized, shared pool of facilities.

Furthermore, production workflows will be structurally decoupled. Control rooms will no longer maintain a static 1:1 relationship with specific TV studios but rather be able to flexibly operate multiple studios across NRK. The WPS is the core enabler of this, where failure to achieve high facility utilization would compromise the principles of the Normannsløkka building, potentially forcing NRK to lease supplemental facilities or construct additional space, severely increasing NRK’s emissions.

Because production spaces will operate at high utilization rates, ensuring both rapid turnaround times between successive productions and having the ability to control multiple studios from a single gallery, are absolute necessities. The WPS is critical in realizing both of these objectives.

Adherence to the requirements is covered in must-requirement 1.1.1 and 1.1.3. However NRK also wants to focus on sustainability in the cloud deliveries with the following must-requirements.

Minimum Environment and Sustainability Requirements

The offered solution must support NRK's environmental sustainability objectives by enabling granular power and resource management. The Supplier must verify compliance with the following requirements:

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.6.1 | Must | The primary data centre(s) hosting the solution must have a documented Power Usage Effectiveness (PUE) of no higher than 1.6 at the time of contract signing. Documentation can be added as a combined PDF-file. The Supplier must, on request, provide NRK with a status report documenting the current PUE and detailing specific technical or operational measures implemented to improve energy efficiency. |
| 1.6.2 | Must | The Supplier and/or the subcontractor hosting the cloud service must have documented routines for responsibly recycling and reusing physical IT hardware (E-waste/WEEE). Documentation: Submit one of the following to prove compliance: a relevant environmental certification (e.g., ISO 14001, R2 or equivalent) including the specific E-waste routine, proof of a national WEEE registration (e.g., ERP, RENAS or equivalent) alongside an internal policy, an ITAD policy involving a certified recycling partner, or other equivalent documentation showing responsible E-waste management. Documentation can be added as a combined PDF-file. Maintenance: Compliance must be maintained throughout the contract. NRK must be notified and receive updated documentation if the chosen method, cloud provider, or recycling partner changes. |

### 1.7. Infrastructure requirements

Datacentre

The WPS is to be delivered as a SaaS solution, and support a distributed architecture allowing for operation across NRK. The solution must be connected to multiple of NRKs data centres.

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.7.1 | Must | The WPS system must be provided via a proven redundant model, for high availability and to support disaster recovery. Describe your redundancy model (active-passive / active-active), and the geographical split on data centres (zone split in one data centre or multi region/country. |
| 1.7.2 | Must | The WPS system must support NRK’s dual datacentre architecture as described previously in appendix 1. |
| 1.7.3 | Should | The WPS system is transparent in terms of sub-suppliers and hosting providers. NRK is informed on any changes in hosting providers. Inform about which hosting partners you use, as well as the location for data storage. Inform about how any changes in data storage will occur (will you inform/consult the customer, inform about choices, the customer's ability to reject the transfer of data storage (notice period)). Inform about the supply chain, and how you inform the customer about any changes in the supply chain. |
| 1.7.4 | Should | The WPS system has a database, with redundancy and resilience capabilities. Describe your model for database resilience, e.g. synchron replication and if there is automatic failover. |
| 1.7.5 | Should | In case of a system failure, recovery functionalities that minimize recovery time are in place. Describe your recovery model, and estimated time for recovery. |
| 1.7.6 | Should | Hosting and providing a service like WPS, which are crucial for operations, should have a systematic approach to observability/ monitoring and scaling around performance in the system. <Describe your model for system performance, ICT performance and scaling.> |

### 1.8 Documentation and training (Clause 3.4)

It is expected that the WPS will be a complex system and potentially be made up of many elements. To aid NRK in taking ownership of the WPS, clear training and documentation is required that is regularly reviewed and updated by the Supplier. Training materials can be in English or Norwegian language.

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.8.1 | Must | The Supplier must provide sufficient documentation for the system. Please state how documentation is provided, and the form of it. Specify if there are training videos available etc. Specify how customers get access to documentation (requires log in etc.) |
| 1.8.2 | Must | The Supplier must provide suitable training, that can be tiered to users of varying levels, from operators through to developers and administrators of the system. Describe your recommended training, and estimate the amount of training hours pr. Group of users: - Planners (production)/ Technical operating managers - MOC operators - System administrators |
| 1.8.3 | Should | Specify how possible tailor made solutions for NRK are documented, and how NRK-users will be guided to that documentation. |
| 1.8.4 | Should | Training to be offered for Planners (production)/ Technical operating managers: Please explain what content that is proposed included in a training session like this. Please add an indicative agenda that shows topics and timing for a typical session like this. |
| 1.8.5 | Should | Training to be offered for MOC operators: Please explain what content that is proposed included in a training session like this. Please add an indicative agenda that shows topics and timing for a typical session like this. |
| 1.8.6 | Should | Training to be offered for System administrators: Please explain what content that is proposed included in a training session like this. Please add an indicative agenda that shows topics and timing for a typical session like this. |
| 1.8.7 | Should | Training for specific user groups should be possible to provide on site at customers premises. Training sessions will be ordered by NRK. Please state how this is handled regards to: - Time from order to completion - Cost pr. User group session, including travel (Costs to be specified in 2.2 Appendix 6a - Price scheme - RFP_WPS, table 8). |
| 1.8.8 | Should | Online training should be offered that allows WPS users, developers and administrators to gain further knowledge and experience at their own speed. |
| 1.8.9 | Should | It is desirable that there is a forum/community available to NRK to connect to existing users, please elaborate if this is available for the solution. |
| 1.8.10 | Should | The Supplier should provide a forum/community/way of NRKs system experts/administrators connecting directly to Supplier SMEs for help and support. |

### 1.9 Personal data (Clause 6.2)

Document 2.4 Attachment 4 - Data Processing Agreement with subdocuments. regulates the use of personal data. In addition the following requirements will apply.

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.9.1 | Must | The Supplier and the Cloud Service Providers or anyone acting on their behalf shall comply with the requirements of the European General Data Protection Regulation ("GDPR"). |
| 1.9.2 | Should | The Supplier should describe the Legal Entities (including Legal Entities of Supplier, the Cloud Service Providers and/or others) that will have access to personal data (including by remote access) on behalf of NRK in connection with the WPS. |
| 1.9.3 | Must | The processing of personal data on behalf of NRK in relation to the WPS (including processing by Cloud Service Providers and/or others) shall be subject to a Data Processing Agreement in accordance with GDPR art 28. |
| 1.9.4 | Should | The Supplier should describe which personal Data processing activities in the WPS that will be governed by the Data processing agreement (DPA) executed with the Supplier, and should also describe which data processing activities in the WPS that will be governed by the Cloud Service Providers' own DPA. |
| 1.9.5 | Must | NRK's data in the WPS shall be stored in data centres within the EEA or in third countries (countries outside the EEA) subject to an adequacy decision issued by the EU Commission (cf. art 45 GDPR). |
| 1.9.6 | Should | The Supplier should describe the following for all applications and services in the WPS: the physical locations where data is stored, including backups (ie data centre location); and from where (physical location) the data may be accessed, including by remote access (eg support/security personnel) or by other means. |
| 1.9.7 | Must | Where the Supplier's or the Cloud Service Providers' services involves that NRK's personal data is transferred to a country outside the EEA ("third country"), including where said personal data is processed by the Supplier or the Cloud Service Providers or anyone acting on their behalf by way of remote access from a third country, the data processing activities shall: take place solely in jurisdictions recognised by the European Commission as providing adequate level of protection for personal data (art 45 GDPR); or the transfer of personal data is subject to appropriate safeguards pursuant to art 46 GDPR, such as the EU Standard Contractual Clauses (Commission decision 2021/914 of 4th June 2021) ("SCCs") or Binding Corporate Rules ("BCRs") approved by a relevant data protection authority, provided that the requirements for using such Clauses or BCRs are met. |
| 1.9.8 | Must | If any of NRK's data, including personal data for which NRK is the Controller, is transferred to a third country as part of the WPS, including by remote access, the Supplier shall answer the questions outlined in the attachment "Checklist - Transfer to Third Countries" and complete the Checklist. |
| 1.9.9 | Must | The WPS shall provide security for personal data, which covers all personal data processed in relation to the WPS in accordance with ISO 27701 and / or equivalent. |
| 1.9.10 | Must | The Supplier shall document how requirements for personal data security are ensured, including with regard to what organizational and technical security measures are implemented by the Supplier or by the Cloud Service Provider. |
| 1.9.11 | Must | The Supplier shall describe how the principle of Privacy by Design described in article 25 as well as the security requirements in art 32 of the GDPR are implemented in the WPS and related services. |

### 1.10 Information security (Clause 6.1)

All procurement in NRK must follow these requirements as a minimum Attachment 8 Vendor Security Requirements.

Additional requirements the Supplier must comply with requirements in the following table:

| # | Type of req | Requirement |
| --- | --- | --- |
| 1.10.1 | Must | The solution shall support granular network segmentation. |
| 1.10.2 | Must | The Supplier shall provide documentation on how to implement and integrate the solution in a secure manner, including API integrations, changing default passwords, and equivalent. |
| 1.10.3 | Must | The Supplier shall test self-developed solutions against known vulnerabilities such as OWASP Top10 or equivalent. |
| 1.10.4 | Should | SaaS solutions should follow the requirements of CSA CAIQ. |
| 1.10.5 | Should | The WPS should support single sign-on using oauth2/openid and/or SAML. |
| 1.10.6 | Should | The WPS can use either Active Directory or Azure Active Directory for users and groups. If the WPS has its own internal user and group database, the WPS should support provisioning users and groups using SCIM. |
| 1.10.7 | Must | The WPS shall support role-based access control. |
| 1.10.8 | Should | Communication between the users and the WPS system should be encrypted using TLS1.3 or higher. |
| 1.10.9 | Should | The WPS should be able to send security logs to a remote SIEM (Security Incident and Event Management) system. Currently NRK uses Azure Sentinel. |
| 1.10.10 | Should | The WPS should support API access using the Supplier’s specific access tokens or OAuth2. |
| 1.10.11 | Must | NRK's data and use of the WPS shall not be used for training or developing AI models or other AI capabilities without NRK's express written permission. |
| 1.10.12 | Should | The Supplier shall provide functionality or APIs that enable the Customer to export all configurations, workflows, and provisioning data in a standardized, machine-readable format upon termination of the agreement. |
| 1.10.13 | Should | The Supplier shall, upon request, provide and regularly update a Software Bill of Materials (SBOM) for the WPS in a standard machine-readable format |

# Appendix 2: The Supplier’s description of the service

Note: Requirement IDs in Appendix 2 correspond directly to the IDs used in Appendix 1 to ensure seamless mapping during evaluation.

## 2.1 Scope of the Agreement & the Service (Clause 1.1 & 2.1)

The objective of this RFP and the subsequent dialogue is to provide NRK with a WPS capable of meeting NRK’s long-term strategic goals.

### 2.1 Value for NRK

#### 2.1.1 Requirements - Value for NRK

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.1.1 | Must | Dynamic Resource Pooling & Sharing The system must provide real-time visibility into resource availability across all of NRK. It should automatically find and allocate the best-suited shared resources (whether physical studios, hardware, or software compute) for a given production, eliminating the need for dedicated, single-use setups. Minimum fulfilment of requirement is to be able to connect shared data centre resources to a production facility. Describe how your solution is able to make visibility of resources and how it operates and how it facilitates resource sharing. Describe how the solution will behave if there is a lack of available resources in the pool. |  |
| Supplier's Response |  |  |  |
| 1.1.2 | Should | Editorial Empowerment & Automation Non-technical staff must be able to plan and book standard setups independently through intuitive self-service solutions. By automating routine provisioning and routing tasks, the system will reduce human errors, accelerate time-to-air, and free MOC/technical operators from doing equipment allocation to focus on complex productions rather than manual routing. Describe how your solution supports Non-technical staff on planning and booking. Describe how the solution will behave if there is a lack of available resources in the pool. |  |
| Supplier's Response |  |  |  |
| 1.1.3 | Must | Support Sustainability The platform must make it easier to make sustainable choices. By making possible goals for "work from anywhere", remote productions and maximizing the reuse of centralized resources. As a minimum the system must directly contribute to reducing NRK’s carbon footprint by making centralised equipment available to productions or sites. Hence reducing the need for travel to or transport of media production equipment. Describe how your solution can support NRK in reducing carbon footprint for NRKs future production platform. If relevant use cases, please share one-pager about the environmental benefits in the different use cases. One-pager can be added as a combined pdf-file gathering all use cases into one document. |  |
| Supplier's Response |  |  |  |
| 1.1.4 | Should | Produce More with Confidence & Resilience The system must provide the structural reliability needed to safely schedule more concurrent productions on the same technical footprint. This includes robust exception handling and "Strategic Preemption"—the ability to automatically tear down or re-route lower-priority workflows to free up capacity for Priority 1 events like Breaking News. Describe how your solution supports scheduling productions and exception handling. Describe if the solution supports some kind of “demand for elevated rights” to cancel ongoing bookings, to free up resources. |  |
| Supplier's Response |  |  |  |

### 2.2. Users and respective value from WPS

#### 2.2.1 Direct Users

Direct users are those that NRK believes initially will interact directly with the WPS through its own user interfaces.

##### Requirements - Operators at the Media Operations Center (MOC)

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.2.1.1 | Should | Unified operational truth The system should act as the definitive repository for all routing states and provisioning data, providing a single, accurate overview of connections to replace manual logging. Describe how this will be managed by the solution, and in particular how the solution will make sure to get updated state from other systems, e.g. the Broadcast control system. |  |
| Supplier's Response |  |  |  |
| 1.2.1.2 | Should | Proactive Conflict Management The system should proactively detect scheduling Clashes and generate intelligent warnings to prevent the accidental severance of active, on-air broadcasts. Describe how the solution will distinguish between immediate resource conflicts that must be handled now, and future resource challenges that can be handled later. NRK should be able to adjust threshold values ​​for when and where "close" events that require immediate handling. Please describe how the system allows that. |  |
| Supplier's Response |  |  |  |
| 1.2.1.3 | Should | Automated Alternative Routing Upon detecting a resource conflict, the system should automatically calculate, validate, and suggest viable alternative routing paths to the operator |  |
| Supplier's Response |  |  |  |

##### Requirements - Ingest

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.2.2.1 | Should | Automated Port Allocation The system should facilitate the automated allocation and management of recording ports based on scheduled broadcast events. |  |
| Supplier's Response |  |  |  |
| 1.2.2.2 | Should | Real-Time Stream Monitoring The system should provide live, real-time visibility into the status of active ingest streams to quickly identify and mitigate potential data loss. |  |
| Supplier's Response |  |  |  |
| 1.2.2.3 | Should | Autonomous Configuration Capabilities The system should provide standardized templates allowing staff to independently configure complex ingest requirements without dependency on technical staff. |  |
| Supplier's Response |  |  |  |
| 1.2.2.4 | Should | Ingest time-slot expansion Due to the nature of ingest, there will be situations where a pre-ordered ingest-time, and the following resource allocation will need to be extended. Please describe how the solution will support this. |  |
| Supplier's Response |  |  |  |

##### Requirements - Post-production

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.2.3.1 | Should | Template-Based Provisioning The system should allow users to trigger standard, pre-defined workflow templates to quickly configure and access post-production resources. |  |
| Supplier's Response |  |  |  |
| 1.2.3.2 | Should | Dynamic Resource Pooling The system should seamlessly allocate shared data centre resources (such as compute/GPU power) based on real-time availability and task requirements. |  |
| Supplier's Response |  |  |  |
| 1.2.3.3 | Should | Affinity-based provisioning The system should be capable to allocate resources based on data centre affinity for the users profile, eg. If a user in the northern most office of NRK requests a post-production compute capacity, the system must book and allocate a resource from the northern most DC if there is capacity. If there is not available resources, the system should be capable of suggesting utilizing potential available resources from other DC-location. |  |
| Supplier's Response |  |  |  |

#### 2.2.2 Indirect Users

Indirect users will continue to utilise existing systems within NRK, however these systems and interfaces are subject to change.

##### Requirements - Technical Operations Managers (TOMs)

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.2.4.1 | Should | The system should allow for operational users to bulk activate connections between backend and front end devices and configurations on devices. This should be achievable by selecting a job or event for use. |  |
| Supplier's Response |  |  |  |

##### Requirements - Editorial Offices & Production Planners

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.2.5.1 | Should | Self-Service Provisioning The system should provide a dedicated, non-technical interface allowing editorial staff to independently plan and book standard setups. |  |
| Supplier's Response |  |  |  |
| 1.2.5.2 | Should | Advance Capacity Reservation The system should allow users to reliably reserve specific contribution equipment, physical rooms, and transmission lines well in advance of the actual production date. |  |
| Supplier's Response |  |  |  |
| 1.2.5.3 | Should | Schedule Integration The system should offer an integrated overview (e.g., calendar integration) that aligns technical resource requests directly with the editorial broadcast schedule. |  |
| Supplier's Response |  |  |  |
| 1.2.5.4 | Should | Allow Pre-order of jobs and events In a situation where users are planning a future event, the system should allow jobs/events to be created, if the status of the job/event is pre-order then no resources are allocated, however the system validates availability and highlights to the operator if the availability has a status of good or bad. |  |
| Supplier's Response |  |  |  |
| 1.2.5.5 | Should | Highlight future resource-limitations The system should be able to highlight and show future resources capacity issues. This so that the organization can prepare for any capacity expansion or other mitigating steps. |  |
| Supplier's Response |  |  |  |

##### Requirements - Management & Strategy

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.2.6.1 | Should | Analytics and Utilization Reporting The system should provide macro-level dashboards and extractable data demonstrating how frequently specific physical and digital resources are utilized. |  |
| Supplier's Response |  |  |  |
| 1.2.6.2 | Should | Output Forecasting The system should supply predictive capacity metrics to help identify future resource deficits or surpluses well in advance of large events. |  |
| Supplier's Response |  |  |  |
| 1.2.6.3 | Should | Financial Optimization Insights The system should allow analysts to evaluate resource availability against historical booking trends to identify inefficiencies and empirically justify budget decisions. |  |
| Supplier's Response |  |  |  |

### 2.3. NRK Concepts

NRK concepts highlight NRKs desire to increase efficiency in planning, execution and monitoring of jobs and events. Is it believed that the WPS is best suited to enable these desired outcomes.

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.3.1 | Should | End to End Production Planning Please reflect about how proposed solution could support this |  |
| Supplier's Response |  |  |  |
| 1.3.2 | Should | Contextualised Monitoring & Alarming Please reflect about how proposed solution could support this |  |
| Supplier's Response |  |  |  |
| 1.3.3 | Should | Programme Templates Please reflect about how proposed solution could support this |  |
| Supplier's Response |  |  |  |

### 2.4 . Functional and technical requirements

#### 2.4.1. Resource Management

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.4.1.1 | Should | The WPS should have a method for creating resource pools. |  |
| Supplier's Response |  |  |  |
| 1.4.1.2 | Should | The WPS should have a method for adding capabilities to resources. |  |
| Supplier's Response |  |  |  |
| 1.4.1.3 | Should | The WPS should have a method for adding ‘minimum requirements’ to resources in the case of software services for example. |  |
| Supplier's Response |  |  |  |
| 1.4.1.4 | Should | The WPS should have a method for enabling users to create ‘composite resources’ for example video and audio ST2110 essence can be combined to create a single resource. |  |
| Supplier's Response |  |  |  |
| 1.4.1.5 | Should | The WPS should provide a method allowing users to ‘breakaway’ essence within a composite resource. |  |
| Supplier's Response |  |  |  |
| 1.4.1.6 | Should | The WPS should allow for metadata to be added to resources such as location. |  |
| Supplier's Response |  |  |  |
| 1.4.1.7 | Should | The WPS should allow for the linking of resources, for example where certain resources are always used with another resource, having a link to provision both at the same time is desirable. |  |
| Supplier's Response |  |  |  |
| 1.4.1.8 | Should | The WPS should have a method for capturing system virtual resources, for example intercom IFBs & Conferences. |  |
| Supplier's Response |  |  |  |

#### 2.4.2 Scheduling & Clash Management

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.4.2.1 | Should | The WPS should have a scheduling function allowing for planning of events up to 24 months in the future, including resource allocation and warnings regarding conflicts. |  |
| Supplier's Response |  |  |  |
| 1.4.2.2 | Should | The WPS should have a ‘repeat event’ function to allow for replication of events that occur on a daily, weekly, monthly, yearly basis. |  |
| Supplier's Response |  |  |  |
| 1.4.2.3 | Should | The WPS should allow for repeat events to be modified in isolation without affecting others within the series. |  |
| Supplier's Response |  |  |  |
| 1.4.2.4 | Should | The WPS should allow the user to choose if changes to a repeat event should be replicated to all future events or only to the selected event. |  |
| Supplier's Response |  |  |  |
| 1.4.2.5 | Should | The WPS should have a method for detecting where resources are not available and highlighting to the operator. This should be a continuous procedure as opposed to waiting until all data is populated. |  |
| Supplier's Response |  |  |  |
| 1.4.2.6 | Should | The WPS should be able to display connections between resources that are made using a third party system such as the BCS. |  |
| Supplier's Response |  |  |  |
| 1.4.2.7 | Should | The WPS should not break any connections made by third party systems automatically. Describe how conflicts will be highlighted to the user, and could be handled. |  |
| Supplier's Response |  |  |  |
| 1.4.2.8 | Should | Should a third party system connection utilise resources that are planned for an upcoming event, the WPS highlight this to the relevant operational users and suggest alternative resources if available |  |
| Supplier's Response |  |  |  |
| 1.4.2.9 | Should | The WPS should allow for multiple views of schedule. This includes a job/event view and Resource views. |  |
| Supplier's Response |  |  |  |
| 1.4.2.10 | Should | All views should also show third party connections |  |
| Supplier's Response |  |  |  |

#### 2.4.3 Workflows

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.4.3.1 | Should | The WPS should support workflows, where resource pools are added and connected together. |  |
| Supplier's Response |  |  |  |
| 1.4.3.2 | Should | It should be possible within a workflow to set rules for individual resource pools. |  |
| Supplier's Response |  |  |  |
| 1.4.3.3 | Should | Workflows may be applied to multiple jobs concurrently. |  |
| Supplier's Response |  |  |  |
| 1.4.3.4 | Should | Workflows may be edited on a ‘per job’ basis, without change affecting existing jobs utilising the same workflow nor the originating workflow. |  |
| Supplier's Response |  |  |  |
| 1.4.3.5 | Should | It should be possible to create workflows via API |  |
| Supplier's Response |  |  |  |
| 1.4.3.6 | Should | It should be possible to modify workflows via API |  |
| Supplier's Response |  |  |  |

#### 2.4.4 API

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.4.4.1 | Must | The system must have an API. The API must be documented. |  |
| Supplier's Response |  |  |  |
| 1.4.4.2 | Should | The API documentation should be complete, describing all endpoints. Documentation of this requirement is solved by giving NRK access to either OpenAPI or a PDF-file containing the complete API documentation. |  |
| Supplier's Response |  |  |  |
| 1.4.4.3 | Should | The WPS should have a HTTPS REST or GraphQL API. |  |
| Supplier's Response |  |  |  |
| 1.4.4.4 | Should | The WPS should support north and south bound APIs. To connect to WPS from other systems and to push actions in other systems. |  |
| Supplier's Response |  |  |  |
| 1.4.4.5 | Should | The API should cover all functionality in the Product. Describe what is not possible via API, if any. |  |
| Supplier's Response |  |  |  |
| 1.4.4.6 | Should | The API should be documented using OpenAPI. |  |
| Supplier's Response |  |  |  |
| 1.4.4.7 | Should | The API should have an interactive Swagger interface. |  |
| Supplier's Response |  |  |  |
| 1.4.4.8 | Should | The WPS should have support for configuration using Ansible or Terraform with an Ansible module or Terraform provider supported by the Supplier. |  |
| Supplier's Response |  |  |  |
| 1.4.4.9 | Should | The WPS should support administrator-configurable webhooks to interact with other generic APIs (e.g., on status updates). |  |
| Supplier's Response |  |  |  |
| 1.4.4.10 | Should | The WPS should support sending and receiving events on a service bus using AMQP. |  |
| Supplier's Response |  |  |  |

#### 2.4.5 User Interface & User Experience

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.4.5.1 | Should | The WPS should have a standardised set of ‘out of the box’ (OOTB) user interfaces. |  |
| Supplier's Response |  |  |  |
| 1.4.5.2 | Should | It should be possible to customise these ‘OOTB’ interfaces. |  |
| Supplier's Response |  |  |  |
| 1.4.5.3 | Must | The WPS user interface must be accessible as a responsive web interface. |  |
| Supplier's Response |  |  |  |
| 1.4.5.4 | Should | The WPS should meet the latest Web Content Accessibility Guidelines (WCAG) at levels A and AA. |  |
| Supplier's Response |  |  |  |
| 1.4.5.5 | Should | The WPS web interface should support the common browser versions from Edge, Chrome, Firefox, and Safari. |  |
| Supplier's Response |  |  |  |
| 1.4.5.6 | Should | The user interface should be adaptable to NRK's own design guidelines and visual identity. The solution must be flexible enough to accommodate specific user needs when configuring tools for different workflows. This includes the ability to override the system's default styling, as well as supporting the creation and use of custom views, templates, or presets for specialized use cases. |  |
| Supplier's Response |  |  |  |
| 1.4.5.7 | Should | The WPS should support usage of standard design tools, such as Figma, for the creation and development of user interfaces. |  |
| Supplier's Response |  |  |  |
| 1.4.5.8 | Should | It should be possible to develop a standalone user interface in any language NRK chooses and connect it to the WPS with full capabilities available. |  |
| Supplier's Response |  |  |  |
| 1.4.5.9 | Should | It should be possible to reuse components across multiple user interfaces as opposed to creating bespoke components for each interface. |  |
| Supplier's Response |  |  |  |

#### 2.4.6 Monitoring & Alarming

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.4.6.1 | Should | The WPS should be capable of connecting to external data aggregators and visualizing in monitoring overview. |  |
| Supplier's Response |  |  |  |
| 1.4.6.2 | Should | The WPS should be capable of taking external data and contextualising it with data already stored to provide a more useful overview to operators. |  |
| Supplier's Response |  |  |  |
| 1.4.6.3 | Should | The WPS should have an integration to Grafana, Loki, Mimir. If not in place at the current point of time, please describe how this will be performed and the estimated time of delivery. |  |
| Supplier's Response |  |  |  |

### 2.5 Future development of WPS and roadmap

| # | Type of req | Requirement |  | Compliance Y/P/N |
| --- | --- | --- | --- | --- |
| 1.5.1 | Must | 3-year roadmap The Supplier must share their roadmap for the next three years. |  |  |
| Supplier's Response |  |  |  |  |
| 1.5.2 | Should | Roadmap alignment to NRK needs The Supplier should indicate where the roadmap items map to NRK requirements and state where the Supplier is willing to prioritise items to increase alignment with the NRKs timeline. |  |  |
| Supplier's Response |  |  |  |  |
| 1.5.3 | Should | Design & architecture alignment The proposed solution in combination with the roadmap fully aligns with NRKs design principles, system architecture and technical requirements within the given timelines. |  |  |
| Supplier's Response |  |  |  |  |

### 2.6 Environment And Sustainability Requirements

Adherence to the requirements is covered in must-requirement 1.1.1 and 1.1.3. However NRK also wants to focus on sustainability in the cloud deliveries with the following must-requirements.

Minimum Environment and Sustainability Requirements

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.6.1 | Must | The primary data centre(s) hosting the solution must have a documented Power Usage Effectiveness (PUE) of no higher than 1.6 at the time of contract signing. Documentation can be added as a combined PDF-file. The Supplier must, on request, provide NRK with a status report documenting the current PUE and detailing specific technical or operational measures implemented to improve energy efficiency. |  |
| Supplier's Response |  |  |  |
| 1.6.2 | Must | The Supplier and/or the subcontractor hosting the cloud service must have documented routines for responsibly recycling and reusing physical IT hardware (E-waste/WEEE). Documentation: Submit one of the following to prove compliance: a relevant environmental certification (e.g., ISO 14001, R2 or equivalent) including the specific E-waste routine, proof of a national WEEE registration (e.g., ERP, RENAS or equivalent) alongside an internal policy, an ITAD policy involving a certified recycling partner, or other equivalent documentation showing responsible E-waste management. Documentation can be added as a combined PDF-file. Maintenance: Compliance must be maintained throughout the contract. NRK must be notified and receive updated documentation if the chosen method, cloud provider, or recycling partner changes. |  |
| Supplier's Response |  |  |  |

### 2.7. Infrastructure requirements

The WPS is to be delivered as a SaaS solution, and support a distributed architecture allowing for operation across NRK. The solution must be connected to multiple of NRKs data centres.

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.7.1 | Must | The WPS system must be provided via a proven redundant model, for high availability and to support disaster recovery. Describe your redundancy model (active-passive / active-active), and the geographical split on data centres (zone split in one data centre or multi region/country. |  |
| Supplier's Response |  |  |  |
| 1.7.2 | Must | The WPS system must support NRK’s dual datacentre architecture as described previously in appendix 1. |  |
| Supplier's Response |  |  |  |
| 1.7.3 | Should | The WPS system is transparent in terms of sub-suppliers and hosting providers. NRK is informed on any changes in hosting providers. Inform about which hosting partners you use, as well as the location for data storage. Inform about how any changes in data storage will occur (will you inform/consult the customer, inform about choices, the customer's ability to reject the transfer of data storage (notice period)). Inform about the supply chain, and how you inform the customer about any changes in the supply chain. |  |
| Supplier's Response |  |  |  |
| 1.7.4 | Should | The WPS system has a database, with redundancy and resilience capabilities. Describe your model for database resilience, e.g. synchron replication and if there is automatic failover. |  |
| Supplier's Response |  |  |  |
| 1.7.5 | Should | In case of a system failure, recovery functionalities that minimize recovery time are in place. Describe your recovery model, and estimated time for recovery. |  |
| Supplier's Response |  |  |  |
| 1.7.6 | Should | Hosting and providing a service like WPS, which are crucial for operations, should have a systematic approach to observability/ monitoring and scaling around performance in the system. <Describe your model for system performance, ICT performance and scaling.> |  |
| Supplier's Response |  |  |  |

### 2.8 Documentation and training (Clause 3.4)

It is expected that the WPS will be a complex system and potentially be made up of many elements. To aid NRK in taking ownership of the WPS, clear training and documentation is required that is regularly reviewed and updated by the Supplier. Training materials can be in English or Norwegian language.

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.8.1 | Must | The Supplier must provide sufficient documentation for the system. Please state how documentation is provided, and the form of it. Specify if there are training videos available etc. Specify how customers get access to documentation (requires log in etc.) |  |
| Supplier's Response |  |  |  |
| 1.8.2 | Must | The Supplier must provide suitable training, that can be tiered to users of varying levels, from operators through to developers and administrators of the system. Describe your recommended training, and estimate the amount of training hours pr. Group of users: - Planners (production)/ Technical operating managers - MOC operators - System administrators |  |
| Supplier's Response |  |  |  |
| 1.8.3 | Should | Specify how possible tailor made solutions for NRK are documented, and how NRK-users will be guided to that documentation. |  |
| Supplier's Response |  |  |  |
| 1.8.4 | Should | Training to be offered for Planners (production)/ Technical operating managers: Please explain what content that is proposed included in a training session like this. Please add an indicative agenda that shows topics and timing for a typical session like this. |  |
| Supplier's Response |  |  |  |
| 1.8.5 | Should | Training to be offered for MOC operators: Please explain what content that is proposed included in a training session like this. Please add an indicative agenda that shows topics and timing for a typical session like this. |  |
| Supplier's Response |  |  |  |
| 1.8.6 | Should | Training to be offered for System administrators: Please explain what content that is proposed included in a training session like this. Please add an indicative agenda that shows topics and timing for a typical session like this. |  |
| Supplier's Response |  |  |  |
| 1.8.7 | Should | Training for specific user groups should be possible to provide on site at customers premises. Training sessions will be ordered by NRK. Please state how this is handled regards to: - Time from order to completion - Cost pr. User group session, including travel (Costs to be specified in 2.2 Appendix 6a - Price scheme - RFP_WPS, table 8). |  |
| Supplier's Response |  |  |  |
| 1.8.8 | Should | Online training should be offered that allows WPS users, developers and administrators to gain further knowledge and experience at their own speed. |  |
| Supplier's Response |  |  |  |
| 1.8.9 | Should | It is desirable that there is a forum/community available to NRK to connect to existing users, please elaborate if this is available for the solution. |  |
| Supplier's Response |  |  |  |
| 1.8.10 | Should | The Supplier should provide a forum/community/way of NRKs system experts/administrators connecting directly to Supplier SMEs for help and support. |  |
| Supplier's Response |  |  |  |

### 2.9 Personal data (Clause 6.2)

Document 2.4 Attachment 4 - Data Processing Agreement with subdocuments. regulates the use of personal data. In addition the following requirements will apply.

|  | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.9.1 | Must | The Supplier and the Cloud Service Providers or anyone acting on their behalf shall comply with the requirements of the European General Data Protection Regulation ("GDPR"). |  |
| Supplier's Response |  |  |  |
| 1.9.2 | Should | The Supplier should describe the Legal Entities (including Legal Entities of Supplier, the Cloud Service Providers and/or others) that will have access to personal data (including by remote access) on behalf of NRK in connection with the WPS. |  |
| Supplier's Response |  |  |  |
| 1.9.3 | Must | The processing of personal data on behalf of NRK in relation to the WPS (including processing by Cloud Service Providers and/or others) shall be subject to a Data Processing Agreement in accordance with GDPR art 28. |  |
| Supplier's Response |  |  |  |
| 1.9.4 | Should | The Supplier should describe which personal Data processing activities in the WPS that will be governed by the Data processing agreement (DPA) executed with the Supplier, and should also describe which data processing activities in the WPS that will be governed by the Cloud Service Providers' own DPA. |  |
| Supplier's Response |  |  |  |
| 1.9.5 | Must | NRK's data in the WPS shall be stored in data centres within the EEA or in third countries (countries outside the EEA) subject to an adequacy decision issued by the EU Commission (cf. art 45 GDPR). |  |
| Supplier's Response |  |  |  |
| 1.9.6 | Should | The Supplier should describe the following for all applications and services in the WPS: the physical locations where data is stored, including backups (ie data centre location); and from where (physical location) the data may be accessed, including by remote access (eg support/security personnel) or by other means. |  |
| Supplier's Response |  |  |  |
| 1.9.7 | Must | Where the Supplier's or the Cloud Service Providers' services involves that NRK's personal data is transferred to a country outside the EEA ("third country"), including where said personal data is processed by the Supplier or the Cloud Service Providers or anyone acting on their behalf by way of remote access from a third country, the data processing activities shall: take place solely in jurisdictions recognised by the European Commission as providing adequate level of protection for personal data (art 45 GDPR); or the transfer of personal data is subject to appropriate safeguards pursuant to art 46 GDPR, such as the EU Standard Contractual Clauses (Commission decision 2021/914 of 4th June 2021) ("SCCs") or Binding Corporate Rules ("BCRs") approved by a relevant data protection authority, provided that the requirements for using such Clauses or BCRs are met. |  |
| Supplier's Response |  |  |  |
| 1.9.8 | Must | If any of NRK's data, including personal data for which NRK is the Controller, is transferred to a third country as part of the WPS, including by remote access, the Supplier shall answer the questions outlined in the attachment "Checklist - Transfer to Third Countries" and complete the Checklist. |  |
| Supplier's Response |  |  |  |
| 1.9.9 | Must | The WPS shall provide security for personal data, which covers all personal data processed in relation to the WPS in accordance with ISO 27701 and / or equivalent. |  |
| Supplier's Response |  |  |  |
| 1.9.10 | Must | The Supplier shall document how requirements for personal data security are ensured, including with regard to what organizational and technical security measures are implemented by the Supplier or by the Cloud Service Provider. |  |
| Supplier's Response |  |  |  |
| 1.9.11 | Must | The Supplier shall describe how the principle of Privacy by Design described in article 25 as well as the security requirements in art 32 of the GDPR are implemented in the WPS and related services. |  |
| Supplier's Response |  |  |  |

### 2.10 Information security (Clause 6.1)

All procurement in NRK must follow these requirements as a minimum Attachment 8 Vendor Security Requirements.

Additional requirements the Supplier must comply with requirements in the following table:

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 1.10.1 | Must | The solution shall support granular network segmentation. |  |
| Supplier's Response |  |  |  |
| 1.10.2 | Must | The Supplier shall provide documentation on how to implement and integrate the solution in a secure manner, including API integrations, changing default passwords, and equivalent. |  |
| Supplier's Response |  |  |  |
| 1.10.3 | Must | The Supplier shall test self-developed solutions against known vulnerabilities such as OWASP Top10 or equivalent. |  |
| Supplier's Response |  |  |  |
| 1.10.4 | Should | SaaS solutions should follow the requirements of CSA CAIQ. |  |
| Supplier's Response |  |  |  |
| 1.10.5 | Should | The WPS should support single sign-on using oauth2/openid and/or SAML. |  |
| Supplier's Response |  |  |  |
| 1.10.6 | Should | The WPS can use either Active Directory or Azure Active Directory for users and groups. If the WPS has its own internal user and group database, the WPS should support provisioning users and groups using SCIM. |  |
| Supplier's Response |  |  |  |
| 1.10.7 | Must | The WPS shall support role-based access control. |  |
| Supplier's Response |  |  |  |
| 1.10.8 | Should | Communication between the users and the WPS system should be encrypted using TLS1.3 or higher. |  |
| Supplier's Response |  |  |  |
| 1.10.9 | Should | The WPS should be able to send security logs to a remote SIEM (Security Incident and Event Management) system. Currently NRK uses Azure Sentinel. |  |
| Supplier's Response |  |  |  |
| 1.10.10 | Should | The WPS should support API access using the Supplier’s specific access tokens or OAuth2. |  |
| Supplier's Response |  |  |  |
| 1.10.11 | Must | NRK's data and use of the WPS shall not be used for training or developing AI models or other AI capabilities without NRK's express written permission. |  |
| Supplier's Response |  |  |  |
| 1.10.12 | Should | The Supplier shall provide functionality or APIs that enable the Customer to export all configurations, workflows, and provisioning data in a standardized, machine-readable format upon termination of the agreement. |  |
| Supplier's Response |  |  |  |
| 1.10.13 | Should | The Supplier shall, upon request, provide and regularly update a Software Bill of Materials (SBOM) for the WPS in a standard machine-readable format |  |
| Supplier's Response |  |  |  |

## 2.11 The rights of the parties (Clause 7.1)

The Supplier must evaluate whether NRK has specified any modifications to the parties' existing intellectual property or usage rights in Appendix 1, and provide their response in the table below.

| ID | Supplier’s response |
| --- | --- |
| 2.11.1 | <Supplier response> |

## 2.12 Reconstruction of data (Clause 8)

The Supplier must evaluate whether NRK has specified any extended liability for data reconstruction in Appendix 1, and state their corresponding obligations in the table below.

| ID | Supplier’s response |
| --- | --- |
| 2.12.1 | <Supplier response> |

# Appendix 3: Plan for the establishment phase

## Deliverables and timeline

This chapter outlines the defined deliverables and integrations for the WPS within the defined scope of the first two years. Each deliverable and integration are directly linked to the fulfilment of NRK’s key strategic concepts for the WPS, as detailed in the table below.

This chapter outlines the functional and operational milestones for the project, which shall be addressed within the detailed implementation timeline in Chapter 4.1.

Additionally, Section 3.1 provides an overview of the programme and implementation schedules to give the Supplier clear insight into the schedule and functionality NRK expects.

## 3.1 Plan for the establishment phase (Clause 3.2)

NRKs aim is to have the WPS core functionality in place, tested and ready for operation in Q1-2028 when we prepare to vacate the Marienlyst site and move into Normannsløkka.

It is crucial that by this point functionality is available to support at least the operational deliverables detailed below so that the system ‘works from day one’ for operations. A failure to achieve this may lead to a lack of confidence in the system and lack of uptake in use.

Shown below are our initial thoughts on operational deliverables and defined integrations, these are indicative deliverables that NRK believes will be more specified during design and development, in cooperation with the selected Supplier.

Figure 4. There is a high-resolution PDF attached to the RFP.

### 3.1.1 Pre-requisite  - LAB and Stage Environment

It is paramount that NRK have a non-production environment to develop and test new features and workflows.

To enable this, there is a need for a system to be provided that is part of NRKs Lab or Staging environments. These environments serve two different purposes so it may only be required to have a WPS instance for one.

###### Lab

The lab environment is a sandbox where multiple technology teams are able to connect devices and services together on a network and run testing and evaluation.

###### Stage

The Stage environment is designed to be a mimic of the production platform, in that the network and other systems are configured as they are in production to provide a real testable environment prior to the onboarding of new devices or release of new software and workflows to the Production environment.

### 3.1.2 Operational Deliverables

NRK believes that the best approach for the delivery of a complete WPS is to identify smaller deliverables and focus on one at a time as opposed to attempting to develop and then deliver the system in its entirety.

###### 3.1.2.1 Ingest Management - EVS Ingest, BCS

Ingest management entails the WPS managing the Ingest system record ports, allocating available ports to all users who have selected sources they wish to have recorded. After initial setup the WPS would be required to be aware of all ingest ports in both datacentres, their availability, which datacentre the source originates in and any rules regarding single or dual recording of sources. Some preapproved sources may come from outside the datacentres, the WPS would know which these sources are and how to handle them.

The WPS would then be expected to pass the relevant metadata to the ingest system to create recordings with name standard titles, and then instruct the routing system to connect the source to the ingest ports destination at the appropriate time. Last minute changes to the recording schedule by the user would also be handled without the need for manual intervention.

###### 3.1.2.2 Radio Scheduling - What's On, DigAs, BCS

Radio schedules are created in separate systems, these schedules include both live shows and also pre-recorded shows played from auto-players.

By receiving this data, the WPS can provision auto-players for pre-recorded shows and where the schedule details the location of the live show, it can book a studio and resources required.

###### 3.1.2.3 Device Onboarding A/B

Device onboarding refers to the creation of resource pools for a specific device type. It is anticipated that there will be many devices that NRK wish to onboard and these shall be phased accordingly based upon the functionality deliverables. It is not in this instance about the integration of parameter control of the device.

It is anticipated that the integration of devices for provisioning will be a staggered approach as more services become available. In this example, devices are split into two separate deliveries, prioritising those that fit best with the operational requirements.

###### 3.1.2.4 Gallery Configuration A

Gallery configuration is a larger implementation with the need for phasing in its delivery.

Not only does this encompass connecting gallery surfaces to backend services, it also includes the activation of programme specific configuration of devices, these include slow mo servers, intercom and multiviewers, this is data capture within the Production Manifest.

The initial release of Gallery Configuration will be to deploy a Production Manifest configuration to a gallery.

It is desirable that when a TOM arrives to a gallery they are able to ‘activate’ an Event/Job that will trigger the connection of services and configuration of devices.

###### 3.1.2.5 Gallery Configuration B

The latter release is required to encompass the run-time changes that have been made during a production, such as intercom panel configuration & UMD updates.

It must be possible to then deploy the Production manifest with runtime configuration changes to a second gallery in the event of a failure or a need to change to a different location.

###### 3.1.2.6 Auto Job/Event Creation

The purpose of Auto Event Creation is where the WPS ‘listens’ to integrated systems for uses rules and potentially AI to determine if there are items within a news rundown for example that require a job or event to be created and resources provisioned.

###### 3.1.2.7 Self Service A/B

A core objective is that end users are able to request resources themselves without the need for MOC to assist. The key benefits of this is reducing the operational load on MOC operators so that they are able to focus more on their core tasks.

This is considered as a phased deliverable, whereby in the first instance MOC operators will be able to create a job and select a defined workflow e.g “audio contribution via  Quantum-codec”. This must be enough details for a MOC operator to add, to create the job and allocate a resource from a resource pool specified within the workflow. The purpose of this phase is to prove that the rules and workflows that have been created are functioning correctly before exposing it to a wider user group for testing.

The latter phase of this is a simplified user interface (integrated in their preferred system, or stand alone) that allows end users to request resources for their specific show, ideally without the need to specify where they are working, this is autodetected based on login.

Users would need to be able to request resources.

###### 3.1.2.8 Contextual Alarming

Contextual alarming relies on both the WPS and PMCS stack.

Utilising the contextual data that the WPS has of what an event is called and when, what devices are in use for that event, what transmission paths are in use etc; and from PMCS having the monitoring and alarming data the WPS is able to create dashboards for events dynamically that populate with video/audio monitoring and also alarming data. Coupling this functionality with Penalty boxing gives MOC greater context and access to monitoring of the sources that matter most.

### 3.1.3 Integrations

It is anticipated that there will be many integrations between the WPS and existing systems and devices that NRK is utilising across its estate.

The following systems have for now been identified for integration:

##### 3.1.3.1 Vimbiz

Vimbiz is a specialized Broadcast Enterprise Resource Planning (ERP) platform designed to centralize and streamline media operations. It serves as a unified ecosystem that integrates scheduling, rentals, and engineering workflows to maximize resource efficiency and reduce operational costs for broadcasters and media companies. Media organizations utilize VimBiz to handle complex technical and human resource scheduling, track equipment rental logistics and availability, and manage the end-to-end lifecycle of technical assets through preventative maintenance and service tracking. By consolidating these disparate workflows into a single system, it enhances collaboration and operational productivity across demanding, budget-conscious production environments.

Integration need

The integration to VimBiz is to provide more data that can be used to calculate actions in the WPS. For example, Vimbiz holds details of employees and the roles that they are capable of doing, this is of high value to the WPS as it provides pools of resources and defines their capabilities.

##### 3.1.3.2 NetBox

NetBox is a comprehensive infrastructure resource management (IRM) application designed to serve as the single source of truth for network automation and engineering. It is primarily utilised to document and model the physical and logical layout of network infrastructure, combining IP address management (IPAM) with data centre infrastructure management (DCIM). Network teams use NetBox to track physical hardware, rack layouts, cabling, virtual machines, and IP address allocations in a centralized platform. By maintaining an accurate, API-driven model of the network's intended state, it eliminates disparate spreadsheets and enables organizations to safely automate provisioning and configuration workflows.

Integration need

The WPS is required to have an integration with NetBox so that the WPS can capture relevant logical details such as, Organisation, Devices, IP addressing. This integration will provide a large amount of the data required to be associated with resources and also be utilised by rules to make decisions.

##### 3.1.3.3 Broadcast Control System (BCS)

The chosen BCS for NRK is Sony Nevions VideoIpath. The BCS is the tool that operators use for making connections between endpoints, endpoint groups and virtual groups.

Integration need

The WPS is required to have bi-directional communication with the BCS to send commands and receive responses. It is also required that the WPS shall receive notifications from the BCS of certain activities that take place, for example endpoint connections.

##### 3.1.3.4 PMCS Stack

NRK employs a full PMCS stack made up of a number of different products. These act as collectors, data stores, analysers, graphical visualisers and alert notifications.

The approach that NRK are aiming to take is that PMCS will be the central resource for capturing data and alarms and making them available to other systems or visualising them directly.

Integration need

It is expected that there will be integration between PMCS and WPS as the WPS will hold the contextual data that can be used to collate the relevant data and alarms together. For example; WPS will be aware of an event and all resources that have been provisioned. This data may be passed to PMCS which will then generate displays and alarming dashboards, or it may be that the WPS queries PMCS about the resources to get health and alarm metrics to display.

##### 3.1.3.5 ENPS

Associated Press ENPS is a news production system designed to streamline and manage the entire broadcast journalism workflow. It serves as a centralized hub where news teams collaborate to create, edit, and organize content for television, radio, and digital platforms. Journalists and producers utilize ENPS to write scripts, manage wire feeds, monitor breaking news, and build show rundowns in real time. By integrating directly with control room automation, teleprompters, and media servers, it ensures seamless communication between the newsroom and the studio during live broadcasts.

Integration need

The long term aim is that the WPS will be able to read rundowns and correlate them to scheduled programmes (from What's On) and then calculate from the rundown items if any of them need resources provisioning, auto-creating jobs with the relevant titles based on the rundown data.

##### 3.1.3.6 What's On

What's On is used by NRK to generate schedules for both TV and Radio channels.

Integration Need

It is believed that having an integration to What’s On will greatly enhance the potential for automation of services within NRK.

By utilising schedules, the creation of jobs and events may take place. Where there are shows that are daily or weekly, these patterns can be learnt and monitored.

##### 3.1.3.7 David Systems - Digas

DigAs is the ecosystem for producing and publishing radio content in NRK. Whereas What's On produces a container for the content Object with an unique ID, called Program ID, DigAs will handle all functionality and assets attached to each container. Context such as production mode:

- live

- pre-production

- radio

- podcast,

- self operated

Relation to other rooms, function, studios, metadata, actions in the playlist that triggers functionality (external contributions).

All content assets produced by DigAs ecosystems are made available for the web player in an automated process.

All these capabilities are needed to tailor an efficient and robust user experience that enables the user to focus on producing a good story and relation to all contributors of the content.

Integration Need

It is believed that having an integration to Digas will greatly enhance the potential for automation of services within NRK.

By utilising schedules, the creation of jobs and events may take place. Where there are shows that are daily or weekly, these patterns can be learnt and monitored.

##### 3.1.3.8 Configuration Manager

The Configuration Manager is NRKs intention to have it as a NRK developed platform. The core purpose of this is the deployment of Media Functions and configuration data to end devices. NRK will explore how WPS and Configuration Management will complement each other together with the selected Supplier.

Integration need

It is seen that the WPS would connect to the Configuration Manager (developed inhouse at NRK) and instruct it as to where to provision devices/services (Datacentre or server) and also what configuration to load, this may include parameters that are specific to the event that the devices have been booked for.

Figure 5. There is a high-resolution PDF attached to the RFP.

##### 3.1.3.9 Other plausible integrations

NRK believes that there might be needs for other integrations like Entra ID, Riedel Intercom etc. Please advice.

##### Requirements - Integrations

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 3.1.3.1 | Should | Asynchronous messaging All integrations should support asynchronous messaging. |  |
| Supplier's Response |  |  |  |
| 3.1.3.2 | Should | Vimbiz integration The solution should have an integration to Vimbiz. If not in place at the current point of time, please describe how this will be performed and the estimated time of delivery. |  |
| Supplier's Response |  |  |  |
| 3.1.3.3 | Should | NetBox integration The solution should have an integration to NetBox. If not in place at the current point of time, please describe how this will be performed and the estimated time of delivery. |  |
| Supplier's Response |  |  |  |
| 3.1.3.4 | Should | Sony Nevion VideoIPath integration (BCS) The solution should have an integration to Sony Nevion VideoIPath. If not in place at the current point of time, please describe how this will be performed and the estimated time of delivery. |  |
| Supplier's Response |  |  |  |
| 3.1.3.5 | Should | Grafana integration The Supplier should have an integration to Grafana, Loki, Mimir. If not in place at the current point of time, please describe how this will be performed and the estimated time of delivery. |  |
| Supplier's Response |  |  |  |
| 3.1.3.6 | Should | ENPS integration The Supplier should have an integration to ENPS. If not in place at the current point of time, please describe how this will be performed and the estimated time of delivery. |  |
| Supplier's Response |  |  |  |
| 3.1.3.7 | Should | What's On integration The solution should have an integration to What's On. If not in place at the current point of time, please describe how this will be performed and the estimated time of delivery. |  |
| Supplier's Response |  |  |  |
| 3.1.3.8 | Should | DAVID System, Digas The solution should have an integration to David systems Digas. If not in place at the current point of time, please describe how this will be performed and the estimated time of delivery. |  |
| Supplier's Response |  |  |  |
| 3.1.3.9 | Should | Other plausible integrations NRK believes that there might be needs for other integrations like Entra ID, Riedel Intercom etc. Please advice based on your experience. |  |
| Supplier's Response |  |  |  |

### 3.1.4 Execution plan and approach: Deliverables and Integrations

Request for information on timeline, NRK involvement and Way of working.

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 3.1.4.1 | Should | Timeline Based on the Operational deliveries and integrations stated above, and the order indicated on the estimated timeline from NRK, please reflect on how this can be done, and how you as the Supplier can facilitate that NRK has a working WPS system as soon as possible, with at least at Minimum Viable Product (MVP) by Q2-27, and fully working system by Q1 28. |  |
| Supplier's Response |  |  |  |
| 3.1.4.2 | Should | NRK involvement A successful implementation of the WPS requires close collaboration. NRK aims to minimize the burden on its internal resources while ensuring progress and quality. The Supplier shall describe their methodology for managing and minimizing NRK’s required involvement throughout the project. The description must, at a minimum, include: Roles and responsibilities (RACI) A clear overview of the key roles NRK must provide and their specific responsibilities during each project phase. Activity plan for involvement A concrete schedule of the activities requiring NRK’s participation, including the expected time commitment for each activity (Table 4 & Table 6 in “2.2 Appendix 6a - Price scheme - RFP_WPS”). Resource optimization An explanation of how the Supplier’s standardized methods, tools, or best practices ensure that NRK’s time investment is kept as low as possible without increasing project risk. Pricing model and risk mitigation The Supplier should state how their delivery model supports a fixed-price structure for the implementation project. —-------------------------- When evaluating this requirement, NRK will emphasize: Low operational involvement Methodologies and solutions that demonstrably lead to a streamlined implementation process with low resource demand from NRK will be rewarded. Clarity and realism The maturity, detail, and predictability of the Supplier’s plan for NRK’s involvement. Vague or unrealistic assumptions will result in a lower score. Risk reduction Proposals that offer a robust, comprehensive delivery framework backed by a fixed-price model for the implementation project will be rewarded, as this shifts the delivery risk away from NRK and indicates a highly predictable and mature implementation methodology. |  |
| Supplier's Response |  |  |  |
| 3.1.4.3 | Should | Way of working Please describe how you as the Supplier would have organized the work to achieve the Defined Operational Deliverables and Defined Integrations. (Amount of workshops, ways of technical testing, user testing etc.) |  |
| Supplier's Response |  |  |  |

Suggested timeline from suppliers will be discussed and potentially changed as part of the negotiation phase. Scope of work will be subject to change and negotiation.

### 3.1.5 Capacity and competence

Supplier is asked to state capacity and competence for delivering this solution within the time of delivery that is estimated in Appendix 3.

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 3.1.5.1 | Must | The Supplier must verify that they have the needed capacity to fulfil the required deliveries, based on the descriptions and requirements in SSA-L Appendix 1. Confirmation from the CEO, or authorized person on behalf of the company, must confirm in writing that the Supplier will have the capacity to implement the requested solution described in Appendix 1 in accordance with Appendix 3. |  |
| Supplier's Response |  |  |  |
| 3.1.5.2 | Must | The Supplier must demonstrate the necessary competence to carry out the delivery, in accordance with the descriptions and requirements in SSA-L Appendix 1-9. The Supplier documents this by attaching ONE combined PDF-file with CVs for core personnel who will deliver and manage the delivery of the solution. It is expected that at least CVs for personnel who will occupy the following roles are provided: - Integration developer - Project manager - System expert CV’s are limited to personnel assumed to be directly involved in the delivery to NRK. The CVs must contain (max 3 A4-pages pr. CV): - Name - Role - Affiliation with the Supplier (employee/subcontractor) - Relevant education - Relevant experience from deliveries of similar solutions, including customer information, year of delivery. |  |
| Supplier's Response |  |  |  |

# Appendix 4: Service level with standardised damages

The Supplier shall complete the table below based on its standardized Service Level Agreements (SLAs). All times shall be specified in minutes and shall apply on a 24/7/365 basis, unless otherwise explicitly stated.

To ensure a uniform and equitable evaluation, the Supplier is requested to provide a description of SLA offerings in tables below. This should include the following topics:

- Uptime.

- Downtime definition.

- Maintenance notification and impact for customers.

- Severity level definition

- Response time

- Resolution time

- Support operating hours

- Compensation if breaches

- Measurement and reporting of SLA

- Recovery;

  - RTO (Restoration after total failure)

  - RPO (data loss)

- Root cause analysis transparency

  - Elaborate on how you share RCA with customers.

Supplier response:

| Category | Notification Time (Time elapsed until NRK receives notification) | Response Time (Time elapsed until active mitigation/analysis is initiated) |
| --- | --- | --- |
| P1 - Critical | <Supplier Insert Time> | <Supplier Insert Time> |
| P2 - High | <Supplier Insert Time> | <Supplier Insert Time> |
| P3 - Moderate | <Supplier Insert Time> | <Supplier Insert Time> |
| P4 - Low | <Supplier Insert Time> | <Supplier Insert Time> |

Supplier response

<Supplier response>

## 4.1 Service levels and damages

| # | Type of req | Requirement | Compliance Y/P/N |
| --- | --- | --- | --- |
| 4.1.1 | Should | NRK requires the WPS system to be continuously available to ensure uninterrupted production. Where the Supplier provides a proprietary platform: State the guaranteed uptime for the platform and dashboard, including the calculation formula. Briefly describe how the redundancy mechanisms operate. Where the Supplier utilizes NRK's platform: Describe the notification protocols and escalation procedures if the Supplier detects that NRK's platform is unavailable or ceases to receive logs. |  |
| Supplier's Response |  | <Supplier response> |  |
| 4.1.2 | Should | NRK requires predictability in day-to-day collaboration extending beyond acute incidents. This encompasses requests for advisory services, minor configuration changes, or the extraction of data and reports. The Supplier shall describe: Guaranteed Response Time: Specify the maximum time elapsed until the initial response or acknowledgement of a request. Process and Workflow: Detail how such requests are submitted, tracked, and managed through to resolution. |  |
| Supplier's Response |  | <Supplier response> |  |
| 4.1.3 | Should | The Supplier shall describe its standard model for price reductions (Service Credits) in the event that the guaranteed service levels (SLAs) are not met. The description must include: Calculation of Service Credits: Detail how the service credit is calculated per incident or on a monthly basis in the event of breaches related to response times or system availability. Automatic Application: Confirm whether the service credits are automatically credited on the subsequent invoice, or if NRK must formally submit a claim based on the monthly performance reports. Maximum Cap: Specify any upper limits or caps on the total cumulative monthly service credits. |  |
| Supplier's Response |  | <Supplier response> |  |
| 4.1.4 | Should | SLA history Please provide the actual achieved uptime for the offered service over the past 24 months, broken down by month. Describe how many times over the past 24 months your system has breached the guaranteed SLA thresholds for uptime or response times for existing customers on this platform? |  |
| Supplier's Response |  | <Supplier response> |  |

# Appendix 5: Administrative provisions

## 5.1 The representatives of the parties (Clause 1.5)

For NRK

Name:

Position:

Telephone: +47 –

E-mail:

For the Supplier

Name:

Position:

Telephone:

E-mail:

Changes of authorised representatives of the Supplier shall be notified one month in advance in writing. It is the Supplier’s duty to ensure proper transfer of competence to the new representative(s).

Please note that the authorised representatives should be agreed by the parties on signing the contract and are not required to be stated before this time.

## 5.2 Duration (Clause 5.1)

The Agreement is ongoing and shall enter into force on the date of signature, with a fixed duration of 4 years from the date of signature.

The Customer has a unilateral option to extend the Agreement by 1 year at a time on unaltered terms.

The extension shall apply automatically for 1 year at a time, unless NRK gives the Contractor written notice of termination no later than 6 months prior to the expiry of the current agreement period.

The Supplier may terminate the Agreement by giving twelve 12 months' notice prior to the renewal date.

## 5.3 Cancellation (Clause 5.2)

The Customer may cancel the service contracted under this Agreement in full or in part by giving three (3) months’ written notice. Other terms of notice of cancellation may be agreed in Appendix 5.

Unless another cancellation fee has been agreed in Appendix 6, on cancellation the Customer will pay:

- The amount due to the Supplier in respect of the part of the Service that has already been completed.

- The Supplier’s necessary and documented direct costs in relation to the reassignment of personnel.

- Other documented direct costs incurred by the Supplier as the result of the cancellation, including disbursements and costs that have been incurred by the Supplier prior to its receipt of the notice of cancellation, and which the Supplier is unable to make use of for other purposes.

- A cancellation fee equivalent to 10% of the consideration invoiced to the Customer during the last three months prior to cancellation. To be replaced with:

A cancellation fee equivalent to 10 % of the License-fee invoiced to the Customer during the last three months prior to cancellation.

In the event of partial cancellation, the cancellation fee is calculated on the basis of pro rata settlement of the fee, as agreed in (d).

To the extent that it has been agreed that the service is scalable with respect to payment on any reduction of the number of users, functionality or similar, the Customer's use of this agreed scalability will not be considered to be cancellation.

## 5.4 Pay and working conditions (Clause 11.2)

If there is a general collective wage agreement or nationwide collective wage agreement for the relevant industry, the subsequent sections will apply.

In respect of areas covered by the Regulations relating to Generalised Collective Wage Agreements, the Supplier shall ensure that its own and any subcontractors' employees who contribute directly to the performance of the Supplier’s obligations under the Agreement do not receive pay or have working conditions that are inferior to those stipulated in the Regulations relating to Generalised Collective Wage Agreements. In areas not covered by generalised collective wage agreements, the Supplier shall ensure that the same employees do not receive pay or have working conditions that are inferior to those stipulated in any applicable nationwide collective wage agreements relating to the relevant trade. This applies to work performed in Norway

All agreements that are entered into by the Supplier and that involve the performance of work that contributes directly to the performance of the Supplier’s obligations under the Agreement shall include corresponding terms and conditions.

If the Supplier fails to comply with this obligation, the Customer will be entitled to withhold part of the contract price corresponding to approximately two times (twice) the saving made by the Supplier, until it has been documented that the issue has been remedied.

Fulfilment of the Supplier's obligations as mentioned above shall be documented in Appendix 5 by means of either a self-declaration or a third-party declaration showing conformity between the relevant collective wage agreement and the actual pay and working conditions relating to compliance with the Supplier's and any subcontractors' obligations.

The Supplier shall, at the request of the Customer, disclose documentation relating to the pay and working conditions which are applied. Each of the Customer and the Supplier may request that the information be submitted to an independent third party appointed by the Customer to examine whether the requirements of this provision have been complied with. The Supplier may require the third party to sign a declaration that the information will not be used for any purpose other than to ensure fulfilment of the Supplier's obligations pursuant to this provision. The documentation obligation also applies to subcontractors.

<Supplier’s response is to be added on the next page>

Self-declaration on wages and working conditions

Legal authority: Law No. 69 on Public Procurement dated July 16th 1999 § 11a, cf Regulation on wages and working conditions dated February 8th 2008.

This self-declaration applies for:

| Company |  |
| --- | --- |
| Organization Number |  |
| Address |  |
| Zip/City |  |
| State/Country |  |

We hereby confirm that all employees in our company as well as hires and subcontractors employed in the execution of services and construction work for NRK do not have lower wages or worse working conditions than those according to applicable nationwide standard wage agreements or otherwise normal for the geographic location and occupation in question.

The Contractor and possible subcontractors who directly contribute to the fulfilment of the contract can document the wages and working conditions to employees and hires who contribute the fulfilment of the contract.

We acknowledge that NRKs might initiate necessary sanctions if we or possible subcontractors do not comply with the contract’s article on compliance with the UN ILO conventions for international labor standards. Possible sanctions shall be suitable to influence the Supplier or subcontractor to fulfil the obligations with reference to the referred contract article.

CEO (sign.): ___________________________________________ Place/Date: _________________

# Appendix 6: Total price and pricing provisions

Detailed pricing is set forth in Document 2.2, Appendix 6a (Price Scheme). The prices and rates specified within the Price Scheme shall be governing and binding for the Contract. NRK prefers NOK (Norwegian currency) as the currency for all submitted prices. Any deviation from this preference may result in a calculated risk adjustment being applied to the evaluation price.

The price evaluation is based on a simulated 6-year Total Cost of Ownership (TCO) to provide a complete and transparent financial overview of all costs, enabling a fair evaluation and award of the Agreement.

The Price Scheme is structured to capture all relevant costs associated with the delivery. To ensure full financial transparency, the Supplier is responsible for ensuring that all potential cost components are included. To the extent that certain necessary cost elements or services cannot be accommodated within the predefined lines of the Price Scheme, the Supplier must list and specify these in the designated table for “Any other costs” in Table 9 in “2.2 Appendix 6a - Price scheme - RFP_WPS”. Any cost omitted by the Supplier will be considered included in the overall offer.

The Defined Operational Deliverables and the Defined Integrations are expected to be offered on a fixed-price basis, demonstrating the Supplier’s experience and maturity in delivering these services. The total offered price for all Defined Deliverables, including defined training costs and any other costs, will be weighted at 70% of the total price evaluation. For evaluation purposes, training costs shall be based on 9 on-site training sessions; however, this volume is for evaluation purposes only and does not cap operational requirements. Any necessary integrations defined by the Supplier for the establishment phase (Appendix 3) that are not predefined in the Price Scheme must be added to Table 9 (“Any other costs”).

NRK will utilize active staging and “lab” environments. The Supplier shall provide and support all staging and lab environments at no additional cost to NRK. Fees for implementation and maintenance services shall apply exclusively to the production system actively in use. If the Supplier makes a reservation against this or any other fixed-price requirement, NRK reserves the right to apply a calculated risk adjustment to the evaluation price.

The necessary NRK involvement is included in the Price Scheme structure for visibility, but will be evaluated qualitatively under the award criterion “Quality” (Requirement 3.1.4.2.).

Professional Services are included in the Price Scheme. While these services will constitute an important part of the scope of the Agreement, the exact scope remains uncertain. Consequently, NRK has opted for a weighted score based on blended hourly rates rather than attempting to estimate an unpredictable volume.

The blended rates for Professional Services are weighted at 30% of the price evaluation, and these rates will also apply to any required Exit Services.

| Section - Price scheme | Cost | Score | Weight |
| --- | --- | --- | --- |
| Fixed-price deliverables: - Defined operational deliverables - Defined integrations - Licenses, support and maintenance - Cost of training - Any other costs | X NOK | 0 to 10 | 70% |
| Professional services - blended rates | X NOK | 0 to 10 | 30% |

## 6.1 Documentation and training (Clause 3.4)

For evaluation purposes, training costs shall be based on 9 on-site training sessions; however, this volume is for evaluation purposes only and does not cap operational requirements. NRK may, at its own discretion, order training sessions at the same rate. This rate shall be all-inclusive.

## 6.2 Agreement Invoicing dates and payment terms (Clause 4.2)

### 6.2.1 Payment plan

#### 6.2.1.1 Operational deliverables

Invoicing and payment for operational deliverables and defined integrations shall commence following formal Acceptance, and shall be executed in accordance with the payment schedule stated below.

The payment plan below is proposed by NRK as a basis for discussion with the Supplier.

| Payment plan for Operational Deliverables |  |  |  |
| --- | --- | --- | --- |
| Invoice | Payment milestone | The invoice covers the following deliveries: | Percentage of total fixed price to be invoiced: |
| 1 | After delivery of Operational deliverables 0.3 | Operational deliverables 0.1 - 0.3 | 15% |
| 2 | After delivery of Operational deliverables 0.6 | Operational deliverables 0.4 - 0.6 | 15% |
| 3 | After delivery of Operational deliverables 0.9 | Operational deliverables 0.7 - 0.9 | 15% |
| 4 | After delivery of Operational deliverable 1.0 | Operational deliverable 1.0 | 40% |
| 5 | After delivery of Operational deliverable 1.1 | Operational deliverable 1.1 | 15% |

#### 6.2.1.2 Licenses, support and maintenance

Software license fees shall only accrue and become payable once the system with deployed functions has been successfully deployed and put into active production use. No licensing fees shall be charged during the implementation, configuration, or testing phases. Supplier is to describe how licenses are to be activated and invoiced.

| Supplier’s Payment plan for Licenses |  |  |
| --- | --- | --- |
| ID | License | Milestone ID for activation |
| 6.2.1.2.1 |  |  |
| 6.2.1.2.2 |  |  |
| … |  |  |

In addition to completing Tab 3 in the Price Scheme (Licenses, support and maintenance), the Supplier shall provide a written description of their license price model. This description must include relevant volume and cost drivers, as well as any assumptions made to calculate the 6-year TCO in Tab 3. This information will be used to quality assure and validate the cost estimate during the price evaluation.

| ID | Supplier’s response |
| --- | --- |
| 6.2.1.2 | <Supplier response> |

## 6.3 Price adjustments (Clause 4.5)

The Prices specified in this Agreement shall remain fixed and shall not be subject to alteration prior to January 1, 2028.

With effect from January 1, 2028, and at the beginning of each subsequent calendar year, the Supplier’s prices may be adjusted by an amount equivalent to the increase in the retail price index (the main index) of Statistics Norway for the preceding full calendar year (measured from December to December).

As the index for December is normally not published until after January 1, the price adjustment shall be implemented on the first invoice issued after the official index figures are available, with retroactive effect from January 1, 2028.

# Appendix 7: Changes to the general contractual wording

## 7.2 Interpretation – Ranking (Clause 1.3)

Changes to the general contractual wording must be added here, unless the general contractual wording refers such changes to a different Appendix, cf. clause 1.3 of the Agreement.

Changes may be made to any clause of the Agreement, even where the contractual wording does not explicitly allow for this. The Contractor should, however, be aware that deviations, reservations and changes to the Agreement may result in rejection of the bid.

All changes to the contractual wording must be stated here, so that the text in the general contractual wording remains unchanged. It must be stated, clearly and unequivocally which clause or clauses of the Agreement have been changed and the results of the change.

| Reference to the clause and possibly the section of the Agreement | To be replaced by / To be added |
| --- | --- |
| 2. SSA-L Agreement - 2.2 The Supplier’s responsibility for third-party deliverables (Clause 2.2) | To be replaced by: The Supplier’s responsibility for third-party deliverables (Clause 2.2) To the extent that third-party deliverables are included in the services of the Supplier, a copy of the terms for the Customer's access to and use of third-party deliverables shall be attached in Appendix 9. The terms are binding on the Customer. In Appendix 9, as can be expected from a professional supplier, the Supplier must describe which obligations the terms impose on the Customer and which limitations of liability are reserved by the third party. The Supplier is liable to the Customer for any errors in third-party deliverables arising after the delivery date. This entails that the Customer may invoke any non-fulfilment of agreed quality requirements after the delivery date, including service level requirements, if this is due to an error in third-party deliverables. This also applies to the loss or destruction of data. In the case of errors in third-party deliverables, the Supplier will be obliged to report the error to the third party, explain to the Customer why the scope and/or consequences of such errors could not or should not have been limited, and keep the Customer informed about the status of the error rectification. If the Supplier is responsible for the installation of third party deliverable(s) as part of the Agreement, the Supplier must test that the third party service(s) function after the error rectification, in accordance with any guidelines specified by the Customer in Appendix 1, cf. clause 1.1 of the Agreement. If the functionality agreed in Appendices 1 and 2 is removed from third-party deliverables and the Supplier does not have the ability to prevent this, any such loss of functionality would entitle the Customer to require a price reduction in accordance with clause 9.2.1, third paragraph, and/or cancellation in accordance with clause 9.2.5, second paragraph. The same will apply if the agreed functionality does not exist in the original delivery and this is not corrected within a reasonable time, cf. clause 9.2.1, fourth paragraph, and clause 9.2.5, third paragraph, of the Agreement. |

# Appendix 8: Changes to the service after the establishment of the Agreement

## 8.1 Changes to the service after the establishment of the Agreement (Clause 1.4)

This Appendix is not to be completed before the Agreement is entered into, but it must be attached, even if it is empty so far.

If the Customer and the Supplier have reached agreement on a change agreement (both in relation to the content, any change in consideration and change of time schedule), the change (content, adjusted consideration and adjusted time schedule) must be stated here.

Each amendment must be signed by the authorised representatives of the parties.

The Supplier is responsible for maintaining an ongoing directory of the changes, which constitutes Appendix 8. The Supplier is also responsible for the Customer receiving an updated copy without undue delay. The Customer must maintain its own overview of the change requests it has sent, and the change estimates it has received.

Change directory

| Change number | Description of the change and any adjustment of the consideration and adjustment of the time schedule | Effective date |
| --- | --- | --- |

# Appendix 9: Terms for the Customer’s access to and use of third-party deliverables

## 9.1 The responsibility of the Supplier for third-party deliverables (Clause 2.2)

To the extent that third-party deliverables are included in the services of the Supplier, a copy of the terms for the Customer's access to and use of third-party deliverables shall be attached in Appendix 9. The terms are binding on the Customer.

In Appendix 9, as can be expected from a professional supplier, the Supplier must describe which obligations the terms impose on the Customer and which limitations of liability are reserved by the third party.

Cf. Appendix 7: The Supplier is liable to the Customer for any errors in third-party deliverables arising after the delivery date. This entails that the Customer may invoke any non-fulfilment of agreed quality requirements after the delivery date, including service level requirements, if this is due to an error in third-party deliverables. This also applies to the loss or destruction of data.

In the case of errors in third-party deliverables, the Supplier will be obliged to report the error to the third party, explain to the Customer why the scope and/or consequences of such errors could not or should not have been limited, and keep the Customer informed about the status of the error rectification. If the Supplier is responsible for the installation of third party deliverable(s) as part of the Agreement, the Supplier must test that the third party service(s) function after the error rectification, in accordance with any guidelines specified by the Customer in Appendix 1, cf. clause 1.1 of the Agreement.

If the functionality agreed in Appendices 1 and 2 is removed from third-party deliverables and the Supplier does not have the ability to prevent this, any such loss of functionality would entitle the Customer to require a price reduction in accordance with clause 9.2.1, third paragraph, and/or cancellation in accordance with clause 9.2.5, second paragraph. The same will apply if the agreed functionality does not exist in the original delivery and this is not corrected within a reasonable time, cf. clause 9.2.1, fourth paragraph, and clause 9.2.5, third paragraph, of the Agreement.

| Third party | Description of the service delivered by the third party | Reference to terms which are binding on the Customer (may be a link) |
| --- | --- | --- |

The Supplier must describe which obligations the terms impose on the Customer and which limitations of liability are reserved by the third party.

It is not expected that the WPS would put live studios to air.

Parameter control is currently under review within NRK with regard to its placement within systems.