A.1 Scope of Work: Toilet waste tanks Type 73

- Background and purpose

Reports from the operators and maintainers show cracks formation in wastewater toilet tanks for Type 73. Both for the 800 litre tanks and 400 litre tanks.  The purpose of this delivery is to restore watertight functions by providing new tanks.

Scope of Supply

The Scope of Supply includes all activities necessary to manufacture, test and deliver replacement toilet waste tanks with nominal capacities of 800L and 400L.

The existing design shall be used as reference for the replacement tanks. The solutions similarity shall be explained compared to existing solution, both in design and in terms of maintenance.

Tenderers may propose improvements to the existing waste tank design. Improvements will be evaluated based on their potential to:

Improve cleanability

Increase robustness (e.g. reduced risk of cracking)

Enhance operational performance

All proposed improvements shall be documented and shall not affect compliance with the specified requirements.

The delivery is strictly limited to replacement waste tanks and does not include any surrounding systems, components or modifications outside the tank itself.

The replacement tanks shall be designed and manufactured to be equivalent to the existing tanks currently installed on the vehicles, ensuring compatibility with existing interfaces, mounting arrangements and system connections. The weight shall also be equal or lower to existing solution. Any change in weight must be documented.

The Contractor shall be responsible for delivering complete and compliant waste tanks ready for installation. This includes, but is not limited to, the following:

- Manufacturing and assembly of 10 800L and 5 400L toilet waste tanks

- Replication of functional and interface characteristics of existing tanks

- Procurement of all required materials and components forming part of the tanks

- Quality control and testing prior to delivery

- Delivery of finished tanks to agreed location.

  - The delivery needs to be marked with NT article number provided by Norske tog (NTXXXXXXXXX)

- Preparation and submission of required documentation

The supplied tanks shall be robust, suitable for railway applications, and ensure reliable operation under the same operating and environmental conditions as the existing solution.

Norske tog reserves the right to include optional scope (options) under this contract. This includes, but is not limited to, additional deliveries of 800L and 400L toilet tanks. The Contractor shall provide unit prices, lead times and conditions for such options in the tender. Norske tog is not obligated to exercise any options and may procure additional quantities during the contract period.

Norske tog reserves the right to procure up to 10 units of the 800L tank and 5 units of the 400L tank. The price shall be stated per unit in the pricing schedule, Exhibit B1. The option can be between 1 and max number of toilet waste tanks pr. Litres.

- Specifications

- EVAC 2000 Collection Tank — Technical Data

| Parameter | Value (English) |
| --- | --- |
| Dimensions (mm) | 1,950 × 796 × 530 mm |
| Air volume | Approx. 435 liters |
| Pressure air connection | Formeto Ø 12 × 3/8× |
| Liquid volume (99% sensor) | Approx. 400 liters |
| Plate thickness | 3 mm |
| Maximum vacuum level | 40% |
| Insulation | 50 mm cell foam |
| Weight | 200 kg |
| Inlet connection | Ø 48.3 mm |
| Vent pipe | Ø 76 mm |
| Wash connection | Ø 32 mm |

- EVAC 2000 Transfer Collection Tank — Technical Data

| Parameter | Value (English) |
| --- | --- |
| Dimensions (mm) | 2,871 × 960 × 530 mm |
| Air volume | Approx. 900 liters |
| Pressure air connection | Formeto Ø 12 × 3/8× |
| Liquid volume (99% sensor) | Approx. 800 liters |
| Transfer tank | Approx. 40 liters |
| Plate thickness | 3 mm |
| Maximum vacuum level | 20% |
| Weight | 320 kg |
| Inlet connection | Ø 48.3 mm |
| Wash connection | Ø 48.3 mm |
| Vent pipe | Ø 76 mm |

- See drawings and descriptions for more detailed dimensions and geometry.

- Requirements

  - Material requirements

All materials used in the construction of the waste tanks shall comply with EN 45545 requirements for fire behaviour of materials and components.

The applicable Hazard Level (HL) and requirement sets shall be defined based on the vehicle category. As a minimum, all materials shall meet the requirements relevant for interior and exterior components exposed to railway operating conditions. The hazard level for Type 73 is HL2.

Materials shall be selected to ensure:

- Adequate fire performance (heat release, smoke density and toxicity)

- Resistance to chemical exposure from wastewater and cleaning agents

- Long-term durability under operational conditions

- 4.2 Volume and tolerances

The waste tanks shall have the following nominal capacities:

- 800 litres

- 400 litres

The actual usable volume shall not deviate more than ±5% from the stated nominal capacity, unless otherwise agreed.

The internal geometry shall ensure effective drainage and prevent accumulation of residues during operation.

The design shall minimize dead zones and support complete emptying during service operations.

  - Interfaces and connections

The replacement tanks shall be fully compatible with the existing installation.

All mechanical interfaces, mounting points, and connections shall match the existing design, including:

- Inlet and outlet connections

- Venting interfaces

- Mounting brackets and fixation points

The Contractor shall ensure that no modification to the vehicle or surrounding systems is required for installation.

All connections shall ensure leak-tight performance under all operating conditions.

  - Pressure and vacuum requirements

The waste tanks shall be designed to withstand the operational pressures and vacuum conditions experienced during:

- Filling

- Operation

- Emptying (vacuum discharge systems)

The tanks shall:

- Withstand internal overpressure without permanent deformation or leakage

- Withstand vacuum conditions without structural collapse

The design shall ensure structural integrity and functionality throughout the operational life of the tank.

  - Drainage and serviceability

The tanks shall be designed to ensure efficient and hygienic emptying.

The design shall:

- Allow complete drainage during emptying operations

- Avoid internal geometry that traps waste or residue

- Be compatible with existing waste discharge systems

Service and maintenance considerations shall include:

- Easy access to inspection points (if applicable)

- Cleaning-friendly internal surfaces

- Robust design to minimize maintenance needs

The existing frost protection arrangement shall be maintained and replicated.

This includes:

- Heating mats installed under the inner tank

- Heating mats installed on the transfer tank

The heating system shall:

Provide sufficient frost protection under normal operating conditions

Be equivalent in function and performance to the existing solution

The insulation and heating arrangement shall be equivalent to the existing solution.

- Quality and inspection

- All work shall be performed in accordance with good workmanship practice.

- Completed work shall be subject to visual inspection.

- Any deviations from the agreed scope shall be documented and reported to Norske tog.

- Standards and technical regulations

- The work shall be carried out in accordance with relevant parts of applicable standards and requirements, including but not limited to:

- EN 12663

- UIC 566

- EN 15227

- EN 15085

- EN 50125

- EN 45545

- EN 16922

- DIN EN 16362

- TSI Loc & Pas

- Documentation

The Contractor shall deliver complete and structured documentation for the manufactured waste tank. The documentation shall follow the document plan.

Norske tog will provide drawings and design documentation as basis for the delivery. The Contractor shall supplement this with all necessary documentation related to manufacturing, calculations, materials, quality and verification.

As a minimum, the following documentation shall be provided:

- Documentation which is needed for maintenance, repair, overhaul and onboard operation.

- Manufacturing drawings and/or as-built drawings reflecting the delivered tanks

- Confirmation of compliance with provided design documentation

- Material certificates and material datasheets for all relevant components

- Description of manufacturing processes, with calculations

- Test and inspection documentation, including quality control reports

- Documentation of performed tests and verification activities

- Any deviations from provided design, subject to prior approval by Norske tog

All documentation shall be delivered in English.

Text documents shall be delivered in editable Word format and searchable PDF. Drawings shall be delivered in editable format (e.g. DWG) and PDF.

All documentation shall be complete, consistent, and reflect the delivered product.

- Delivery plan

The tenderer shall provide a delivery plan with the shortest feasible delivery time. The plan shall describe the key activities and milestones, explain how the tenderer intends to achieve and maintain the proposed schedule, and include relevant risk factors.

The critical path shall be clearly identified, and the associated risks shall be described.

- Attachments

- See attached files for drawings and descriptions.

A.2 Requirements

A.3 Drawings and descriptions

A.4 Document plan