[Saksnummer i DocuLive/Kontraktsnummer]

CNC Machine

Part 2 – Annex B

Requirement specification

Introduction

This Annex provides a description of all of the Purchaser's requirements to the performance. The requirements will apply to all deliveries under the agreement.

Requirement table

The Purchaser's requirements

The Purchaser has listed their requirements in the following manner in the tables below:

No.:	Unique serial number for the requirement

Description:	Description of the requirement

Documentation type:	Description of the documentation verifying the fulfilment of the requirement

Requirement type:	See the table below

| Absolute requirement: A | The requirement must be met. The requirement constitutes a minimum requirement which is not subject to relative assessment. |
| --- | --- |
| Absolute requirement: A* | The requirement must be met. The asterix indicates that the requirement is a minimum requirement, exceeding the requirement will be subject to evaluation under the relevant award criteria. |
| Requested requirement: B | The requirement should be met, but is not required. The requirement is subject to evaluation under the relevant award criteria. |

Requirement to the item

Requirement

| Purchaser's requirements |  |  |  | Contractor's response |  |
| --- | --- | --- | --- | --- | --- |
| No. | Description | Documentation type | Requirement type | Offered (Yes/No/ Partially) | Solution |
|  | Safety and Certification: The entire installation (including the bar feeder and chip conveyor as a unified system) must satisfy the Machinery Directive 2006/42/EC and be CE marked. All documentation must be delivered in Norwegian, a Scandinavian language, or English. |  | A |  |  |
|  | Construction: The machine base must be made of heavy-duty cast iron (cast construction) to ensure maximum vibration dampening, stability, and superior repeatability during heavy-duty machining. The machine's net weight (excluding bar feeder and chip conveyor) must be at least 8,500 kg to ensure sufficient mass, rigidity, and vibration dampening during continuous operation. Designs based on welded steel plates or bolted frames are not accepted. |  | A |  |  |
|  | Configuration: Turning center with integrated live/driven tooling (M), Y-axis (Y), and counter-spindle/sub-spindle (S) for complete machining in a single setup. The machine must feature a full-fledged configuration with X, Y, and Z axis values, as well as an independent, continuous contouring C1-axis (main spindle) and C2-axis (sub-spindle). |  | A |  |  |
|  | C-Axis Resolution: Continuous contouring C-axis on both the main and sub-spindle (C1 and C2) with a positioning resolution of at least 0.0001 degrees. Both the C1 and C2 axes must support full, synchronized interpolation with the linear axes and be equipped with a mechanical/hydraulic brake for secure locking during milling operations. |  | A |  |  |
|  | Capacity: The maximum turning length between spindles/chucks must be at least 1250 mm. The maximum machining diameter must be at least 400 mm. Furthermore, the spindle bore (spindle through-hole) must accommodate bar stock with a diameter of at least Ø102 mm. |  | A |  |  |
|  | Workholding & Spindle Drive: Main spindle equipped with a hydraulic chuck of at least 12 inches. Sub-spindle equipped with a hydraulic chuck of at least 8 inches. Both the main and sub-spindle must feature a built-in motor spindle or direct-drive configuration to ensure maximum torque and balance. Belt-driven spindles are not accepted on either spindle. The main spindle must have an internal bearing diameter of at least 140 mm. |  | A |  |  |
|  | - Spindle Support (Weight Capacity): The maximum allowable weight (including chuck/workholding and workpiece) must be at least 500 kg for the main spindle (Spindle 1) and at least 150 kg for the sub-spindle (Spindle 2) to ensure stable machining of heavy workpieces. |  | A |  |  |
|  | Feedrates, Guideways, and Precision Rapid Traverse / Feed rate X-axis: At least 30,000 mm/min Z-axis: At least 30,000 mm/min Y-axis: At least 15,000 mm/min All linear axes (X, Y, and Z) must utilize heavy-duty, preloaded roller-type linear guideways of the highest precision class. The ballscrews must be double-nut, preloaded, and directly coupled to the servomotors without intermediate gears or belts. |  | A |  |  |
|  | Thermal Auto-Correction: The machine must be equipped with an active thermal compensation system. The system must automatically correct the axes in real time based on temperature changes in the workshop environment and the machine, ensuring that tolerances are maintained from the very first part (without requiring a machine warm-up cycle). To counteract thermal expansion during continuous operation, the machine must feature active temperature control/cooling. |  | A |  |  |
|  | Positioning and Repeatability Accuracy must fulfil ISO 230-2 The specified accuracy requirements must be documentable for the specific machine upon delivery in the form of an official laser measurement protocol (e.g., ballbar test and laser interferometer test) performed at the factory prior to shipment. |  | A |  |  |
|  | Tooling and Machining Capabilities Rotary Tool Capacity: The turret and driven tools must have sufficient torque and power to perform continuous drilling and milling in steel with a tool diameter of at least Ø20 mm. The turret must be servo-driven with hydraulic locking via a high-precision curvic coupling (or equivalent three-piece tooth coupling). Tool indexing time (station to adjacent station) must be a maximum of 0.3 seconds. The time for a full rotation of the turret (full turn) cannot exceed 1.0 second. |  | A |  |  |
|  | Threading Capacity: The system must support rigid tapping up to at least M24 on the rotary tools, and must support variable thread starting positions (re-cutting/synchronization). |  | A |  |  |
|  | Ancillary Systems (Coolant, Chips, and Handling) Coolant System: Integrated coolant tank with a capacity of at least 240 liters and a high-pressure system delivering at least 15 bar. The coolant system must include an automatic filtration system (e.g., cyclone or paper band filter with a filtration degree of 50 µm or finer) to protect internal channels against microparticles. |  | A |  |  |
|  | Chip Management: A complete chip conveyor (hinged belt type) and an associated chip bin must be included. |  | A |  |  |
|  | Parts Catcher: Integrated, automatic parts catcher with a collector for unmanned operation. |  | A |  |  |
|  | Control System, Processing, and Safety Control System and Interface Architecture: The CNC control system must feature native, dialog-based programming (Conversational Programming) as standard, allowing the operator to generate complete toolpaths directly at the machine interface without requiring external CAM software. The control system's processor must have a Block Processing Time (BPT) of 0.5 milliseconds or less to ensure seamless, uninterrupted axis interpolation during high-speed execution of complex profiles. |  | A |  |  |
|  | Native 3D CAD Model Import & Feature Recognition: The control system must support direct, native import of 3D CAD models (specifically STEP and IGES formats) directly via the operator panel touchscreen. The interface must feature automatic feature recognition, allowing the operator to select 3D geometries, contours, holes, and thread structures directly from the solid model to instantly generate error-free toolpaths on the machine floor. |  | A |  |  |
|  | Unit Switching: Seamless switching between metric units (mm) and imperial units (inches) with a single keystroke on the control, without loss of coordinates or the need for a system reboot. |  | A |  |  |
|  | Real-Time 3D Crash Monitoring (Virtual Twin): The machine must be equipped with an intelligent, dynamic, real-time 3D collision avoidance system embedded directly into the control architecture. The system must continuously calculate and simulate the spatial relationship of the entire machine geometry, including the main spindle, sub-spindle, turret, live tool holders, custom workholding/chucks, and the actual evolving raw material (workpiece blank). The system must automatically trigger an emergency interference stop before a physical impact occurs. This safety functionality must be fully active and operational in both automatic execution mode and manual override/setup modes (JOG/MPG mode) |  | A |  |  |
|  | Engraving: Software option for text/serial number engraving on flat and cylindrical surfaces. |  | A |  |  |
|  | Bar Feeder Interface & Options: The machine must be fully equipped with a bar feeder interface. To accommodate uncertain future production demands, the supplier must offer two alternative bar feeding solutions that physically fit within the available workshop space of 5 x 11 meters (including required service clearance): Option A (Short Bar Loader): Integrated magazine capable of handling round material with a diameter up to 100 mm and bar lengths up to a maximum of 1000 mm. Option B (Full-Length Bar Feeder): Integrated automatic magazine capable of handling round material with a diameter of up to 100 mm and bar lengths up to a minimum of 3000 mm. |  | A |  |  |

Requirements to service/maintenance

Requirement

| Purchaser's requirements |  |  |  | Contractor's response |  |
| --- | --- | --- | --- | --- | --- |
| No. | Description | Documentation type | Requirement type | Offered (Yes/No/ Partially) | Solution |
|  | Service/maintenance Supplier must be able to do preventive maintenance according to machine specification, which must include geometrical calibration (via laser interferometer and ballbar testing) to ensure continuous compliance with aviation tolerances. |  | A |  |  |
|  | Response Time & Technical Support: Guaranteed remote technical support within a maximum of 2 hours during standard working hours. For critical machine failures (Machine Down), an authorized on-site field service technician must arrive within 3 to 5 days. |  | A |  |  |
|  | The Supplier must guarantee availability of critical mechanical, electrical, and control components throughout the machine's service life. |  | A |  |  |
|  | The supplier must maintain a certified field service organization with technicians based in Scandinavia, fluent in a Scandinavian language or English. |  | A |  |  |

Requirements to training

Requirements

| Purchaser's requirements |  |  |  | Contractor's response |  |
| --- | --- | --- | --- | --- | --- |
| No. | Description | Documentation type | Requirement type | Offered (Yes/No/ Partially) | Solution |
|  | At delivery: Comprehensive, on-site training for operators focusing on dual-spindle operations, multi-axis interpolation, and direct shop-floor 3D conversational programming without external CAM. |  | A |  |  |
|  | At delivery: Maintenance & Troubleshooting Training: On-site training for internal maintenance staff covering basic mechanical troubleshooting, electrical diagnostics, and daily/weekly preventive care procedures. |  | A |  |  |