Annex 1.1 Specification for snow removal equipment for use in railway infrastructure

Bane NOR is responsible for snow clearance on railway tracks and therefore requires new and modern snow removal equipment. This framework agreement is intended to facilitate the procurement of all necessary equipment for efficient snow removal on the railway network. The agreement also includes necessary support during the post‑commissioning period, as well as any modifications and adaptations of the equipment.

Bane NOR already possesses a substantial portfolio of snow removal equipment, and the new framework agreement is based on the premise that any new equipment shall provide functionality and performance broadly equivalent to existing solutions. If the supplier has previously delivered equipment to Bane NOR, such equipment shall also be included within the scope of the agreement with regard to support, modifications, and adaptations.

The current equipment is used as attachments on several types of vehicles. Deliveries under this agreement shall therefore be adapted to ensure compatibility with existing vehicles, including mechanical, electrical, and hydraulic interfaces.

All equipment shall be CE-marked (EU) and comply with the requirements of the Working Environment Act, the Machinery Directive, and applicable railway legislation, including associated regulations.

This specification sets out overall requirements, equipment lists, and product descriptions. The supplier shall confirm whether these requirements can be met and provide a description of the proposed solutions.

Requirements for snow equipment

Bane NOR currently uses rotary brushes, track cleaners, snow blowers, and wedge ploughs for snow removal along the railway infrastructure (a descriptive illustration is provided in Annex 1.2). Snow clearance also includes the removal of ice buildup in and around the track.

- Snow broom

- Track cleaner

- Snow blower

- Snow plow

- Hydraulic power units

- The supplier’s proposal for alternative methods and equipment for snow removal

- Attachment to vehicle: Standard Volvo quick coupling (reverse of that used on the wheel loader).

- Color: Yellow, color code RAL 1003

The snow equipment must be suitable for the vehicle types:

- LT11

- LTR-17

- LT18

- See detailed descriptions of vehicle types below and in the annexes.

Environmental requirements for snow equipment

The minimum requirements apply to equipment for all vehicles:

No chemical products may be used in the surface treatment (including pre‑treatment, primer, and topcoat) that contain substances listed on the Norwegian Priority List or ECHA’s Candidate List (SVHC), as these lists exist at the time of the tender.

Surface treatments shall be designed and suitable in such a way that the risk of pollution from fading, wear, or other particle release into the external environment is minimized during normal use of the equipment.

The equipment shall be designed so that main components and wear parts can be disassembled and replaced, and so that materials can be recycled to the greatest extent possible at the end of their service life, where this is technically feasible.

Where the equipment uses hydraulic oil, biodegradable hydraulic oil suitable for the area of ​​application shall be used. If there are documented technical conditions that indicate that such oil cannot be used, this must be specifically justified.

Plastic particles released from the brush may remain in the groove and potentially affect the external environment. The brush shall be designed and manufactured to minimize this risk.

Innovation

The framework agreements are long-term agreements. Bane NOR SF anticipates that significant     developments in control systems, autonomy, data acquisition, and technical solutions will take place during the framework agreement period. Collectively, these innovations will contribute to a more reliable, safer, and cost-effective railway, better equipped to meet the challenges of a demanding winter climate. Bane NOR seeks to actively support this process through the testing, evaluation, and development of such solutions.

The Supplier shall provide a description of its process for identifying opportunities for innovation and for further developing innovative solutions.

General requirements for equipment deliveries

- Delivery location and terms: Equipment shall be delivered to a location to be agreed, DDP.

- The tender shall include drawings and other relevant information.

- STEP files shall be included with the tender where available.

- All equipment shall be CE-marked.

- All documentation shall be in Norwegian and delivered in digital format. Digital text information shall be provided in formats that can be edited using standard Microsoft Office applications, as well as in PDF format. Bane NOR will revert regarding the required file formats for drawings. In addition to documentation for the delivered equipment, instructions for preventive and routine maintenance shall be provided to ensure the economic service life of the equipment.

Vehicle limitations (regarding the weight of snow removal equipment)

LT11: The weight limitations for attached equipment on the LT11 are 1,500 kg with a moment arm of 1.6 m from the buffer plate to the center of gravity of the snow equipment on the cabin end, and 1,900 kg with a moment arm of 1.35 m from the buffer plate to the center of gravity of the snow equipment on the crane end.  It is advantageous for the weight of the equipment to be as low as possible, provided that this does not compromise its strength.

LTR-17: The weight limitations for attached equipment on the LTR17 are 2,500 kg at a distance of 1.2 m from the buffer plate on the cabin end, and 2,225 kg on the crane end. It is advantageous for the weight of the equipment to be as low as possible, provided that this does not compromise its strength.

LT18: The weight limitations for attached equipment on the LT18 are 5,000 kg at a distance of 0.956 m from the center of the mounting bracket on the cabin end, and 5,000 kg on the crane end.  It is advantageous for the weight of the equipment to be as low as possible, provided that this does not compromise its strength.

Additional constraints are defined in Profile NO1 (see Annex 1.2). These profile limitations apply to all equipment when mounted on the vehicle.

Supplier’s response for each equipment type

Rotating broom

The units shall be easy to maintain, including the replacement of wear parts. Describe solution:

Please state the estimated delivery time calculated from the call‑off date

Please provide guaranteed delivery time for spare parts/service parts.

Warranties: Please specify the warranties for the supplied components. The warranty period is 36 months from approved delivery.

Easy to install and remove from the vehicle. Please describe.

Easy to store/put away and also easy to transport. Please describe.

The equipment must be of a type that does not damage railway infrastructure, such as cables or similar components. Please describe.

A spare parts list shall be provided as part of the delivery. Please specify how this requirement will be fulfilled:

Documentation shall be provided for:

- Installation

- Technical maintenance

Please specify how the requirement will be fulfilled:

Weights shall be verified through a weighing report and accompanied by drawings indicating the center of gravity for each item of equipment.

Please specify how the requirement will be fulfilled:

Track cleaner

The units shall be easy to maintain, including the replacement of wear parts. Describe solution:

Please state the estimated delivery time calculated from the call‑off date.

Please provide guaranteed delivery time for spare parts/service parts.

Warranties: Please specify the warranties for the supplied components. The warranty period is 36 months from approved delivery.

Easy to install and remove from the vehicle. Please describe.

Easy to store/put away and also easy to transport. Please describe.

The equipment must be of a type that does not damage railway infrastructure, such as cables or similar components. Please describe.

A spare parts list shall be provided as part of the delivery. Please specify how this requirement will be fulfilled:

Documentation shall be provided for:

- Installation

- Technical maintenance

Please specify how the requirement will be fulfilled:

Weights shall be verified through a weighing report and accompanied by drawings indicating the center of gravity for each item of equipment.

Please specify how the requirement will be fulfilled:

Snowblower

The units shall be easy to maintain, including the replacement of wear parts. Describe solution:

Please state the estimated delivery time calculated from the call‑off date.

Please provide guaranteed delivery time for spare parts/service parts.

Warranties: Please specify the warranties for the supplied components. The warranty period is 36 months from approved delivery.

Easy to install and remove from the vehicle. Please describe.

Easy to store/put away and also easy to transport. Please describe.

The equipment must be of a type that does not damage railway infrastructure, such as cables or similar components. Please describe.

A spare parts list shall be provided as part of the delivery. Please specify how this requirement will be fulfilled:

Documentation shall be provided for:

- Installation

- Technical maintenance

Please specify how the requirement will be fulfilled:

Weights shall be verified through a weighing report and accompanied by drawings indicating the center of gravity for each item of equipment.

Please specify how the requirement will be fulfilled:

Snow plow

The units shall be easy to maintain, including the replacement of wear parts. Describe solution:

Please state the estimated delivery time calculated from the call‑off date.

Please provide guaranteed delivery time for spare parts/service parts.

Warranties: Please specify the warranties for the supplied components. The warranty period is 36 months from approved delivery.

Easy to install and remove from the vehicle. Please describe.

Easy to store/put away and also easy to transport. Please describe.

The equipment must be of a type that does not damage railway infrastructure, such as cables or similar components. Please describe.

A spare parts list shall be provided as part of the delivery. Please specify how this requirement will be fulfilled:

Documentation shall be provided for:

- Installation

- Technical maintenance

Please specify how the requirement will be fulfilled:

Weights shall be verified through a weighing report and accompanied by drawings indicating the center of gravity for each item of equipment.

Please specify how the requirement will be fulfilled:

Hydraulic Power Units

Hydraulic power units are used for older machines that do not have sufficient onboard hydraulic capacity to operate the relevant snow-clearing equipment. The units shall function as a separate supply system for the equipment described in this request. They are typically mounted on a dedicated loading platform on older vehicles or wagons. Further details will be specified for each individual call-off.

The units shall be easy to maintain, including the replacement of wear parts. Describe solution:

Please state the estimated delivery time calculated from the call‑off date.

Please provide guaranteed delivery time for spare parts/service parts.

Warranties: Please specify the warranties for the supplied components. The warranty period is 36 months from approved delivery.

Easy to install and remove from the vehicle. Please describe.

Easy to store/put away and also easy to transport. Please describe.

The equipment must be of a type that does not damage railway infrastructure, such as cables or similar components. Please describe.

A spare parts list shall be provided as part of the delivery. Please specify how this requirement will be fulfilled:

Documentation shall be provided for:

- Installation

- Technical maintenance

Please specify how the requirement will be fulfilled:

Weights shall be verified through a weighing report and accompanied by drawings indicating the center of gravity for each item of equipment.

Please specify how the requirement will be fulfilled:

S-Lift

S-Lift frame for lifting/lowering and lateral adjustment of attachments with HK mounting, see Annex 1.2. The S-Lift is used to some extent for site clearing and at low speeds where profile extension is required. Further details will be specified for each individual call-off.

The units shall be easy to maintain, including the replacement of wear parts. Describe solution:

Please state the estimated delivery time calculated from the call‑off date.

Please provide guaranteed delivery time for spare parts/service parts.

Warranties: Please specify the warranties for the supplied components. The warranty period is 36 months from approved delivery.

Easy to install and remove from the vehicle. Please describe.

Easy to store/put away and also easy to transport. Please describe.

The equipment must be of a type that does not damage railway infrastructure, such as cables or similar components. Please describe.

A spare parts list shall be provided as part of the delivery. Please specify how this requirement will be fulfilled:

Documentation shall be provided for:

- Installation

- Technical maintenance

Please specify how the requirement will be fulfilled:

Weights shall be verified through a weighing report and accompanied by drawings indicating the center of gravity for each item of equipment.

Please specify how the requirement will be fulfilled:

## Ice cutter for installation in S-Lift

The units shall be easy to maintain, including the replacement of wear parts. Describe solution:

Please state the estimated delivery time calculated from the call‑off date.

Please provide guaranteed delivery time for spare parts/service parts.

Warranties: Please specify the warranties for the supplied components. The warranty period is 36 months from approved delivery.

Easy to install and remove from the vehicle. Please describe.

Easy to store/put away and also easy to transport. Please describe.

The equipment must be of a type that does not damage railway infrastructure, such as cables or similar components. Please describe.

D A spare parts list shall be provided as part of the delivery. Please specify how this requirement will be fulfilled:

Documentation shall be provided for:

- Installation

- Technical maintenance

Please specify how the requirement will be fulfilled:

Weights shall be verified through a weighing report and accompanied by drawings indicating the center of gravity for each item of equipment.

Please specify how the requirement will be fulfilled:

Broom blower

The units shall be easy to maintain, including the replacement of wear parts. Describe solution:

Please state the estimated delivery time calculated from the call‑off date.

Please provide guaranteed delivery time for spare parts/service parts.

Warranties: Please specify the warranties for the supplied components. The warranty period is 36 months from approved delivery.

Easy to install and remove from the vehicle. Please describe.

Easy to store/put away and also easy to transport. Please describe.

The equipment must be of a type that does not damage railway infrastructure, such as cables or similar components. Please describe.

D A spare parts list shall be provided as part of the delivery. Please specify how this requirement will be fulfilled:

Documentation shall be provided for:

- Installation

- Technical maintenance

Please specify how the requirement will be fulfilled:

Weights shall be verified through a weighing report and accompanied by drawings indicating the center of gravity for each item of equipment.

Please specify how the requirement will be fulfilled:

Information to the customer following each call-off.

The Supplier shall, as early as possible, inform the Client of which information, data, and decisions are required to carry out the delivery, as well as when such information must be provided to ensure the agreed progress. The Supplier shall also notify the Client of any changes to the delivery, including the need for additional or supplementary investigations.

Components and spare parts

During the ordinary service life of the equipment, components and parts shall be obtained from the Supplier. No later than at the time of delivering the equipment, the Supplier shall deliver complete spare parts lists in Norwegian and/or English, including complete parts estimates.

For any components supplied by subcontractors, the Supplier shall ensure that complete type designations and makes are delivered no later than at the time of delivery of the equipment.

The Supplier shall be obliged to supply spare parts of equal quality and functionality for the Delivery for a minimum period of ten years, calculated from the date of the final delivery. When phasing out certain components, the Supplier shall notify the Contracting Authority as soon as it becomes aware of such plans.

The Supplier shall be obliged to impose corresponding provisions in agreements with subcontractors.

Maintenance services

Bane NOR generally performs routine maintenance on its own snow-clearing equipment. However, Bane NOR shall also have the option to procure maintenance services from the selected equipment supplier, and the tenderer is therefore requested to provide pricing for such services in Annex 5.1 Price Matrix.

In the performance of warranty work, major repairs, or any modifications to equipment, Bane NOR may—by special agreement—make its own workshop facilities available to the Supplier. Bane NOR reserves the right to require compensation for such use of its workshops

Routine maintenance of equipment, including preparation for a new snow-clearing season, may be carried out at a workshop designated by Bane NOR. For this type of use of Bane NOR’s workshops, no separate compensation will be required from the Supplier..

Attachment

Annex 1.2