Appendix 1 – Requirement specification with the Supplier's reply

Portable Ballistic Guard Booths (Vaktboder)

Case number: 26/00214

Introduction and structure of this document

This Appendix 1 specifies the Customer's requirements for portable ballistic guard booths (Norwegian: vaktboder) to be delivered under a 4-year framework agreement between Politiets fellestjenester (the Customer) and the selected Supplier. The end user under the framework is Oslo politidistrikt (OPD), as well as other police districts that may issue call-offs under the agreement.

The requirements are presented in tables and classified as follows:

- M (Mandatory minimum requirement): Absolute requirement. Failure to fulfil one or more M-requirements shall lead to rejection of the tender.

- E (Evaluated requirement): Requirements that will be evaluated under the award criterion "Quality" and/or "Environment". The Supplier shall describe its solution and enclose documentation.

Supplier's reply: The Supplier shall fill in the right-hand column of every row. For M-requirements the Supplier shall confirm compliance ("Yes/No") and, where documentation is requested, attach it or reference its location in the tender. For E-requirements the Supplier shall describe its proposed solution and refer to enclosed documentation. Unanswered rows will be interpreted as non-compliance.

Reservations: Any reservations against requirements in this Appendix shall be stated in Appendix 3 – Reservations. Reservations in this Appendix will not be accepted. Significant reservations will lead to rejection of the tender, cf. the Rules of competition.

Background

Oslo politidistrikt (OPD) is responsible for the protection of foreign embassies and certain other sensitive locations in Oslo. Following events in 2025, including a hand-grenade incident at Bislett and a generally heightened threat picture, OPD has assessed that the existing guard booths (placed amongst others at the Israeli and Iranian embassies) no longer provide an adequate level of protection for police personnel on duty.

The Customer therefore needs portable guard booths with a substantially higher protection level than the present solutions, in particular against firearms and the splinter / blast effects from hand grenades. The booths shall also be relocatable, suited to Norwegian climate (cold, snow, rain) and provide an acceptable working environment for the personnel year-round.

Definitions

| Term | Meaning |
| --- | --- |
| Booth / Unit | One complete portable ballistic guard booth as specified in this Appendix. |
| Call-off | A written purchase order issued by the Customer under the framework agreement. |
| Delivery | Booth delivered, installed and ready for use at the location specified in the call-off. |
| Site | The physical location where the booth is placed (typically in central Oslo). |

General and contractual requirements

| ID | Type | Requirement | Supplier reply / compliance & documentation |
| --- | --- | --- | --- |
| 1. | M | The Supplier shall comply with all applicable Norwegian and EU/EEA regulations relating to the Product, including but not limited to product safety, electrical safety (EN 60335 or equivalent), CE-marking where applicable, and the Norwegian Working Environment Act. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 2. | M | The Supplier shall hold any export licence(s) required for delivery to Norway and shall, on request, document that the Products may legally be exported from the country of manufacture to Norway. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 3. | M | The Supplier shall be able to deliver to any location in mainland Norway specified by the Customer in a call-off. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 4. | M | The Supplier shall provide a named, technically competent contact person in English, dedicated to the Customer throughout the contract period (cf. General contract terms, clause on Supplier's responsibilities). | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 5. | E | The Customer needs assurance that the Supplier can manage deliveries under the framework agreement, including capacity, project organisation, project management and delivery follow-up from call-off to approved delivery. Describe how this is met. Service, maintenance and support in the operational phase is assessed under requirement 41 and shall not be described here. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 6. | M | Provide the Supplier's complete legal name, organisation number, country of registration and place of manufacture for the offered Product. | [Supplier shall fill in: confirmation, comments and reference to documentation] |

Ballistic and blast protection (security)

These requirements are the core of the procurement. M-requirements are absolute. E-requirements are weighted under the award criterion "Quality".

| ID | Type | Requirement | Supplier reply / compliance & documentation |
| --- | --- | --- | --- |
| 7. | M | All exterior walls, the door, the roof and any windows of the booth shall as a minimum provide ballistic protection corresponding to BR6NS or equivalent. The protection level shall be 360° around an occupied position inside the booth and shall include against the floor at standard standing/sitting positions. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 8. | M | The booth shall protect occupants against the splinter effect of a hand grenade according to Exr2ns. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 9. | E | The Customer needs the best possible protection of personnel and has an interest in protection beyond the mandatory minimum, e.g. higher ballistic class, blast-overpressure resistance, anti-tamper, anti-drone or IED standoff capability, and firing port(s) (desired but not mandatory). Describe what is offered above the minimum, with test certificates / engineering reports. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 10. | M | Windows shall provide the same ballistic class as the walls (cf. requirement 7) and shall offer a good field of view from inside the booth. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 11. | M | The booth shall have one (1) primary door and one (1) emergency exit, located on different sides of the unit. Both shall provide the ballistic class according to requirement 7 when closed. The primary door shall easily be openable from inside without a key. The primary door shall be delivered complete in accordance with the specified security class, including locks and fittings (hinges, handles, strike plates, multi-point locking as relevant), and shall be pre-prepared for electronic access control (cable routing, cut-outs and mounting provisions for card reader, electric strike/motor lock and door position sensor – drawings/specific requirements will be sent by customer before production). The actual access-control hardware and system is supplied and installed by the Customer. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 12. | M | All locks, hinges and fasteners exposed on the exterior of the booth shall be tamper-resistant and shall not compromise the ballistic protection. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 13. | M | The windows must have privacy protection that also works at night, for example roller blinds or similar. | [Supplier shall fill in: confirmation, comments and reference to documentation] |

Mobility, transport and deployment

| ID | Type | Requirement | Supplier reply / compliance & documentation |
| --- | --- | --- | --- |
| 14. | M | The booth shall be transportable on a standard semi-trailer or flatbed truck on Norwegian public roads within maximum legal dimensions without special transport permit (dispensasjon), cf. vegtrafikkloven and forskrift om bruk av kjøretøy: width ≤ 2.55 m, height ≤ 4.00 m, length ≤ 12 m, weight ≤ 50 t. Forskrift om bruk av kjøretøy also permits transport widths up to 3.00 m. Suppliers may therefore offer a wider variant, with external width up to 3.00 m, as a separately priced option in Appendix 2. The supplier shall describe any consequent transport conditions (e.g. load marking, routing or time restrictions, escort vehicle) for such a variant. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 15. | E | The Customer needs a booth that is quick and simple to install, anchor, remove and relocate between sites with minimum downtime, using standard mobile-crane equipment and without specialist tools, and with a reusable foundation/anchoring system where possible. Describe how this is met, and enclose a written installation/removal procedure and load drawings. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 16. | M | Internal usable floor area shall be ≥ 9 m² with internal head clearance ≥ 2.10 m. Internal dimensions are derived from the external dimensions specified in requirement 14 less wall/construction thickness; the supplier is responsible for achieving the stated usable area within that external envelope. Larger variants may be offered as a separately priced option in Appendix 2. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 17. | M | All infrastructure connections (power, fibre/Ethernet, water/drainage if applicable) shall enter the booth from below (from the ground) and shall be physically protected against tampering, mechanical impact and weather along the entire route from ground level into the booth. The protection shall not compromise the ballistic class of the booth. The Supplier shall describe and illustrate the solution. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 18 | M | The booth must be capable of being securely positioned on various surfaces (e.g. asphalt, gravel, grass) and must be able to stand level regardless of variations in the terrain. | [Supplier shall fill in: confirmation, comments and reference to documentation] |

Climate adaptation, ventilation and working environment

| ID | Type | Requirement | Supplier reply / compliance & documentation |
| --- | --- | --- | --- |
| 19 | M | The booth shall be designed for outdoor year-round use in Norwegian climate, with operational temperature range −30 °C to +35 °C, exposure to driven snow, rain, salt mist and UV. Construction materials shall be corrosion-resistant or treated accordingly | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 20 | M | The booth shall maintain an internal temperature of 20 ± 3 °C at any outside temperature between −30 °C and +35 °C, with two occupants inside and the door closed. Heating and cooling shall be electrically powered and shall comply with EN 60335 (or equivalent). | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 21 | M | The booth shall provide forced ventilation dimensioned for continuous (24/7) use by two persons, ensuring a satisfactory indoor working environment. As a minimum, CO₂ concentration shall be kept ≤ 1000 ppm under full occupancy. Intake and exhaust shall be positioned so that they cannot be obstructed by external actors and so that water/snow ingress is prevented. The supplier shall describe how this is achieved. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 22. | E | The Customer needs an effective and energy-efficient heating and cooling solution, well-insulated walls/roof/floor, and a low internal noise level (target ≤ 45 dB(A)) for a good working environment. Describe how this is met. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 23. | M | The booth shall comply with the Norwegian Working Environment Act (Arbeidsmiljøloven) and Forskrift om utforming og innretning av arbeidsplasser og arbeidslokaler (best. nr. 702) for permanent workplaces, including lighting, ventilation, acoustic conditions and visibility. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 24. | M | The booth shall provide a functional workspace for two persons seated, including seating positions and desk/work surface for two. As a minimum it shall provide a clear internal width of ≥ [2.30] m and usable depth allowing two adjacent seated workstations, with internal head clearance ≥ 2.10 m. The layout shall not obstruct entry/exit or the ventilation intakes/exhaust (cf. requirement 21). The supplier shall describe and illustrate the internal layout for two persons. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 25. | M | The booth shall have an emergency ventilation system, independent of the primary ventilation, that protects two occupants both if the primary ventilation fails and if the outside air is contaminated (e.g. tear gas, smoke). In case of outside contamination it shall be possible to shut off the outside air and/or supply filtered air, so the occupants can stay safely inside or evacuate in a controlled way. The Supplier shall state how long the system keeps the air safe for two occupants. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 26. | M | The booth shall include a dedicated area, prepared for the later installation of storage for both personal and shared/communal equipment, including: - Shields, sandbags, breaching equipment - Personal protective equipment - Rack for long guns (two-handed firearms) The actual storage units and weapon rack are supplied separately by the Customer. The booth shall be delivered ready-prepared for mounting, with the necessary fixing points / brackets (or equivalent provisions) so that these can be installed without compromising the ballistic protection (cf. requirement 7). The Supplier shall describe and illustrate the prepared area and the mounting provisions. | [Supplier shall fill in: confirmation, comments and reference to documentation] |

Power, communications and IT interfaces

| ID | Type | Requirement | Supplier reply / compliance & documentation |
| --- | --- | --- | --- |
| 27. | M | The booth shall operate on a single 230 V AC / 50 Hz single-phase supply (Norwegian standard). Internal distribution shall include automatic fuses, RCD protection. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 28. | M | All electrical installations shall comply with NEK 400:2026 (Norwegian wiring regulations), or the version of NEK 400 in force at the time of design | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 29. | M | The booth shall have minimum six (6) internal 230 V power outlets and minimum two (2) USB-C outlets, accessible from the seated/standing working position. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 30. | M | The booth shall be prepared for fibre-optic and power cabling, with an internal patch panel and cable entry point that can be sealed against weather and that maintains the ballistic protection class. The booth shall include internal cable trays / conduits (føringsveier) from the cable entry point to the IT rack position, to the door (for access control) and to relevant interior outlets, and to security cameras on the roof. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 31. | E | The Customer needs a cabling solution that makes it easy to route cables to the various positions inside and outside the booth (IT rack, door/access control, roof cameras, antennas), that has good spare capacity for future needs, and that does not impair other functionality, in particular the ballistic protection (cf. requirement 7) and the ventilation (cf. requirement 21). Describe how this is met. Note: all radio, IT and communication equipment (including TETRA/Nødnett, 4G/5G, IP-cameras and intercom) is supplied and installed by the Customer. The Supplier shall provide only the necessary preparation (cable trays, entry points, mounting provisions and antenna pass-throughs). | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 32. | M | The booth shall be equipped with exterior floodlights / lighting that can be switched on as needed, and with adjustable interior lighting including a dimmable night-light setting and a red-light (night-vision-preserving) mode for the interior. All lighting, interior and exterior, shall be electronically controllable from the working position (e.g. switch panel or control unit), allowing it to be switched and dimmed, including being quickly turned off. The Supplier shall describe the lighting and control solution. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 33. | M | The booth must be equipped with a speaker/intercom system that allows the occupant to speak from inside to those outside. | [Supplier shall fill in: confirmation, comments and reference to documentation] |

Interior, ergonomics and aesthetics

| ID | Type | Requirement | Supplier reply / compliance & documentation |
| --- | --- | --- | --- |
| 34 | M | The interior shall include: a height-adjustable seat (sitting and standing positions), a fold-down or fixed work surface, internal lighting (general + task light, both dimmable, ≥ 300 lux at work surface), coat hook, and storage compartment for personal equipment. The booth shall include a dedicated, ventilated space for an IT rack for technical equipment, with separate 230 V power feed, earthing point and connection to the internal cable trays. The rack space shall be flexible so that it can accommodate different standard rack solutions; as a minimum it shall accommodate equipment in standard 19-inch format. The Supplier shall state the usable internal dimensions of the space offered (width, depth and height in rack units, U) and describe the adaptations the offered solution allows. Exact rack dimensions and mounting details will be sent by the Customer before production, within the flexibility described by the Supplier. The rack position shall not obstruct the working area for the occupants and shall be accessible for maintenance. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 35 | E | The Customer has a need for some welfare provision for the occupants, e.g. a worktop and storage for small appliances such as a coffee machine and microwave, with power on a separate circuit that does not impair the booth's electrical capacity (cf. requirement 27) or climate control (cf. requirement 20). Describe how this is met. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 36 | M | The exterior shall be finished in a neutral, low-visibility colour by default. The Supplier shall be able to offer alternative RAL colours at no significant extra cost; the price of customisation shall appear in Appendix 2. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 37 | E | The Customer needs booths that appear professional and discreet rather than obviously military, since some will be placed in front of foreign embassies. Describe the visual / aesthetic options offered. | [Supplier shall fill in: confirmation, comments and reference to documentation] |

Lifetime, maintenance, warranty and service

| ID | Type | Requirement | Supplier reply / compliance & documentation |
| --- | --- | --- | --- |
| 38 | M | The booth shall have a design service life of at least 15 years under continuous outdoor use in Norwegian climate, with normal maintenance. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 39 | M | The Supplier shall provide a minimum warranty of five (5) years on the structural construction (including ballistic integrity) and two (2) years on electrical and HVAC components, from the date of approved delivery. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 40 | M | The Supplier shall provide a complete preventive maintenance plan in English (or Norwegian), spare parts list and operating manual, all in editable electronic form, prior to first delivery. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 41 | E | The Customer needs a reliable service and maintenance offering over the booths' service life, including annual inspection and predictable response times for corrective maintenance. Describe how this is met. Prices for service shall appear in Appendix 2. | [Supplier shall fill in: confirmation, comments and reference to documentation] |
| 42 | M | The Supplier shall guarantee availability of critical spare parts (ballistic panels, doors, windows, locks, HVAC, controls) for a minimum of ten (10) years after the last delivery under the framework agreement. | [Supplier shall fill in: confirmation, comments and reference to documentation] |

Delivery time, planning and milestones

| ID | Type | Requirement | Supplier reply / compliance & documentation |
| --- | --- | --- | --- |
| 43 | M | The Supplier shall be able to deliver a booth within twelve (12) weeks from the date of the first call-off. | [Supplier shall fill in: confirmation, comments and reference to documentation] |

Environment and sustainability

These requirements are evaluated under the award criterion "Environment" (weight 30 %). In line with the Public Procurement Act § 5 and the climate / environment regulations, environmental impact over the full life cycle of the booth is assessed.

| ID | Type | Requirement | Supplier reply / compliance & documentation |
| --- | --- | --- | --- |
| 44 | E | The Customer needs to minimise the environmental impact of the booths over their life cycle. Describe the total greenhouse-gas emissions per booth (kg CO₂-equivalents) over its life cycle. Lower documented emissions are evaluated positively. | [Supplier shall fill in: confirmation, comments and reference to documentation] |

HSE during installation and operation

| ID | Type | Requirement | Supplier reply / compliance & documentation |
| --- | --- | --- | --- |
| 45 | M | Personnel performing installation in Norway shall have HMS-card (HSE card from Arbeidstilsynet) where applicable and shall hold valid certificates for any lifting/crane operations. | [Supplier shall fill in: confirmation, comments and reference to documentation] |

Documentation and training

| ID | Type | Requirement | Supplier reply / compliance & documentation |
| --- | --- | --- | --- |
| 46 | M | The Supplier shall deliver, with each booth: (a) operating manual in Norwegian or English, (b) maintenance manual, (c) electrical drawings, (d) ballistic certificate (copy), (e) declaration of conformity, (f) spare-parts list with part numbers. All in editable digital form. | [Supplier shall fill in: confirmation, comments and reference to documentation] |

Options and add-ons

The following options shall be priced separately in Appendix 2 – Price. Options will not be evaluated at award unless explicitly stated, but their availability and price will inform the Customer's decision.

- Water supply and drainage (vann og avløp), where standard solutions exist for other booths in the Customer's portfolio. The Supplier shall describe the solution and price it separately in Appendix 2.

- Larger Booths with capacity for more people and toilet/small kitchen which also fulfills all relevant requirements in this requirement specification.

- As a priced option, the booth shall provide an internal UPS / battery backup giving minimum 60 minutes of operation for lighting and communication equipment at −20 °C. The backup unit may be located internally or externally, provided it does not compromise the ballistic protection (cf. requirement 7) and is protected against the climate (cf. requirement 19). The price of this option shall be stated in Appendix 2. The Supplier shall describe the proposed solution, including location, capacity and expected backup time.

Supplier's overall declaration

By signing this Appendix the Supplier declares that the offered Product meets all M-requirements and that the description of E-requirements is correct and complete to the best of the Supplier's knowledge.

Date: _______________________

Place: ______________________

Signature: ___________________________

Name and position (printed): ____________________________________________