Appendix 3: Customer´s technical platform

- Table of Contents

1	Agreement, Section 1.1 Scope of the Agreement	2

1.1	Description on the current solution	2

1.2	Description of the solution	2

1.3	Description of integration interfaces in Bane NOR	3

1.4	Requirements for interaction with other equipmen/solutions	3

1.5	Administration tools	3

1.6	Description of monitoring solution	3

Agreement, Section 1.1 Scope of the Agreement

Description on the current solution

Description of the solution

System description

Bane NOR currently operates a centralized passenger information system (the “KARI” system) for providing real-time information to passengers. As part of this system, there is an audio announcement queue mechanism that distributes all audio files to be played to the public nationwide. All audio files are generated using the Acapella text-to-speech engine and encoded in OGG format. The audio files are generated as RAW PCM 8 kHz / 22 kHz, RAW A-law (without header) 8 kHz, and RAW µ-law (without header) 8 kHz.

Locations where audio announcements are used include:

- Platforms for boarding and alighting

  - A single platform serving only one track

  - A single platform located between two tracks

  - Two platforms, one platform per track, but with multiple sets

  - Larger stations with multiple tracks and a combination of the above solutions

- Small waiting rooms associated with a station

- Medium-sized waiting rooms associated with a station

- Large waiting areas combined with other functions, such as ticket sales

- Tunnels/stations (warning systems)

Different locations may require different solutions for audio announcements. The size of the room/area and the number of zones will require cost-effective solutions. In most cases, Bane NOR’s IP/MPLS-based Ethernet network is available.

Description of integration interfaces in Bane NOR

Bane NOR’s centralized passenger information system, KARI, communicates with the amplifiers. The passenger information system sends audio files in OGG format to the amplifiers via SSH and makes an API call to the amplifier to trigger playback. The passenger information system manages the queue and keeps track of the playback priority on the amplifiers. When playback has been completed, the amplifier sends an API request back to KARI to confirm that playback has been completed. Heartbeat signals are also regularly sent from the passenger information system to verify that the amplifier is available.

Requirements for interaction with other equipmen/solutions

Our centralized passenger information system must be able to transfer audio files to the amplifiers and communicate via API.

This includes:

- Support for transferring audio files via SSH or API.

- Ability to receive API requests from the system.

- Ability to send API requests back upon completion of playback.

The passenger information system maintains an overview of the queue and priority levels for announcements on the amplifiers.

Administration tools

Bane NOR currently uses Foreman and Ansible for operating system installation and configuration management. The amplifiers should be capable of being configured and installed using these tools to ensure efficient operation and maintenance. The units must therefore support UEFI boot and be configurable via SSH.

Description of monitoring solution

Bane NOR uses Zabbix as its monitoring system. It is desirable to be able to install the Zabbix agent on the amplifiers to enable continuous monitoring and status reporting.